3GPP TSG-RAN WG2 #123 	R2-2307470

Toulouse, France 21 – 25 August 2023

Agenda Item:	7.19.3

Source:	NEC

Title:	Further discussions on early indication and access restrictions for eRedCap

Document for:	Discussion, Decision

Introduction

RAN2#122 meetings further discussed access restrictions for eRedCap and achieved following agreements [1]:

Agreements - RAN2 confirms there can be cell(s) supporting Rel-18 eRedCap only, i.e., not allowing Rel-17 RedCap UE to camp and access. - We introduce R18 versions of 1Rx and 2Rx barring bits and we don’t introduce a R18 version of the HD-FDD allowed-bit, i.e., the R17 HD-FDD allowed-bit is reused for and applied by R18 eRedCap UEs. - All R18 eRedCap UEs uses the two new LCIDs for Msg3/MsgA PUSCH for CCCH/CCCH1 during Random Access, i.e., both those with peak rate reduction + BB BW reduction, and those with only peak rate reduction.

This contribution further discusses the access restrictions for eRedCap.

Discussion

Early indication

RAN1#113 meeting achieved following agreements on eRedcap Msg1/MsgA PRACH early indication.

Agreement - A network-configurable additional separate early indication in Msg1 for Rel-18 eRedCap UEs is supported. - When Msg1 indication for Rel-18 eRedCap UEs is configured, it is used by Rel-18 eRedCap UEs (with or without UE BB bandwidth reduction). - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. - Note: Rel-18 eRedCap UEs will be differentiated from Rel-17 RedCap UEs based on Msg3 of Rel-18 eRedCap UEs. - Additional early indication in MsgA PRACH is not supported.

Based on RAN1 and RAN2 agreements, Rel-17 RedCap and Rel-18 eRedCap early indication are summarized in Table 1.

Table 1 Rel-17 RedCap and Rel-18 eRedCap early indication

|  | Rel-17 RedCap | Rel-18 eRedCap |
| --- | --- | --- |
| Msg1 early Indication | Support (optional) | Support (optional) - When Msg1 indication for Rel-18 eRedCap UEs is not configured while Msg1 indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. |
| MsgA PRACH early indication | Support (optional) | Not support - When MsgA PRACH indication for Rel-17 RedCap UEs is configured, Rel-18 eRedCap UEs shall share the PRACH that is configured for Rel-17 RedCap UEs. |
| Msg3 early indication | Support (mandatory) | Support (mandatory) - All R18 eRedCap UEs uses the two new LCIDs |
| MsgA PUSCH early indication | Support (mandatory) | Support (mandatory) - All R18 eRedCap UEs uses the two new LCIDs |

There is no open issue for Rel-18 eRedCap early indication.

Access Restriction

During RAN2#121-bis meeting, the followings were agreed

- Introduce R18 eRedCap UE specific IFRI in SIB1. - The new R18 eRedCap UE specific IFRI functionality works as follows: - - Controls cell selection/reselection to intra-frequency cells for eRedCap UEs when this cell is considered barred by the eRedCap UE, as specified in TS 38.304 [20]. - - Working assumption (pending check in running CRs): If not present, an eRedCap UE treats the cell as barred, i.e., the UE considers that the cell does not support eRedCap.

In Rel-17, a separate IFRI was introduced. If the Rel-17 RedCap specific IFRI is not present in SIB1, Rel-17 eRedCap UE consider the cell as barred and perform barring as if Rel-17 RedCap specific IFRI is set to allowed. In our understanding, the work assumption highlighted in yellow defines necessary Rel-18 eRedCap UE behaviours when Rel-18 eRedCap UE try to access a legacy NW without broadcasting Rel-18 eRedCap UEs related indications in SIB1. Therefore, we propose to confirm the RAN2#121-bis working assumption.

RAN2 confirm the working assumption: If the Rel-18 eRedCap specific IFRI is not present in SIB1, Rel-18 eRedCap UE consider the cell as barred and perform barring as if eRedCap specific IFRI is set to allowed.

Similar to early indication, Rel-17 RedCap and Rel-18 eRedCap access restriction are summarized in Table 2 based on agreements achieved in RAN2 meeting so far.

Table 2 Rel-17 RedCap and Rel-18 eRedCap access restriction

|  | Rel-17 RedCap | Rel-18 eRedCap |
| --- | --- | --- |
| intraFreqReselection in SIB1 | Support - intraFreqReselectionRedCap | Support - Introduce R18 eRedCap UE specific IFRI in SIB1 (RAN2#121-bis). - functionality works in the same way as RedCap (per Proposal 1) |
| halfDuplexRedCapAllowed in SIB1 | Support | Not support - the R17 RedCap HD-FDD allowed-bit is reused for and applied by R18 eRedCap UEs |
| cellBarredRedCap1Rx cellBarredRedCap2Rx in SIB1 | Support | Support - introduce R18 versions of 1Rx and 2Rx barring bits - eRedCap UE behaves in the same way as RedCap UE |
| interFreqCarrierFreq allowed in SIB4 | Support - redCapAccessAllowed in SIB4 | Support - Introduce eRedcapAccessAllowed-r18 in interFreqCarrierFreqList in SIB4, about the frequency of neighbour cell supporting eRedCap, similar to R17 |

There is no open issue for Rel-18 eRedCap access restriction.

Summary

This contribution discussed the access restrictions for eRedCap.

Observation 1 There is no open issue for Rel-18 eRedCap early indication.

Observation 2 There is no open issue for Rel-18 eRedCap access restriction.

Proposal 1 RAN2 confirm the working assumption: If the Rel-18 eRedCap specific IFRI is not present in SIB1, Rel-18 eRedCap UE consider the cell as barred and perform barring as if eRedCap specific IFRI is set to allowed.

- References

[1] R2-2306550 Report from eRedCap breakout session, Session chair (Ericsson)