3GPP TSG-RAN WG2 Meeting#123                                	R2-2307477

Toulouse, France, August 21-25, 2023

Agenda item:	7.6.2.2

Source: 	Google Inc.

Title: 	Discussion on the GNSS Validity Reporting

Document for:	Discussion and Decision

# Introduction

According to the Rel-17 specification (TS 36.300 v17.4.0) [1], a UE shall obtain its valid GNSS position before connecting to a NTN cell, and shall move to the idle state upon outdated GNSS position. This is because UE’s location information is essential for performing time and frequency compensation and a NB-IoT UE is not able to perform the mobile communication task with the BS while accessing the GNSS module. In Rel-18, the enhancements on the GNSS operation are discussed in RAN1/RAN2, with the intention to allow UE to remain in the connected state while conducting the GNSS position fix. In the RAN2#121bis_e and RAN2#122 meetings, the following agreements were made with regard to the GNSS operation enhancement [2][3].

Agreements in RAN2#121bis_e: - There is no need for UE to provide GNSS position fix time duration in Msg3. - RLM is suspended during the GNSS measurement gap while the UE is measuring GNSS - UE can stay in RRC_CONNECTED state when current GNSS position becomes out-of-date if the UE enters a GNSS measurement gap. FFS whether the new GNSS measurement shall be started before, upon or after the current GNSS validity duration expiry - For a UEs that cannot acquire system information and GNSS position at the same time, acquisition of SIB31 may be postponed until GNSS measurement is completed if the UE cannot complete acquisition of SIB31 before the start of GNSS measurement gap - For the NB-IoT CP solution, UE will report the GNSS validity duration by using a MAC CE - RAN2 can discuss UE autonomously reacquire GNSS during inactive state of C-DRX based on RAN1’s input in the next RAN2 meeting - Working Assumption: GNSS validity duration UE reported after GNSS measurement is the remaining validity duration

Agreements in RAN2#122: - Confirm the working assumption that GNSS validity duration UE reports is the remaining validity duration - The UE triggers GNSS measurement reporting every time upon completing the GNSS fix operation - When network triggers GNSS measurement initiation is up to network implementation - Add a note to state some AS operations are suspended when UE is performing GNSS measurement during GNSS measurement gap - GNSS fix time duration should be reported in 1) and 2): - RRCConnectionReestablishmentComplete and RRCConnectionReestablishmentComplete-NB - RRCConnectionReconfigurationComplete for HO case - (FFS whether there are some scenarios where this is not needed or whether there has to be some explicit NW indication to do so) - Working Assumptions - An UL MAC CE for GNSS validity duration reporting is used for NB-IoT user plane solution and eMTC UE as well, in addition to previously agreed NB-IoT control plane solution - A new DL MAC CE is introduced to trigger connected UE to perform GNSS measurement

In this paper, we discuss several leftover issues based on the agreements highlighted in yellow above, and attempt to resolve them with some remedies proposed in the paper.

# Discussion

## GNSS measurement gap

In Rel-17, the UE autonomously determines the GNSS validity duration and reports it to the network during the connection setup, connection resume, handover, and connection reestablishment procedure. Although the reported GNSS validity duration is used by the network to estimate when UE’s GNSS position will become outdated, the UE may receive an outdated indication from the lower layer and goes to the idle state anytime. For instance, if UE starts to move faster, the UE may receive an outdated indication even far before the GNSS validity timer expires.

UE may receive an outdated GNSS position indication and goes to RRC_IDLE anytime, even before the GNSS validity duration expires.

As the UE only reports the GNSS validity duration once per connected state, the network uses the GNSS validity duration to estimate when the GNSS position kept by the UE will become outdated, and then will release the UE connection at the estimated time. The network may of course release the UE connection earlier by sending an RRC Release to the UE before the GNSS validity duration expires, but this is not mandatory because the UE may has already been in the idle state (which is not known to the network) and cannot receive it.

When the GNSS validity duration of a UE expires, the network may assume UE’s GNSS position becomes outdated and release the UE connection without sending an RRC release message.

As the remaining GNSS validity duration is known by both the UE and network, it can be served as the last resort for implicitly releasing the UE connection, if the network does not send the RRC Release message earlier. To make this happen, we need to make the UE go to idle upon the expiry of the GNSS validity timer. The simplest way is to make UE consider the GNSS position as outdated if the GNSS validity duration has expired, since the Rel-17 UE would autonomously go to idle if the GNSS position becomes outdated.

UE considers the GNSS position as outdated upon receiving an outdated indication from the lower layer, or upon the expiry of the GNSS validity duration.

Reporting the GNSS validity duration has an additional effect for the Rel-18 UE: to obtain a GNSS measurement gap that takes place before the UE goes to idle. Since now the UE can stay in the connected state even when the GNSS position becomes outdated if the UE has entered a GNSS measurement gap (agreement in RAN2#121bis_e), to keep the UE in the connected state, the network only needs to make sure the GNSS measurement gap starts before the GNSS validity duration expires (i.e., it is okay if the GNSS validity duration expires before the GNSS measurement gap ends).

It is possible that the GNSS validity duration expires before the GNSS measurement gap ends

Since we have created an exception (i.e., within a GNSS measurement gap) for the UE to remain in RRC_CONNECTED even after UE’s GNSS position becomes outdated, we need to take care of UE’s RRC state at the end of the GNSS measurement gap, as UE may either conduct the GNSS position fix successfully or unsuccessfully in the GNSS measurement gap. If the UE conducted the GNSS position fix successfully, the UE shall remain in RRC_CONNECTED after the GNSS measurement gap, and report again the GNSS validity duration to the network. If the UE failed to conduct the GNSS position fix, the UE shall go to RRC_IDLE immediately at the end of the GNSS measurement gap.

UE goes to the idle state at the end of the GNSS measurement gap, if the GNSS position becomes outdated and the UE failed to conduct the GNSS position fix in the GNSS measurement gap.

On the other hand, the NW has to rely on whether the UE has reported the GNSS validity duration after the GNSS measurement gap, to determine whether to keep the UE in the connected state or not. Therefore, we need a mechanism to make UE report the GNSS validity duration, preferably within a period of time after the GNSS measurement gap ends. If the UE is not able to report the GNSS validity duration within the period, the NW considers the GNSS position fix was not conducted successfully by the UE, and hence transits the UE to the idle state if the GNSS validity duration has also expired. Therefore, we should introduce a new timer and let UE/eNB start the new timer at the end of the GNSS measurement gap, where the new timer is stopped upon UE reporting (or eNB receiving) the GNSS validity duration. Upon the expiry of the new timer, UE transits to the idle state and eNB releases the UE connection, if the GNSS validity duration has also expired.

UE/eNB starts a new timer at the end of the GNSS measurement gap if the GNSS position has become outdated. UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE goes to the idle state when the new timer expires.

After the UE has reported a GNSS validity duration, the GNSS validity duration may change due to UE mobility. Therefore, the next GNSS measurement gap configured to the UE, which is based on the GNSS validity duration reported by the UE, may become not appropriate any more, as the UE may already lose its UL synchronization with the NW before entering the GNSS measurement gap. To address this issue, UE should be allowed to report again its GNSS validity duration (probably with a smaller value) to the NW, once certain triggering condition(s) is/are met, where the triggering condition(s) should be configured by the NW. The triggering condition(s) can be for instance: 1) UE has received a TA command from the eNB indicating a TA adjustment larger than a threshold value; 2) UE has received more than k times the TA command from the eNB in a given period of time; 3) UE’s accumulated TA adjustment value (i.e., NTA) has exceeded a threshold value.

UE can be configured with a triggering condition(s) for reporting the GNSS validity duration. UE transmits an updated GNSS validity duration to the NW, once the triggering condition(s) is/are met.

## GNSS position fix with c-DRX

Apart from the measurement gap, we may also need to consider other mechanisms that could also provide the UE with an inactive period for conducting the GNSS position fix, such as the c-DRX mechanism. In RAN2#121bis_e meeting, it was concluded that RAN2 can discuss UE autonomously reacquire GNSS during inactive state of c-DRX based on RAN1’s input in the next RAN2 meeting. Moreover, RAN1 has made some progress and has sent a LS [4] to RAN2 stating the following:

- From RAN1 perspective, UE is not forbidden to autonomously re-acquire GNSS position fix during inactive state of Connected DRX. - Note: The configured DL/UL transmissions during inactive state of Connected DRX should not be impacted - Note: details are up to RAN2

To utilize the c-DRX mechanism, we think if the UE has been configured with the c-DRX, it is very likely the GNSS position becomes oudated in the c-DRX inactive state (i.e., the c-DRX OFF duration). In this case, similar to the case where the UE is within a GNSS measurement gap, the UE should be allowed to stay in the connected state until it gets back to the c-DRX active state (i.e., the c-DRX ON duration). This is because the UE may need to prioritize the configured DL/UL transmissions (according to RAN1 LS) and hence may not be able to conduct the GNSS position fix in time (i.e., before the GNSS validity duration expires) during the inactive state of the c-DRX.

UE can remain in the connected state when the GNSS position becomes outdated in a c-DRX inactive state (i.e., c-DRX OFF duration).

If the UE has conducted a GNSS position fix in the c-DRX inactive state after the GNSS position becomes outdated, we should let UE/eNB start a new timer at the end of the c-DRX inactive state (i.e., the beginning of the c-DRX active state), where the new timer is stopped upon UE reporting (or eNB receiving) the GNSS validity duration. Upon the expiry of the new timer, UE transitions to the idle state and eNB releases the UE.

When UE’s GNSS position becomes outdated in a c-DRX inactive state, UE/eNB starts a new timer at the beginning of the next c-DRX active state.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE goes to the idle state when the new timer expires.

# Conclusion

In this paper, we discuss several issues on the GNSS measurement gap, with the following observation being made.

Observation 1	UE may receive an outdated GNSS position indication and goes to RRC_IDLE anytime, even before the GNSS validity duration expires.

Observation 2	When the GNSS validity duration of a UE expires, the network may assume UE’s GNSS position becomes outdated and release the UE connection without sending an RRC release message.

Observation 3	It is possible that the GNSS validity duration expires before the GNSS measurement gap ends

Based on the observations and discussion in this paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Proposal 1	UE considers the GNSS position as outdated upon receiving an outdated indication from the lower layer, or upon the expiry of the GNSS validity duration.

Proposal 2	UE goes to the idle state at the end of the GNSS measurement gap, if the GNSS position becomes outdated and the UE failed to conduct the GNSS position fix in the GNSS measurement gap.

Proposal 3	UE/eNB starts a new timer at the end of the GNSS measurement gap if the GNSS position has become outdated. UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE goes to the idle state when the new timer expires.

Proposal 4	UE can be configured with a triggering condition(s) for reporting the GNSS validity duration. UE transmits an updated GNSS validity duration to the NW, once the triggering condition(s) is/are met.

Proposal 5	UE can remain in the connected state when the GNSS position becomes outdated in a c-DRX inactive state (i.e., c-DRX OFF duration).

Proposal 6	When UE’s GNSS position becomes outdated in a c-DRX inactive state, UE/eNB starts a new timer at the beginning of the next c-DRX active state.  UE/eNB stops the new timer upon reporting/receiving the GNSS validity duration. eNB releases the UE connection and UE goes to the idle state when the new timer expires.

# Reference

3GPP TS 36.300 (v17.5.0), E-UTRA and E-UTRAN Overall Description, stage 2.

R2-2306553, Report of 3GPP TSG RAN WG2 meeting #121bis-e.

Draft Report of 3GPP TSG RAN WG2 meeting #122.

R2-2304612, LS on GNSS position fix during inactive state of Connected DRX for improved GNSS operations, MediaTek.