3GPP TSG-RAN2 Meeting #122bis	R2-2307493

Toulouse, 21st – 25th Aug, 2023

Agenda item:	7.11.2.1

Source:	Huawei, Hisilicon

Title:	CP issues for multicast reception for RRC INACTIVE UE

Document for:	Discussion

- Introduction

The following agreements were made during the discussion in RAN2#122 meeting for service continuity and notifications for multicast reception in RRC_INACTIVE.

- The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline. - To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline. - Working assumption (to be confirmed by RAN1 via pending reply LS): One bit in the MCCH DCI is used to notify the change of the multicast MCCH. We reuse the bit used for MCCH change indication from Rel-17 MBS broadcast. This does not cover session deactivation which is FFS. - It is not supported to provide the PTM configuration of intra-gNB neighbour cells in the dedicated signalling. - For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline. - As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH. - FFS how existing MRBs are handled. - Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE. - MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI. - This is assumed to have no/minor impact on RAN1/PHY

In this contribution, we will further analyze the remaining issues based on the progress on multicast reception in RRC_INACTIVE and also some issues related to the Stage-3 RRC.

- Discussion

2.1 PTM configuration and mobility aspects

UE resume due to bad reception quality

In the current RRC running CR, there is an Editor’s Note about resumption due to bad reception quality, as following:

Editor’s note: The details/parameter for the reception quality.

It has been agreed that UE may trigger RRC connection resumption if the reception quality of the multicast data is below a configured threshold. Based on proposals from companies, the reception quality might be BLER or signal quality (RSRP/RSRQ/SINR). Considering that the measurement definition for signal quality is quite mature in NR specification and the relation between SINR and BLER is expectable, the threshold based on signal quality can be used to trigger such resumption to avoid the need of further introducing the measurement definition for BLER.

In addition, different gNBs might have different policy to trigger resume for bad quality UEs, i.e., the thresholds may be different in different cells. Therefore, it is better to configure the threshold in MCCH rather than in RRC release message. Otherwise, after mobility, the UE will need to resume to receive the threshold from the new cell.

Furthermore, it is worth considering whether this threshold can be per service or service group to allow different policy for different services considering QoS is likely to be different for different services.

Proposal 1a: The threshold used to trigger RRC resumption is based on signal quality, i.e., configuration among RSRP, RSRQ, or SINR.

Proposal 1b: The threshold used to trigger RRC resumption is configured in MCCH.

Proposal 1c: Further discuss whether the threshold is per service or per service group or per cell.

NCL for multicast

It has been agreed that NCL is supported for multicast reception in RRC_INACTIVE similar as MBS broadcast NCL. However, the NCL can only indicate whether there is ongoing RRC_INACTIVE multicast service in neighbour cells. The UE still has to read SIB and multicast MCCH after mobility, as long as the service is ongoing in the target cell for RRC_INACTIVE. However, at least for intra-gNB case, it is very likely that the gNB will use exactly the same PTM configuration to provide the same multicast service in different cells. If this information is provided to the UE beforehand and the target cell is using the same PTM configuration as the source cell, the UE can save the interruption time to acquire SIB and multicast MCCH in the target cell during mobility and directly begin to receive service. Note that this comes with little cost as it doesn’t mean to provide full PTM configuration of neighbor cells but just to indicate whether the configuration is the same or not.

Proposal 2: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE.

Frequency prioritization

The frequency prioritization mechanism is beneficial for multicast reception in RRC_INACTIVE at least for the scenario where a multicast service is provided on the same frequency. But different from broadcast, there is no pre-planned area for multicast in RRC_INACTIVE, thus it is not possible to use the FSAI based solution as for MBS broadcast. Since the dedicated frequency priority can be provided via RRC dedicated signaling, we think this legacy mechanism can be reused for frequency prioritization for multicast in RRC_INACTIVE without introducing a new mechanism.

Proposal 3: Legacy mechanism of providing the dedicated frequency priority in RRC dedicated signaling can be reused for multicast frequency prioritization in RRC_INACTIVE.

Resume cause and access category

Based on the discussion in the previous meetings, the UE needs to trigger RRC Resume in several scenarios when configured to receive multicast in RRC_INACTIVE, including:

Case 1: UE receives group paging (without being indicated to stay in RRC_INACTIVE)

Case 2: the UE can’t acquire PTM configuration for an active MBS session after mobility

Case 3: the reception quality of MBS multicast session is bad

When triggering the RRC Resume for these cases, the UE needs to know which resume cause to use. For case 1, it is straightforward to use MT-Access and this would mean the access control is not performed in such case. For case 2 and case 3, there are two possible options for the resume cause setting:

Option 1: reuse MO-data resume cause and access category

Option 2: introduce new resume causes and access categories

We think option 1 is not suitable and option 2 should be used based on the reasons below:

Reason 1: the MO-data is triggered by NAS while MBS resume is triggered by AS. As NAS is not aware of the above two cases, it cannot trigger it. Besides, the UE behavior after having been barred are different for AS triggered resume and NAS triggered resume. Therefore, separate cause and access category should be used to make RRC procedure work and to allow different treatment for AS and NAS triggered resume.

Reason 2: for case 2, if NW can know from the corresponding resume cause that this is only triggered for MBS multicast reception and not due to other services that require the UE to move to RRC_CONNECTED, the NW can directly send RRC release in response to the RRC resume request to keep the UE in RRC_INACTIVE for multicast reception. For case 3, if NW can know from the corresponding resume cause that the UE cannot receive multicast in RRC_INACTIVE any longer, the NW will let it resume.

Proposal 4a: Introduce new MBS resume causes and access categories for cases where the UE needs to resume to continue multicast reception actively.

Similar to the existing resume procedure, the UE may be barred in UAC procedure when the UE resumes to to continue multicast reception actively. In that case, the UE’s AS layer should re-initiate the procedure when barring is alleviated, similar as RNA update procedure.

Proposal 4b: If the resume procedure to continue multicast reception is barred or rejected, the UE’s AS layer should re-initiate the procedure when barring condition is alleviated, similar as RNA update procedure.

In the current RRC running CR, there is an Editor’s note about UE behaviour upon session activation when the PTM configuration was absent in RRCRelease, as following:

Editor’s note: FFS what is the UE behaviour when the session is activated, if the configuration was not configured in RRCRelease due to session deactivation.

In our understanding, the UE should resume after reception of group paging same as a legacy UE, if there was no PTM configuration (i.e., multicastConfigInactive-r18) in RRCRelease. Because anyway the NW should have control of whether to enable the Rel-18 feature and indicate it to the UE explicitly or implicitly via configuration, so that the UE can know whether it should behave as legacy or not. In other words, if there was no PTM configuration in RRCRelease, from both NW and UE’s perspective, the message is the same as Rel-17 and the Rel-18 feature is not enabled. So the UE should resume as a Rel-17 UE after receiving the Group Paging. Otherwise, if NW wants to enable the Rel-18 feature, at least an empty Rel-18 IE can be configured to the UE.

multicastConfigInactive-r18         SetupRelease { MulticastConfigInactive-r18 }                        OPTIONAL   -- Need M

MulticastConfigInactive-r18::=           SEQUENCE {

inactivePTM-Config-r18        OCTET STRING (CONTAINING MBSMulticastConfiguration)    OPTIONAL, -- Need N

inactiveMCCH-Config-r18         OCTET STRING (CONTAINING SystemInformation)             OPTIONAL  -- Need N

}

Observation 1: The NW should have control of whether to enable the Rel-18 feature. If the multicastConfigInactive-r18 is not configured in RRCRelease, UE cannot tell the Rel-18 feature is enabled and should regard the RRCRelease as a legacy IE.

Proposal 5: The UE should behave the same as the legacy Rel-17 UE if the multicastConfigInactive-r18 is not configured in RRCRelease, i.e., trigger resume upon receiving group paging.

2.2 Notification and state transition

Notification for session deactivation

In the current RRC running CR, there is an Editor’s note about notifying session deactivation, as following:

Editor’s note: FFS on the details of notifying session deactivation.

It has been agreed that MCCH is used for notifying MC session deactivation. In Rel-17, when MBS broadcast service stops, the UE is notified by MCCH change notification and the UE acquires MCCH to check whether the concerned service is stopped or not. Similar mechanism can be reused for multicast session deactivation, i.e. the UE is notified by MCCH-RNTI about session deactivation (the same bit as for session change notification can be reused), and then UE can learn which service is deactivated by reading MCCH. There are two options for MCCH content change in case MC session is deactivated:

Option 1: the configurations including TMGI are removed from MCCH for a deactivated MC session.

Option 2: the TMGI is kept and the corresponding configuration is removed

Since it was agreed that the UE needs to resume in case PTM configuration is not available in target cell during mobility for an active MC session, the UE needs to differentiate between a deactivated MC session and a MC session which is not provided for RRC_INACTIVE. In this case, Option 2 is more robust considering that the UE may miss change notification during cell reselection and Option 1 may cause unnecessary resume.

Proposal 6a: UE is notified by MCCH-RNTI about session deactivation (the same bit is reused as for session change notification in Rel-17).

Proposal 6b: In case a MC session is deactivated, the TMGI for this session is kept and corresponding configuration is removed from MCCH

MCCH-RNTI monitoring during session deactivation

Since it has been agreed to use group paging for session activation, it seems better to allow the UE not to monitor MCCH-RNTI during session deactivation period to save the UE power consumption in RRC_INACTIVE. The UE can start to monitor MCCH-RNTI after receiving group paging to acquire the latest PTM configuration for the concerned service.

Proposal 7: The UE is not required to monitor MCCH-RNTI during session deactivation and can start to monitor MCCH-RNTI upon session activation to acquire the latest PTM configuration for the concerned service.

2.3 Other Stage 3 RRC issues

MRB handling in RRC_INACTIVE

In the current RRC running CR, there is two Editor’s notes about MRB handling in RRC_INACTVIE, as follows:

Editor’s note: The details of multicast-inactive MRB establishment is FFS.

Editor’s note: The details of multicast-inactive MRB release is FFS.

Regarding how to handle the multicast MRB in RRC_INACTIVE, there are two possible options:

Option 1: the UE continues to use the multicast MRB configured in RRC_CONNECTED and updates the MRB configuration based on RRCRelease/MCCH, i.e. the UE does not suspend the multicast MRB that is related to the TMGI to be received in RRC_INACTIVE. When the UE resumes to RRC_CONNECTED, the network updates the MRB configuration by RRCResume message.

Option 2: the UE suspends the multicast MRB as legacy and establishes a new multicast MRB based on RRCRelease/MCCH. When the UE enters RRC_CONNECTED, it releases the multicast MRB used in RRC_INACTIVE and resume the suspended multicast MRB.

It has been agreed that the Rel-17 broadcast PTM configuration structure is taken as baseline for the PTM configuration structure on multicast MCCH. This means it is difficult to implement the delta configuration between the multicast MRB used in RRC_CONNECTED and the multicast MRB to be used in RRC_INACTIVE. While usually, the network needs to change some MRB configurations from RRC_CONNECTED to RRC_INACTIVE, due to the non-support of uplink in RRC_INACTIVE and also for the compliance to the minimal UE capabilities. For example, the network may need to disable EthernetHeaderCompression. Option 1 will make the procedure and also the description in specifications complex.

Compared with option 1, option 2 is cleaner and the UE just establishes a new multicast MRB for multicast reception in RRC_INACTIVE and the configuration used in RRC_CONNECTED can be suspended and resumed as legacy. Some may think that there might be some interruption during switch from the multicast MRB used in RRC_CONNECTED to multicast MRB used in RRC_INACTIVE. However, this can be minimized by UE implementation, e.g., by continuing the reception from the multicast MRB used in RRC_CONNECTED before the switch.

Proposal 8a: When the UE is released to RRC_INACTIVE to continue multicast reception, the UE suspends the multicast MRB(s) used in RRC_CONNECTED and establishes new multicast MRB(s) based on the configuration in RRCRelease/MCCH.

Proposal 8b: When the UE resumes to RRC_CONNECTED, The UE releases the multicast MRB(s) used in RRC_INACTIVE and resumes the suspended multicast MRB.

Terminology of multicast MRB used in RRC_INACTIVE

In the current RRC running CR, there is an Editor’s note about terminology of MRB, as follows:

Editor’s note: FFS whether an alternative terminology is used for multicast-inactive MRB.

As mentioned above, the multicast MRB used in RRC_CONNECTED and the multicast MRB used in RRC_INACTIVE are quite different from each other. Currently in RRC spec, multicast MRB refers only to the multicast MRB used in RRC_CONNECTED. So, to facilitate the Stage 3 work, it would be better to define a new terminology such as “multicast-inactive MRB” to differentiate it from the multicast MRB used in RRC_CONNECTED. Otherwise we will have to use a term like “multicast MRB used in RRC_INACTIVE” and change the current terminology “multicast MRB” to “multicast MRB used in RRC_CONNECTED” throughout the specifications.

Proposal 9: Define a new terminology as “multicast-inactive MRB” for the multicast MRB used in RRC_INACTIVE, to differentiate from the multicast MRB used in RRC_CONNECTED.

SDT and MBS multicast reception in RRC_INACTIVE

In the current RRC running CR, there is an Editor’s note about simultaneous configuration of SDT and multicast reception in RRC_INACTIVE, as follows:

Editor’s Note: FFS whether SDT and MBS multicast reception in RRC_INACTIVE can be configured together. And if yes, whether UE configured for MBS multicast reception in RRC_INACTIVE monitors group paging during SDT.

In our view, it is beneficial to allow to configure SDT and MBS multicast reception in RRC_INACTIVE together. Considering the scenario for multicast reception in RRC_INACTIVE is mainly due to NW congestion, the UE is possibly not expected to enter RRC_CONNECTED for small data transmission, and is better to stay in RRC_INACTIVE and use SDT.

Observation 2: It is beneficial to allow SDT and MBS multicast reception in RRC_INACTIVE to be configured together.

In Rel-17, the UE does not need to monitor group paging for MBS during SDT procedure, because the UE can be resumed to RRC_CONNECTED directly by the gNB when there is MBS service requirement. However, in Rel-18, if the UE is configured with multicast reception in RRC_INACTIVE, the UE should be informed of session activation via group paging in RRC_INACTIVE, even during SDT procedure. Therefore, the UE should monitor group paging during SDT when multicast reception in RRC_INACTIVE is configured.

Proposal 10a: The MBS multicast reception in RRC_INACTIVE and SDT can be configured together.

Proposal 10b: If configured with MBS multicast reception in RRC_INACTIVE, the UE monitors group paging regardless of whether SDT procedure is ongoing.

- Conclusion

Based on the above discussion we have the following observations and proposals:

### 3.1 Proposals for PTM configuration and mobility aspects

UE resume due to bad reception quality

Proposal 1a: The threshold used to trigger RRC resumption is based on signal quality, i.e., configuration among RSRP, RSRQ, or SINR.

Proposal 1b: The threshold used to trigger RRC resumption is configured in MCCH.

Proposal 1c: Further discuss whether the threshold is per service or per service group or per cell.

NCL for multicast

Proposal 2: NW should be able to provide the UE with the information on whether the same PTM configuration is being used in neighbour cells for multicast in RRC_INACTIVE.

Frequency prioritization

Proposal 3: Legacy mechanism of providing the dedicated frequency priority in RRC dedicated signaling can be reused for multicast frequency prioritization in RRC_INACTIVE.

Resume cause and access category

Proposal 4a: Introduce new MBS resume causes and access categories for cases where the UE needs to resume to continue multicast reception actively.

Proposal 4b: If the resume procedure to continue multicast reception is barred or rejected, the UE’s AS layer should re-initiate the procedure when barring condition is alleviated, similar as RNA update procedure.

Observation 1: The NW should have control of whether to enable the Rel-18 feature. If the multicastConfigInactive-r18 is not configured in RRCRelease, UE cannot tell the Rel-18 feature is enabled and should regard the RRCRelease as a legacy IE.

Proposal 5: The UE should behave the same as the legacy Rel-17 UE if the multicastConfigInactive-r18 is not configured in RRCRelease, i.e., trigger resume upon receiving group paging.

### 3.2 Proposals for Notification and state transition

Notification for session deactivation

Proposal 6a: UE is notified by MCCH-RNTI about session deactivation (the same bit is reused as for session change notification in Rel-17).

Proposal 6b: In case a MC session is deactivated, the TMGI for this session is kept and corresponding configuration is removed from MCCH

MCCH monitoring during session deactivation period

Proposal 7: The UE is not required to monitor MCCH-RNTI during session deactivation and can start to monitor MCCH-RNTI upon session activation to acquire the latest PTM configuration for the concerned service.

### 3.3 Proposals for Stage 3 RRC CR issues

MRB handling in RRC_INACTIVE

Proposal 8a: When the UE is released to RRC_INACTIVE to continue multicast reception, the UE suspends the multicast MRB(s) used in RRC_CONNECTED and establishes new multicast MRB(s) based on the configuration in RRCRelease/MCCH.

Proposal 8b: When the UE resumes to RRC_CONNECTED, The UE releases the multicast MRB(s) used in RRC_INACTIVE and resumes the suspended multicast MRB.

Terminology of multicast MRB used in RRC_INACTIVE

Proposal 9: Define a new terminology as “multicast-inactive MRB” for the multicast MRB used in RRC_INACTIVE, to differentiate from the multicast MRB used in RRC_CONNECTED.

SDT and MBS multicast reception in RRC_INACTIVE

Observation 2: It is beneficial to allow SDT and MBS multicast reception in RRC_INACTIVE to be configured together.

Proposal 10a: The MBS multicast reception in RRC_INACTIVE and SDT can be configured together.

Proposal 10b: If configured with MBS multicast reception in RRC_INACTIVE, the UE monitors group paging regardless of whether SDT procedure is ongoing.