3GPP TSG RAN WG2#123	R2-2307518

Toulouse, France, 21st - 25th August, 2023

Agenda item:	7.18.2

Source:	Nokia, Nokia Shanghai Bell

Title:	CP aspects for MT-SDT procedure

WID/SID:	NR_MT_SDT-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

RAN has agreed WI [1] for Mobile Terminated-Small Data Transmission (MT-SDT) with the following objectives:

Specify the support for paging-triggered SDT (MT-SDT) [RAN2, RAN3] - MT-SDT triggering mechanism for UEs in RRC_INACTIVE, supporting RA-SDT and CG-SDT as the UL response; - MT-SDT procedure for initial DL data reception and subsequent UL/DL data transmissions in RRC_INACTIVE. Note: Data transmission in DL within paging message is not in scope of this WI.

The following was agreed in RAN#120 meeting:

Agreements

- For RAN paging, MT-SDT indication (at least one bit) is explicitly included per UE via a paging message.  FFS if more information for MT-SDT are needed FFS what the indication will be called.  FFS signalling details

- Rel-18 MT-SDT after the MT-SDT paging trigger is detected, RA-SDT and CG SDT solutions/procedures specified in Rel-17 is re-used as a baseline.  The detailed triggers will be discussed on case by case.  FFS on resources used for access

- UE can use non-SDT random access resources for accessing the network for an MT-SDT transfer.  The UE can also use the configured grant resources and/or MO-RA resources.

- The network should be able to differentiate why the UL access was triggered, i.e. implicit or explicit indication by the UE.

- MT-SDT is data that belongs to bearers that are configured for SDT.    FFS whether the configuration is MO-SDT or MT-SDT specific.  The network can only trigger MT-SDT if the data belongs to those bearers.

- It is possible for the network to configure only MT-SDT without MO-SDT RA resources and/or CG-SDT.  Subsequent UL/DL data belonging to SDT bearers while in INACTIVE is allowed like MO-SDT procedure.  FFS stage 3 details

- New Resume cause in RRC resume will be introduced, one code point MT-SDT indication

The following was agreed in RAN#121 meeting:

Agreement:

- Include a one-bit indication in paging to trigger MT-SDT.   We will ensure that the CCCH message can be transmitted over CG.

- Indication is per UE.  FFS on signalling.

- In case condition for paging triggered MT-SDT is not fulfilled the UE initiates RRC Resume procedure. Resume cause FFS

- Upon receiving MT-SDT trigger, the UE shall initiate SDT procedure if the following checks are satisfied (all these same as Rel-17)

-     FFS 3a: Check for DVT (if UL data becomes available in UL)

-     3b: Check for SDT RSRP threshold

-     3c: Check for TA validation before selecting CG (if applicable)

-     3d: Check for SSB level RSRP threshold for CG resource (if applicable)

5.   When UE resumes for MT-SDT, UE resumes all RBs configured for SDT

6. 	RBs configured for SDT are common for MO-SDT and MT-SDT

7.	If there is valid CG-SDT resources, the UE should use CG-SDT to transmit the response.   FFS on whether we need to optimize for case when CG periodicity is too long

8. To confirm that when SDT is initiated due to MT-SDT, UE can exchange subsequent DL/UL SDT data on the resumed RBs. This clarifies the RB behaviour of related RAN2#120 agreement.

Agreements

- Specify a RRC procedure for RRCResume for MT-SDT initiation without checking for availability of UL data (i.e. if MT-SDT is initiated first the resume cause will be set to MT-SDT)

- UE is allowed to initiate either MO-SDT based resume or non-SDT based resume at any point (before initation RRCResumeRequest for MT-SDT) using separate procedures

- If MT-SDT procedure is initiated, for RACH during subsequent data transfer (i.e. RACH triggered due to SR), UE uses only the non-SDT RACH resources (i.e. like legacy)

The following was agreed in RAN#122 meeting:

Agreements:

- Allow support of only MT-SDT in a cell.  A separate SIB configuration will be introduced.  FFS what is put in there.

- For paging indication signalling, a new list of paging records for MT-SDT indication is optionally included in paging message using non critical extension. Each record in this list optionally includes 1 bit MT-SDT indication. UE identity and access type are not included in paging record of this list.

- gNB may include MT-SDT indication in paging message only if UE’s I-RNTI is included in the paging message (i.e. MT-SDT is only used by RAN initiated paging).

- UE selects '0' as the Access Category when the resumption of the RRC connection is triggered by response to the MT-SDT triggering in a PAGING message

- MT-SDT is only applicable to the legacy MT-Access use case (i.e. it is not applicable to access identities 1, 2 and 11-15).

- SRB2 can be used for MT-SDT (i.e. similar to MO-SDT)

- No additional enhancement is needed specifically for RedCap UE to monitor paging for MT-SDT

- When RRC resume is triggered due to MT-SDT and in the case the condition for paging triggered MT-SDT is not fulfilled, the UE initiates RRC Resume procedure with Resume cause “mt-Access”.

- A separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell.

Agreements:

- RRC explicitly indicates to MAC whether resume is trigged due to MT-SDT

- LCH restrictions are checked for DRBs as in MO-SDT (if UL data is available during SDT procedure).  Ensure CCCH can be transmitted in CG-SDT when MT-SDT is triggered in stage 3 discussion.

- Assumption is that if the UE has UL data the UE can still check and trigger MO-SDT (it is up to UE implementation)

- RA-SDT resources are not used for MT-SDT initiation RACH

- In case CG-SDT resources cannot be used or are not available for MT-SDT, UE uses non-SDT RACH for RA-based MT-SDT.  FFS whether new triggers are defined

- There is no need to define new Rel-18 CG configurations specific to MT-SDT.

- When the UE is configured with both MO and MT SDT the radio bearer configuration is common for both.

- For both MO and MT-SDT, if the next CG-SDT resource is too far, then RACH resource can be selected first.   This is checked at the point of initial resource selection (e.g. CG SDT selection).   FFS what is too far and how this is configured.   Assumption is that we will continue this discussion in SDT session.  CONFIRM with main session [CB]

In this contribution we discuss further details on MT-SDT procedure.

# 2	Discussion

UE assistance on RRC state preference during SDT procedure was discussed in Rel-17 [2][3], but it was not agreed. There was some support it and some companies thought that it could be introduced in Rel-18. Therefore, we would like to discuss whether it could be introduced in Rel-18.

The UE can indicate its RRC state preference in RRC CONNECTED state using UE assistance information procedure. The UE may prefer idle, inactive, connected or outOfConnected. RRC state preference indication seems beneficial from UE power saving perspective also during SDT procedure which may last up to 4 seconds. However connected indication is not needed, because it has been specified that the UE can transmit non-SDT data indication in case non-SDT UL data becomes available for transmission. Also outOfConnected indication would not be needed, because the UE is not in connected state during the SDT procedure. Therefore, we propose the following:

Proposal 1: The UE can indicate its RRC state preference (IDLE or INACTIVE) during SDT procedure using UE Assistance Information procedure.

For REL-17 SDT-ConfigCommonSIB is provided via SIB1. It has been agreed that a separate sdt-RSRP threshold for MT-SDT can be configured, at least in the case where MO-SDT is not configured in the cell. It would be straight forward to define SDT-ConfigCommonSIB for MT-SDT as well.

Proposal 2: MT-SDT specific SDT-ConfigCommonSIB is provided via SIB1

RRC Resume Request may include short or long version of I-RNTI. Long version of the I-RNTI may accommodate more bits for the UE and cell identifier. It has been agreed that in case CG-SDT the configuration is valid only for the same serving cell.

Proposal 3: Short version of the I-RNTI is used in RRC Resume Request for CG-SDT.

# 3	Conclusion

Based on the discussion the following is proposed:

Proposal 1: The UE can indicate its RRC state preference (IDLE or INACTIVE) during SDT procedure using UE Assistance Information procedure.

Proposal 2: MT-SDT specific SDT-ConfigCommonSIB is provided via SIB1

Proposal 3: Short version of the I-RNTI is used in RRC Resume Request for CG-SDT.

# Reference

[1] RP-213583, New WI: Mobile Terminated-Small Data Transmission (MT-SDT) for NR, ZTE Corporation

[2] R2-2208987, Report of [AT119-e][301][Sdata] CP Open issues and CR to 38.331 (ZTE), ZTE Corporation

[3] R2-2208218, RRC state preference during SDT procedure, Nokia, Nokia Shanghai Bell