3GPP TSG-RAN WG2 Meeting #123	R2-2307524

Toulouse, France, 21st – 25th August, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on cell DTX and DRX

Agenda Item:	7.3.2

Document for:	Discussion and decision

# Introduction

In the RAN2#121-bis meeting, the cell DTX/DRX was discussed, with the following agreements [1]:

Agreements

- A periodic cell DTX/DRX configuration is explicitly signalled to the UEs.

- A periodic cell DTX/DRX pattern is configured by UE specific RRC signalling.

- The Cell DTX/DRX configuration contains at least: periodicity, start slot/offset, on duration.

- As a baseline Cell DTX/DRX is activated/deactivated implicitly by RRC signalling, i.e. activated immediately once configured by RRC and deactivated once the RRC configuration is released.

- From RAN2 point of view, majority companies see a benefit with L1 signalling for Cell DTX/DRX activation/deactivation, send a LS to RAN1 (email 308) with our preference and ask about feasibility and design details.   Ask about feasibility and reliability of using L1 signaling.  Clarify that the question is about activation/deactivation copy the agreement from last meeting that we are focusing on single configuration.  Extract a few key benefits of dynamic signaling from email discussion and online discussions

- As baseline, UE doesn’t monitor SPS occasions during Cell DTX non-active period. As baseline, gNB is assumed to be not transmitting PDSCH to that UE on such SPS occasions during the Cell DTX non-active period

- As baseline, UE does not transmit on CG occasions during Cell DRX non-active periods

- As baseline, UE does not transmit SR occasions overlapping with Cell DRX non-active periods, e.g. SR transmissions are dropped during the non-active period

FFS: whether we will allow to configure the UE per SR configuration with whether SR can be transmitted during Cell DRX non-active period to support high priority traffic

- (for the SRs that will be dropped) If SR is not to be transmitted on an PUCCH occasion during Cell DRX non-active time, the UE keep the SR pending, i.e., the UE delays the SR transmission till the Cell DRX active period without triggering RACH.  For the FFS case there may be some exceptions.

- The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time.   UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time.   FFS how to deal with any exceptions (e.g. SR if agreed and RACH).

FFS how to deal with retransmissions

In the RAN2#122 meeting, the cell DTX/DRX was discussed, and the following agreements were reached [2]:

Agreements:

1	UE monitors PDCCH for RAR during Cell DTX non-active time. The ra-ResponseWindow could be started as legacy.

2	UE monitors PDCCH for msg4 during Cell DTX non-active time. The ra-ContentionResolutionTimer could be started as legacy.

3	Working assumption:  When the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy.  It is up to the network whether it schedules retransmissions out of the Cell DTX active period, i.e., when the DRX retransmission timer is running, the UE should monitor PDCCH regardless of the Cell DTX.

4	Once gNB recognizes there is an emergency call or public safety related service (e.g. MPS/MCS), the NW should ensure there is no impact to the emergency call (e.g. may deactivate Cell DTX/DRX).  The behavior is captured in stage 2 spec

5	When an DG grant is received, scheduled by the gNB during cell DRX/DTX, the UE follows the grant assignment (i.e. like in legacy).  This includes DL HARQ feedback.

In this paper, we will further discuss the remaining issues of the cell DTX/DRX mechanism.

# Discussion

## Cell DTX/DRX (de)activation

The reply LS [3] from RAN1 about cell DTX/DRX (de)activation informed RAN2 that group common L1 signalling using PDCCH will be supported to dynamically (de)activate cell DTX/DRX. RAN1 subsequently asked RAN2 to consider additional support of MAC CE based indication. In our understanding, UE specific RRC based implicit activation and group-common L1 signalling indication are enough to indicate to a single UE or a group of UEs to activate or deactivate cell DTX/DRX. Moreover, L1 signalling is faster than L2 in transmitting the (de)activation indication and dedicated L3 is robust. We do not see further benefit in introducing additional MAC CE based indication for cell DTX/DRX (de)activation.

- RAN2 will not introduce additional MAC CE based indication for cell DTX/DRX activation and deactivation.

## 2.2 The gNB/UE behaviour of cell DTX/DRX

In previous meetings, we have discussed many behaviours related to the scheduling assignments. In RAN2#121-b meeting, we have agreed that the UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. However, the group common PDCCH (DCI format 2_0 ~ DCI format 2_6) monitoring behaviour is still unclear. For PDCCHs associated with DCI format 2_0 ~ 2_5, cell DTX can apply to them during cell DTX non-active period and UE can stop monitoring them to achieve more UE energy saving gains. But for DCI format 2_6, which is used for notifying the power saving information outside DRX Active Time for one or more UEs, cell DTX should not apply as the UEs need to monitor it to wake up from DTX non-active time. As the reply LS indicated, the new DCI format 2_X will be used for cell DTX/DRX (de)activation, a UE in cell DTX non-active period should also monitor the new DCI format 2_X for dynamic cell DTX/DRX (de)activation.

- PDCCH associated with DCI format 2_6 and new DCI format 2_X should be monitored by the UE during the cell DTX non-active period.

## 2.3 Cell DTX/DRX configuration and activation granularity in the CA scenario

When cell DTX/DRX mechanism is applied in the CA scenario, there are two options of cell DTX/DRX configuration and activation granularity: per MAC entity and per serving cell. The decision which option to choose has impact on the way MAC and RRC specifications are implemented. Currently the only agreement of the WI phase in this regard is that the pattern configuration for cell DRX/DTX is common for Rel-18 UEs in the cell.

In the per MAC entity option, which is similar to how C-DRX is configured, all serving cells should have the same cell DTX/DRX parameters to align with the one C-DRX configuration. In this option the activation status, if needs to be set on a per cell basis, would need signalling which conveys also information to which of the serving cells the activation/deactivation status applies. This mode is simpler from the UE implementation perspective but has limitations in the NW configuration and it may be difficult to apply separate activation status across serving cells.

For the per serving cell option, different serving cells can have different cell DTX/DRX parameters, which brings flexibility of NW implementation but complexity of UE implementation. In this scenario it would also be easier to convey the activation status as the configuration would be per cell and each serving cell could have a different cell DTX/DRX state.

- RAN2 to discuss whether Cell DTX/DRX configuration and activation status is per MAC entity or per Serving Cell.

## 2.4 Interaction of UE C-DRX and cell DTX

### 2.4.1 UE C-DRX behaviours under cell DTX

We have discussed gNB/UE behaviour of cell DTX/DRX, but the UE C-DRX behaviours under cell DTX is not clear. Based on the agreements, during the cell DTX active period, the gNB and UE behaviours remain unchanged. Therefore, the UE behaviours of C-DRX can also remain unchanged. In our view, during the cell DTX non-active period, the UE should change its behaviours according to the cell DTX configuration, regardless of its C-DRX parameters to save UE power. Fig. 1 shows an example, where a UE can normally wake up or sleep for C-DRX during the cell DTX active period, but transitions to C-DRX sleep (stops PDCCH monitoring) during the cell DTX non-active period since the network will not schedule anything for this UE.

The existing UE C-DRX behaviours mainly depend on the related timers, namely drx-onDurationTimer, drx-InactivityTimer and drx-RetransmissionTimer. The UE should be in the C-DRX active time when these timers are running. When the cell enters cell DTX non-active period, there needs to be a change in existing UE C-DRX behaviours. e.g. drx-InactivityTimer stops or is paused. This can make the UEs stop PDCCH monitoring during cell DTX non-active period and save UE power. In the last meeting, we have agreed that when the retransmission timer is running (if C-DRX is configured), the UE is expected to monitor PDCCH, like in legacy. But the behaviour of drx-HARQ-RTT-Timer is not clear. When the drx-HARQ-RTT-Timer is running, the UE will not monitor PDCCH. When the timer expires, the drx-RetransmissionTimer will be started and UE will monitor PDCCH for retransmission. In our view, it will waste UE energy to start the RetransmissionTimer after the HARQ timer expires. The gNB in sleep mode will not transmit the failed scheduled data. In such case, the HARQ timer may be stopped/paused during the Cell DTX/DRX non-active period and can be resumed/restarted in Cell DTX/DRX active period. The RetransmissionTimer will be started only after the HARQ timer expires in the cell DTX/DRX active period.

Fig.1 UE changing its existing C-DRX behaviours according to the Cell DTX

- Define new conditions for UE behaviour changes when cell DTX non-active period starts during the C-DRX related timer is running, i.e. the drx-InactivityTimer and HARQ timer should be stopped/paused.

When the UE is scheduled with a PDCCH, the NW may configure it with a bwp-InactivityTimer. If a UE does not receive the scheduled DCI within the timer, the BWP will switch to the default downlink BWP to save UE’s energy, since there is no more traffic. However, during the cell DTX non-active period, there will be no PDCCH dynamic scheduling, which will lead to unnecessary switching to the default downlink BWP after the timer expires. If there is a PDCCH scheduled in the next cell DTX active period, it will switch to the larger BWP, which will lead to an overhead and decrease the efficiency and performance due to the delayed NW scheduling. Such mechanism can cause frequent BWP switching with the cell DTX/DRX enabled (Fig.2). Therefore, the bwp-InactivityTimer should be paused during cell DTX non-active period and resumed running during cell DTX active period.

A similar issue can be observed in the SCell which should not be frequently (de)activated when the cell enters cell DTX/DRX active or non-active period. Thus, the sCellDeactivationTimer should also be paused due to the same reason as bwp-InactivityTimer.

Fig.2 Frequent default DL BWP fallback due to bwp-InactivityTimer expiry

- The bwp-InactivityTimer and sCellDeactivationTimer should be paused during cell DTX non-active period and resumed during cell DTX active period to decrease unnecessary BWP fallback and SCell deactivation.

### 2.4.2 UE C-DRX aligned with the cell DTX

When a cell has activated cell DTX and the connected UE has activated UE DRX, there are three alignment possibilities between the cell DTX and the UE DRX:

- No alignment: UE C-DRX on duration has no overlap with the cell DTX on duration period.

- Partial alignment: part of UE C-DRX on duration overlaps with the cell DTX on duration period.

- Full Alignment: the UE C-DRX on duration fully overlaps with the cell DTX on duration period.

Since there will be no DL traffic during the cell DTX non-active period, no PDCCH transmission will take place during this time. The cell DTX on duration period is the guaranteed active period that can transmit PDCCH to UEs, no matter whether there will be active period extension. If there is no alignment between UE C-DRX on-duration and cell DTX/DRX on-duration, the UE cannot be successfully scheduled. Consequentially, the alignment between UE C-DRX and cell DTX/DRX is necessary and beneficial for power consumption and transmission reliability reasons. Some companies explain that it is up to NW implementation to choose appropriate configurations of UE C-DRX and Cell DTX. But if there is no overlapping between the UE and the gNB, the UE cannot be successfully scheduled.  Therefore, when Cell DTX is activated, the UE should adopt a UE DRX at least partially aligned with the Cell DTX.

Observation 1: The alignment between UE C-DRX on-duration and cell DTX/DRX on-duration is necessary and beneficial for UE power consumption and transmission reliability reasons.

The alignment of UE C-DRX with a certain Cell DTX does not result in a on/off pattern exactly the same as the on/off pattern of the Cell DTX. As shown in Fig.3, the on/off pattern of an aligned UE DRX could vary, e.g. have different duration, periodicity, or offset, but still fall within the cell DTX active time. Various UEs can have different DRX patterns, according to traffic characteristics and network scheduling requirements, and still be aligned with a certain Cell DTX.

Fig.3 Different UE DRX patterns aligned with a certain Cell DTX (if Cell DTX is activated).

When the UE C-DRX on duration overlaps with the cell DTX on duration period for all UEs, there is no need to further limit the configurations of UE C-DRX, e.g. by requiring all the UE C-DRX to have the same offset. Some companies think that the offset alignment may increase NES gains. However, the NES gains of cell DTX mainly come from the fixed cell DTX non-active period. Whether extra mechanisms can be enabled during cell DTX active period to have extra NES gains, e.g. C-DRX adjustment (align offset/shorten on-duration), intelligent scheduling, should depend on NW implementations. When the scheduling of different UEs is distributed across the cell DTX on-duration period instead of concentrated on a part of it, NW can configure suitable C-DRX configurations for these UEs to avoid wasting UE energy.

- If Cell DTX is activated, an aligned UE C-DRX configuration with Cell DTX means that the C-DRX on-duration falls within the cell DTX on-duration period, and the offset, periodicity or on-duration of the aligned C-DRX should be determined by NW implementation.

- The periodicity of UE C-DRX configurations in a cell should be same or a multiple of the serving Cell’s DTX periodicity, and the on-duration cannot exceed the Cell DTX on-duration.

When cell DTX is activated, UEs should adopt an aligned C-DRX that may differ from the current C-DRX pattern. And when cell DTX is deactivated, the aligned C-DRX might be released and new C-DRX may be configured for UEs. In Email discussion, we have discussed how to reconfigure multiple UE C-DRX patterns when activating/deactivating cell DTX. The existing mechanism is to reconfigure C-DRX of UEs by UE-specific RRC messages. It is not dynamic enough and may cause high signalling overhead when cell DTX is activated/deactivated. RAN2 should discuss how to efficiently change the C-DRX of UEs when cell DTX is activated/deactivated without increasing signalling overhead. One of the options is to add an alternative C-DRX configuration to the cell DTX parameter set so that the UE would have one legacy and one cell DTX specific C-DRX. The second one would be activated and deactivated together with cell DTX. In that way signalling overhead of the alignment reconfiguration would be significantly reduced.

- RAN2 to introduce an additional UE C-DRX configuration which is used by the UEs once cell DTX is activated for efficient alignment of C-DRX with cell DTX/DRX.

# Conclusion

In this paper, we discussed the details of the Cell DTX/DRX mechanism, and made the following proposals:

- RAN2 will not introduce additional MAC CE based indication for cell DTX/DRX activation and deactivation.

- PDCCH associated with DCI format 2_6 and new DCI format 2_X should be monitored by the UE during the cell DTX non-active period.

- RAN2 to discuss whether Cell DTX/DRX configuration and activation status is per MAC entity or per Serving Cell.

- Define new conditions for UE behaviour changes when cell DTX non-active period starts during the C-DRX related timer is running, i.e. the drx-InactivityTimer and HARQ timer should be stopped/paused.

- The bwp-InactivityTimer and sCellDeactivationTimer should be paused during cell DTX non-active period and resumed during cell DTX active period to decrease unnecessary BWP fallback and SCell deactivation.

- If Cell DTX is activated, an aligned UE C-DRX configuration with Cell DTX means that the C-DRX on-duration falls within the cell DTX on-duration period, and the offset, periodicity or on-duration of the aligned C-DRX should be determined by NW implementation.

- The periodicity of UE C-DRX configurations in a cell should be same or a multiple of the serving Cell’s DTX periodicity, and the on-duration cannot exceed the Cell DTX on-duration.

- RAN2 to introduce an additional UE C-DRX configuration which is used by the UEs once cell DTX is activated for efficient alignment of C-DRX with cell DTX/DRX.

# References

- R2-2304203 Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

- R2-2306543, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

- R1-2306246, Reply LS on Cell DTX/DRX activation/deactivation