3GPP TSG-RAN WG2 Meeting #123	R2-2307525

Toulouse, France, 21st – 25th August, 2023

Source: 	Huawei, HiSilicon

Title: 	Discussion on cell selection/reselection for NES

Agenda Item:	7.3.4

Document for:	Discussion and decision

# Introduction

In RAN2#121 meeting, the cell selection/reselection for NES was discussed with the following agreements [1]:

Agreements:

- RAN2 confirms that non-NES UEs can access to NES cells if NES solution is backwards compatible

In RAN2#122 meeting, the following agreements on the cell selection/reselection for NES were achieved [2]:

Agreements:

- We will define UE capabilities with signaling.  Details are FFS and will be discussed later during the WI phase.

- Separate camping restrictions for NES-capable and non-NES UEs will be defined.  FFS if it is a single bit or more.

In this paper, we discuss further details of cell selection/reselection for NES.

# Discussion

The main intention of preventing legacy UEs from camping on NES enabled cells is to make sure that the NES techniques activated can maximise the network’s NES gain, which may be hindered due to the existence of legacy UEs. For example, in a cell with many legacy UEs which do not understand the configuration of cell DTX/DRX, the opportunity for the activation of cell DTX/DRX would be low and the NES gain of this cell would be small. Therefore, it is beneficial to prevent UEs that do not support a specific NES technique from camping on/accessing to cells that utilise this specific NES technique. In case a cell wants to activate multiple NES techniques, the UEs camping on this cell should support all of these NES techniques, and the NW should have the possibility to bar UEs that do not support these NES techniques.

The remaining issue from the last meeting is to discuss UE capabilities signalling. The NW should maintain such information to decide if it should prevent the non-NES capable UEs from camping on the NES cells. The legacy UE capability transfer mechanism can be reused for the network to retrieve UE NES capabilities. The network can initiate the procedure to request for the NES capabilities of UEs in RRC_CONNECTED. After receiving the request, the UE should report its NES capabilities information in the response message.

The detailed RAN2 related UE NES capability information can include whether to support cell DTX/DRX, CHO enhancement and SSB-less SCell. In total, at least 3 bits are needed to represent the corresponding capability of Rel 18 NES techniques. More bits will be added once RAN1 and RAN4 input is received. The granularity of the capability signalling is still to be decided. Since these NES techniques are for the network energy saving gain, the feature support should be optional for Rel-18 UEs. Whether they should have FDD-TDD or FR1/FR2 differentiation can be decided in the later stage after discussion in RAN2 takes place. Some features are already defined as supported only in FR1, like e.g. SSB-less SCell operation for inter-band CA for FR1 and co-located cells. Other RAN WGs led NES techniques UE capabilities are waiting for further discussion in the corresponding WGs.

Proposal 1: The legacy UE capability transfer procedure can be reused for network to retrieve UE NES capabilities. At least 3 bits are needed to represent the corresponding capability of Rel 18 RAN2 led NES techniques. More UE capability bits will be added once RAN1 and RAN4 input is received on their corresponding NES techniques.

UEs that do not support a concerned NES technique refers to two types of UEs – one is R15/R16/R17 UEs and the other is R18 UEs that do not support the concerned NES technique. For the former ones, the legacy mechanisms and legacy parameters/IEs (e.g. IntraFreqExcludedCellList/InterFreqExcludedCellList, cellBarred, cell reservation) can be used for barring, since such UEs support none of the NES techniques. For the latter ones, new mechanisms should be introduced to bar UEs according to which NES technique(s) the cell utilises. As a first step RAN2 should introduce a new single field in SIB1 to enable separate barring of legacy and NES Rel-18 UEs. More indications of the supported NES techniques can be added in SIB1 to enable better barring granularity, if found necessary by RAN2.

If the cellBarred field in MIB is set to barred, the NES-capable UE should receive and decode SIB1 to see if it is allowed to access this cell. From our current analysis, only cell DTX/DRX mechanism has a potential NES gain decrease if non-cell-DTX/DRX-capable UEs would camp on such NES enabled cell. For the other NES techniques, there is no legacy impact on NES gain identified. Hence, one bit is enough to indicate the cell DTX/DRX NES barring mechanism. The field can be named cellBarredCellDTXDRX for clarity on what feature it relates to. If any other NES techniques are foreseen to have impact on NES, we are open to add more bits to indicate barring for the corresponding NES techniques.

Similarly to NTN, the Rel-18 NES capable UE would ignore the cellBarred field in MIB, and the barring mechanism of cell DTX/DRX would be set with one bit in SIB1, i.e.:

cellBarredCellDTXDRX-r18            ENUMERATED {barred, notBarred}                                         OPTIONAL,  -- Need R

Proposal 2: Introduce a new single field cellBarredCellDTXDRX-r18 in SIB1 to enable separate barring of legacy and NES UEs. If better barring granularity is found necessary by RAN2, the barring mechanism can be extended.

# Conclusion

In this paper, we discussed the cell selection/reselection for NES and made the following proposals:

Proposal 1: The legacy UE capability transfer procedure can be reused for network to retrieve UE NES capabilities. At least 3 bits are needed to represent the corresponding capability of Rel 18 RAN2 led NES techniques. More UE capability bits will be added once RAN1 and RAN4 input is received on their corresponding NES techniques.

Proposal 2: Introduce a new single field cellBarredCellDTXDRX-r18 in SIB1 to enable separate barring of legacy and NES UEs. If better barring granularity is found necessary by RAN2, the barring mechanism can be extended.

# References

- R2-2301903 Report from Session on NES, UAV, Small Data, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

- R2-2306543 Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning