3GPP TSG-RAN WG2 Meeting #123	R2-2307539

Toulouse 21th -25th Aug, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the Temporary UE Capability Restriction Procedure

Agenda item:	7.17.2

Document for: 	Discussion and Decision

- Introduction

In the last meeting, many agreements have been achieved for the temporary capability restriction.

Agreements => Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities => UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case. => UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.  =>Support “early indication” from UE to network during RRC connection setup/resume procedure. =>FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. => FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

In this paper we share our views on the temporary capability reporting procedure based on these agreements.

- Discussion

In this chapter, we first discuss the common framework for the reactive and proactive approach, and then discuss how to indicate the scell/SCG release explicitly, at last we will share our views on the details of “early indication”.

  - Common Framework for the Reactive and Proactive Approach

As agreed in the last meeting, the proactive approach would also be considered in addition to the reactive approach.

Ran2#121bis => Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

Before further discussing the procedure for the proactive approach, it’s better to have a clear picture on the UAI based procedure of the reactive approach. For the UAI based approach, there are main 3 steps for the normal case:

- Step 1: NW signals via RRC that temporary capability restriction is allowed.

- Step 2: UE reports the temporary capability restriction in the UAI

- Step 3: Network response the UE (e.g. Reconfiguration message)

For the step 1 and 2, some agreements have been achieved in the previous meeting.

RAN2#121 The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM.

For the step 3, if the network accept the temporary capability restriction, the network can send reconfiguration message according to the capability restriction. The remaining issue is about the Timer for NW response for capability restriction signaling and UE’s behavior on timer expiry. According to the companies contributions, several methods were proposed as in [1] [2] [3] [4].

- Option 1: UE wait for network’s response within a prohibit timer, after timer expiry, the UE can request to leave NW A (e.g. using Rel-17 MUSIM leave indication)

- Option 2: UE wait for network’s response within a wait timer, after timer expiry, the UE can applies a default configuration.

- Option 3: UE wait for network’s response within a wait timer, after timer expiry, the UE is allowed to use the temporary updated capability autonomously.

Obviously, the main difference is the UE’s behavior on timer expiry. Thus we can get a first easy proposal as below:

Proposal 1: After the UE send the UAI for the temporary capability restriction, the UE shall wait for network’s response within a timer, the timer is configured by the network. FFS on the UE’s behavior on timer expiry.

Based on the proposal 1, we can further discuss the procedure of the proactive approach. The main difference between the reactive and proactive approach is whether the requested capabilities restriction is dependent on the current configuration. For the proactive approach, the UE may send the temporary capability restriction to avoid future collision thus the network doesn’t need to reconfigure the UE currently.

Observation 1: For the proactive approach, the network may don’t need to reconfigure the UE currently.

If the observation 1 is agreed, then for the procedure of the proactive approach, the network need to give some response to the UE to avoid the UE wait for the timer expiry unnecessarily, which would also affect the UE’s performance significantly.

Proposal 2: For the case that the network need to reconfigure the UE currently (e.g. proactive approach), the network shall still give UE a response. FFS on the response detail.

Based on the proposal 1 and proposal 2, it is recommended to specify only a common procedure for the proactive and Reactive approach. Furthermore, whether to trigger a proactive or a reactive MUSIM UAI reporting shall left to the UE implementation.

Proposal 3: Whether to trigger a proactive or a reactive MUSIM UAI reporting can be left to the UE implementation.

  - Explicitly scell/SCG release

For the scell/SCG release, in the last meeting, it has been agreed that the UE can explicitly request the scell/SCG release.

=> UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

To adopt the common framework as the above temporary capability restriction, for this that the UE can indicate the Scell Index to the network to indicate the scell release, if the UE need to release the SCG, the UE can indicate the PScell index.

– ServCellIndex The IE ServCellIndex concerns a short identity, used to uniquely identify a serving cell (i.e. the PCell, the PSCell or an SCell) across the cell groups. Value 0 applies for the PCell, while the SCellIndex that has previously been assigned applies for SCells.

Proposal 4: For the explicitly scell/SCG release, the UE can request scell release by indicating the Scell Index and indicate SCG release by indicating the PScell Index.

The left issue is about how/whether this works for the proactive case, considering that for the proactive case the UE can request capability restrictions independent of current RRC configuration while the serving cell/SCG is the current configuration, thus the scell/SCG release can be seen as an reactive case.

Proposal 5: Scell/SCG release doesn’t work for the proactive case.

  - Early Indication

About the early indication, some FFS issues are left.

=>Support “early indication” from UE to network during RRC connection setup/resume procedure. =>FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted. => FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

FFS how to indicate this and in which message.

For the connection setup procedure, the UE can indicate the “early indication” in the message 5 with 1 bit.

Proposal 6: For the connection setup procedure, the UE can indicate the “early indication” in the message 5 with 1 bit.

For the connection Resume procedure, for some cases the connection may can’t be resumed successfully if the network doesn’t know that there is a temporary capability restriction, and thus the UE has to send the early indication in the Resume request message to avoid resume failure. However, because of the size limitation of the resume request message, it’s not desirable to use the remaining 1 spare bit for MUSIM feature.

Proposal 7: For the connection resume procedure, the UE can indicate the “early indication” in the message 5 with 1 bit.

To avoid unnecessary resume failure, as an implementation method, the UE can enter idle state directly and triggering the RRC connection establish procedure as shown in Fig 1.

Fig 1: Back to Idle for the Temporary Capability Restriction

Proposal 7a: When there is a temporary capability restriction at the Inactive state that may lead to resume failure, as an implementation method, the UE is allowed to go to the idle state directly and establish the connection with connection setup procedure.

When UE can indicate the “early indication”

As agreed in the previous meeting, the UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. Similarly, this principle shall also be complied for the UE to send the “early indication”, thus the network also needs to indicate that it supports “early indication” in the system Information.

Proposal 8: Introduce an indication in SIB1 to indicate whether the UE is allowed to send temporary capability restriction indication in Msg5.

How does it relate to UAI

About how does it relate to UAI, the common framework of the UAI reporting shall be complied, that is the UAI can only be sent after the network configure it through the Other Configure of the Reconfiguration message as shown in Fig 2.

Fig 2: Procedure for the UE Capability Restriction Early Indication

Proposal 9: After sending the early indication in the msg5, the common framework of the UAI reporting shall be complied, that is the UAI can only be sent after the network configure it through the Other Configure of the Reconfiguration message

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: After the UE send the UAI for the temporary capability restriction, the UE shall wait for network’s response within a timer, the timer is configured by the network. FFS on the UE’s behavior on timer expiry.

Observation 1: For the proactive approach, the network may don’t need to reconfigure the UE currently.

Proposal 2: For the case that the network need to reconfigure the UE currently (e.g. proactive approach), the network shall still give UE a response. FFS on the response detail.

Proposal 3: Whether to trigger a proactive or a reactive MUSIM UAI reporting can be left to the UE implementation.

Proposal 4: For the explicitly scell/SCG release, the UE can request scell release by indicating the Scell Index and indicate SCG release by indicating the PScell Index.

Proposal 5: Scell/SCG release doesn’t work for the proactive case.

Proposal 6: For the connection setup procedure, the UE can indicate the “early indication” in the message 5 with 1 bit.

Proposal 7: For the connection resume procedure, the UE can indicate the “early indication” in the message 5 with 1 bit.

Proposal 7a: When there is a temporary capability restriction at the Inactive state that may lead to resume failure, as an implementation method, the UE is allowed to go to the idle state directly and establish the connection with connection setup procedure.

Proposal 8: Introduce an indication in SIB1 to indicate whether the UE is allowed to send temporary capability restriction indication in Msg5.

Proposal 9: After sending the early indication in the msg5, the common framework of the UAI reporting shall be complied, that is the UAI can only be sent after the network configure it through the Other Configure of the Reconfiguration message

- Reference

- R2-2303409	Procedures for MUSIM temporary capability restriction	Apple

- R2-2303639	Overall Dual-RX/TX MUSIM procedure	Ericsson

- R2-2303267	Procedures for MUSIM temporary capability restriction	vivo

- R2-2303470	Further discussion on MUSIM temporary capability restrictions	Huawei, HiSilicon