3GPP TSG-RAN WG2 Meeting #123	R2-2307540

Toulouse 21th -25th Aug, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the Temporary UE Capability Restriction

Agenda item:	7.17.3

Document for: 	Discussion and Decision

- Introduction

In the last meeting, many agreements have been achieved for the temporary capability restriction.

Agreements Ran2#121 => For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted: - Transmission and reception capabilities (e.g. MIMO layers) - Measurement capabilities (e.g. gaps) - Supported bandwidth - Supported band-combinations (FFS whether this is CA or DC or both) => FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion). => RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details. Ran2 #121 bis: => For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. => For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B. => For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI. FFS if this is only for NW A or also NW

Besides, some FFS issues as below are also added as Editor’s note in [1]:

Editor note: FFS what granularity is used for indicating temporary maximum number of MIMO layers. Editor note: FFS whether one configuration to control all temporary capabilities update or introduce individual control for each temporary capability update. Editor note: FFS on how to report the supported band-combinations and the bandwidth.

In this paper we share our views on the temporary capability restriction detail based on these agreements and FFS issues.

- Discussion

In this chapter, we first discuss how to report the supported band-combinations and the granularity of the MIMO layer/ bandwidth, then we discuss how to configure the UE to report the temporary capability restriction at network side.

  - Temporary Capability Restriction reporting

As agreed in the previous meeting, the MIMO layers, bandwidth and the supported band combinations would be impacted, and the granularity of these capacities would also pending on the band conflict discussion. In this chapter, we first discuss the restriction on the BC, then analyze the granularity of the MIMO layers and bandwidth.

Band Combination Restriction

According to the current UE capability structure, the UE would report the supportedBandCombinationList according to the network’s request and the based band and RF capabilities are mainly reported per BC as shown in the Fig 1.

Fig 1: UE capability Structure

The network would configure the UE according to these capabilities. When the UE need to establish another connection or change the configuration on the other network, some of the band combinations may be conflict with the working band on the other network. Generally, there are 2 collision types:

- Collision type 1 : Some Band or Band combinations are forbidden at network A because of the working band of network B;

- Collision type 2: Some Band or Band combinations are f are supported but with lower capabilities (e.g. with lower MIMO layer and/or lower bandwidth) because of the working band of network B.

Based on these 2 collision types, 2 BC types can be defined as below:

- Type 1: The BCs that are forbidden because of the collision;

- Type 2: The BCs that are supported but with lower capabilities (e.g. with lower MIMO layer and/or lower bandwidth) because of the collision.

The Type 1 is forbidden BC, it means that the network A can ignore this BC or take this BC was not supported. For example, the UE is trying to connect network B with band x, but the UE can’t work at band x and band y together, then the BC with band y would be taken as the forbidden BC.

For the Type 2 BC, as described above, the UE can only support the BC but with lower capabilities. For example, the UE report BC1 = Band X (4 MIMO layer) + Band Y (2 MIMO layer), however because of temporary capability restriction, the UE can only work at [Band X (2 MIMO layer) + Band Y (2 MIMO layer)].

Proposal 1: The UE can report the BCs that are affected by the collision, and these affected BCs includes 2 types:

- Type 1: The BCs that are forbidden because of the collision;

- Type 2: The BCs that are supported but with lower capabilities (e.g. with lower MIMO layer and/or lower bandwidth) because of the collision.

For these affected BCs, the UE can indicate it by the Index, the Index indicates the position of the BC that reported in the supportedBandCombinationList of the UE capability message.

Proposal 2: The UE can indicate the affected BCs by Index, the Index indicates the position of the BC that reported in the supportedBandCombinationList of the UE capability message.

For the Type 2 affected BCs, besides the BC index, the UE also need to report the additional affected capabilities (e.g. MIMO layer and/or bandwidth).

Proposal 3: For the Type 2 affected BCs, besides the BC index, the UE also needs to report the additional affected capabilities (e.g. MIMO layer and/or bandwidth, FFS on the Granularity).

The other detail issue about the supported BC is about whether it’s for CA or DC or Both.

Supported band-combinations (FFS whether this is CA or DC or both)

For this issue, considering that the network A can work at the NR-DC, and from the UE capability structure aspect, there is no difference between the CA BC and DC BC, the supported band-combinations can be both. Furthermore, in the current Capability structure, the single CC capabilities is also reported by the BC structure, thus the single CC case shall not be excluded.

– BandCombinationList The IE BandCombinationList contains a list of NR CA, NR non-CA and/or MR-DC band combinations (also including DL only or UL only band).

For example, the UE may support MIMO layer = 8 for a single CC, then because of the MUSIM temporary capability restriction, the UE can only support MIMO layer = 4, the UE can indicate this BC with lower MIMO layer restriction.

Proposal 4: The affected band-combinations can be CA, DC and/or single CC.

MIMO layer and Bandwidth Restriction

Before going to the detail discussion on the MIMO layer and Bandwidth Restriction, it’s better to clarify the relationship between the BC restriction and the MIMO layer/Bandwidth Restriction first. As discussed above, for the forbidden BC, there is no need to report the additional capabilities. The MIMO layer and/or the supported bandwidth restriction shall only work for the Type 2 BCs.

Proposal 5: The MIMO layer and/or the supported bandwidth restriction shall only work for the Type 2 BCs.

Normally, the baseband capabilities are mainly determined by the MIMO layer and the bandwidth and from some aspect, there are tradeoff between these 2 items, thus the MIMO layer and the supported bandwidth shall be given the same granularity.

Proposal 6: The MIMO layer and the supported bandwidth shall be reported with the same granularity.

Furthermore, the MIMO layer and/or the supported bandwidth can be report for the UL and DL respectively, thus for the DL and UL, the same granularity shall be adopted.

Proposal 7: The same granularity shall be adopted for the DL and UL on the MIMO layer and the supported bandwidth reporting.

For the granularity, there are several options as below:

- Option 1: per FR

- Option 2: Per band

- Option 3: Per BC

- Option 4: Per FSPC (per cc Per BC)

In the current UE capability structure as in the Fig 1, the MIMO layer and the supported bandwidth are reported per FSPC, thus it’s recommended to adopt the same granularity.

If report per FR, it means that the UE has to report the supported minimum MIMO layer among the Bands in the corresponding frequency range, obviously, it would affect the network A’s performance significantly.

If report per Band, it means that whatever the UE is working at the single CC or CA/DC on this band with network A on this band, the same MIMO layer would be adopted and thus the UE has to report the supported minimum MIMO layer according to the most complicated case (e.g. CA/DC) especially when the UE take the proactive approach into consideration.

The only concern for per FSPC reporting is about the signaling overhead, to solve this concern, it can also be reported per BC.

Proposal 8: Adopt per FSPC (or per BC) granularity for the MIMO layer and Bandwidth reporting.

  - Network Configuration

About the network configuration, an FFS issue as below was left:

Editor note: FFS whether one configuration to control all temporary capabilities update or introduce individual control for each temporary capability update.

As discussed above, the temporary capability restriction mainly includes the affected BCs, MIMO layers and bandwidth, and these 3 items are dependent, thus it’s recommended to adopt one configuration to control all temporary capabilities update.

Proposal 9: Apply one configuration to control all temporary capabilities update.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: The UE can report the BCs that are affected by the collision, and these affected BCs includes 2 types:

- Type 1: The BCs that are forbidden because of the collision;

- Type 2: The BCs that are supported but with lower capabilities (e.g. with lower MIMO layer and/or lower bandwidth) because of the collision.

Proposal 2: The UE can indicate the affected BCs by Index, the Index indicates the position of the BC that reported in the supportedBandCombinationList of the UE capability message.

Proposal 3: For the Type 2 affected BCs, besides the BC index, the UE also needs to report the additional affected capabilities (e.g. MIMO layer and/or bandwidth, FFS on the Granularity).

Proposal 4: The affected band-combinations can be CA, DC and/or single CC.

Proposal 5: The MIMO layer and/or the supported bandwidth restriction shall only work for the Type 2 BCs.

Proposal 6: The MIMO layer and the supported bandwidth shall be reported with the same granularity.

Proposal 7: The same granularity shall be adopted for the DL and UL on the MIMO layer and the supported bandwidth reporting.

Proposal 8: Adopt per FSPC (or per BC) granularity for the MIMO layer and Bandwidth reporting.

Proposal 9: Apply one configuration to control all temporary capabilities update.