3GPP TSG-RAN WG2 Meeting #123	R2-2307542

Toulouse 21th -25th Aug, 2023

Source: 			ZTE Corporation, Sanechips

Title: 	Consideration on the R17/18 MUSIM Feature Interaction

Agenda item:	7.17.4

Document for: 	Discussion and Decision

- Introduction

In the Rel-17, two per UE level capabilities were reported for the MUSIM.

musim-GapPreference-r17 Indicates whether the UE supports providing MUSIM assistance information with MUSIM gap preference and related MUSIM gap configuration, as defined in TS 38.331 [9]. UE supporting this feature supports 3 periodic gaps and 1 aperiodic gap.

musimLeaveConnected-r17 Indicates whether the UE supports providing MUSIM assistance information with indication of leaving RRC_CONNECTED state as defined in TS 38.331 [9].

In the R18, the UE can support to report the temporary capability restriction, in this paper we share our preliminary thinking on the interaction of R17 MUSIM feature and R18 Temporary capability restriction feature.

- Discussion

In this chapter, we first discuss whether the UE is allowed to support R18 Temporary capability restriction feature only, then analyze whether the network can configure the R17/R18 feature simultaneously, at last we share our views on whether the UE is allowed to report the scheduling Gap and the R18 temporary capability restriction in the UAI at the same time.

Whether the UE is allowed to support R18 Temporary capability restriction feature only

In the release 17, two MUSIM features are supported for the short and long leaving respectively. For the short leave, the UE would request the scheduling Gap while for the long leave the UE would request the UE to release the connection.

When the R18 temporary capability restriction was supported, for both the long and short leaving, the UE can split the RF capabilities to different networks by UAI with temporary capability restriction. However, if the UE need to disconnect with NWA while the network B has higher service requirement, the UE need to support musimLeaveConnected-r17 feature. Another use case is that if the network doesn’t response the UE’s temporary capability request, the UE can request the network A to release the connection immediately.

Proposal 1: For the UE that support R18 Temporary capability restriction feature, the UE shall also support musimLeaveConnected-r17.

Whether the network can configure the R17/R18 feature simultaneously

For the R17 MUSIM, the network can configure musim-GapAssistanceConfig and musim-LeaveAssistanceConfig separately, as described above, for the case that the network doesn’t response the UE’s temporary capability restriction request, the UE can request the network A to release the connection immediately. Thus, the network can configure the R17 musim-LeaveAssistanceConfig and R18 temporary capability restriction reporting simultaneously.

For the musim-GapAssistanceConfig, in the R17, it’s mainly for the idle/inactive state behavior, e.g. paging/system information receiving and measurements. Though the UE can also split part of RF resources to do these operations and thus indicate R18 temporary capability restriction, it still can be left to UE to determine which solution is more proper. Furthermore, the UE can request R17 scheduling Gap for Idle/Inactive state behavior while indicate R18 temporary capability restriction for the connected state behavior.

Proposal 2: The network can configure the R17 (musim-GapAssistanceConfig and/or musim-LeaveAssistanceConfig) and R18 temporary capability restriction feature simultaneously

Whether the UE is allowed to report the R17 scheduling Gap and the R18 temporary capability restriction simultaneously

This issue depends on whether there are some practical use cases. Normally, the UE would request the scheduling gap when the UE is at the Idle/Inactive state and at most 3 periodic Gaps can be requested (e.g. one for SS, one for paging and one for measurement). The UE can execute SS and paging detection by scheduling Gap while do some neighbor cell measurement with split RF resources. For this case the UE may need to report R17 scheduling Gap and the R18 temporary capability restriction simultaneously.

Proposal 3: The UE is allowed to report the R17 scheduling Gap and the R18 temporary capability restriction simultaneously.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Proposal 1: For the UE that support R18 Temporary capability restriction feature, the UE shall also support musimLeaveConnected-r17.

Proposal 2: The network can configure the R17 (musim-GapAssistanceConfig and/or musim-LeaveAssistanceConfig) and R18 temporary capability restriction feature simultaneously.

Proposal 3: The UE is allowed to request the R17 scheduling Gap and the R18 temporary capability restriction simultaneously.