3GPP TSG-RAN WG2 Meeting #123	R2-2307544

Toulouse 21th -25th Aug, 2023

Source: 			ZTE Corporation, Sanechips

Title:           	Further Consideration on the IDC reporting

Agenda item:	7.10

Document for: 	Discussion and Decision

- Introduction

In the last meeting, some agreements have been achieved on the MN-SN coordination, the SN would indicate the candidate frequency ranges to the MN, and the MN would signal the affected carrier range combination to the SN

candidateServingFreqRangeListNR Indicates the candidate frequency ranges configured by SN for IDC. This field is only used in NR-DC.

affectedCarrierFreqRangeCombList This field is signalled upon MN not addressing IDC issue and contains the list of NR carrier frequency range combinations reported by UE to MN for IDC problem caused by the NR-DC frequency combination.

However, on how to report MR-DC IMD interference, companies have divergent understandings. In this paper, we share our views on it and a corresponding TP was also provided.

- Discussion

In this chapter, we first describe different understandings on how to Config/Report MR-DC IDC interference according to the latest running CRs, and then analyze the problem of each understanding one by one, at last we give our modification suggestions.

  - Different Understandings on the MR-DC IMD Config/Report

According to the latest running CR in [1], the MR-DC IDC reporting is described as below:

#### 5.7.4.2 Initiation 1> if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig of a cell group: 2> if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information: 3> if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or 3> if on one or more supported UL CA or MR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message 2> if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems; 3> for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth; 3> for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

In this description, it does not mention the cell group of the candidateServingFreqRangeListNR and thus there would be 3 understandings:

- Understanding 1: The MN and SN configure the candidateServingFreqRangeListNR separately and the UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR.

- Understanding 1a: The UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR by only reporting the frequency range(s) that are included in the candidateServingFreqRangeListNR of the corresponding Cell Group.

- Understanding 1b: The UE report the IMD interference that cross the MCG configured candidateServingFreqRangeListNR and SCG configured candidateServingFreqRangeListNR by reporting the frequency range(s) that are included in the MCG candidateServingFreqRangeListNR and/or the SCG candidateServingFreqRangeListNR

- Understanding 2: The SN transfers the candidateServingFreqRangeListNR at SN side to the MN and the MN includes both the MN candidate frequency ranges and the SN candidate frequency ranges in the MN candidateServingFreqRangeListNR. The UE report the IMD interference to the MN.

To better understand these understandings, we take the below Fig 1 as an example: In this example, both the MN and SN have one candidate frequency range and noted as f1/f2 respectively.

With the understanding 1, the MN configure the Frequency range 1 (f1) while the SN configure the frequency range 2 (f2) to the UE, then the UE detects that there is MR-DC IMD interference with frequency range 1 and frequency range 2 combination. If the UE determine to report this interference to MCG

- For the understanding 1a, the UE would only report the frequency range 1.

- For the understanding 1b, the UE would report combination of frequency range 1 and frequency range 2.

Fig 1: IMD Interference Reporting

With the understanding 2, if the network require the UE to report the MR-DC IMD interference, the SN shall indicate the candidate frequency range list to the MN, then the MN configure both the f1 and f2 to the UE.

Observation 1: According to the current spec, there are 2 different understandings on how to Config/Report MR-DC reporting.

- Understanding 1: The MN and SN configure the candidateServingFreqRangeListNR separately and the UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR.

- Understanding 1a: The UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR by only reporting the frequency range(s) that are included in the candidateServingFreqRangeListNR of the corresponding Cell Group.

- Understanding 1b: The UE report the IMD interference that cross the MCG configured candidateServingFreqRangeListNR and SCG configured candidateServingFreqRangeListNR by reporting the frequency range(s) that are included in the MCG candidateServingFreqRangeListNR and/or the SCG candidateServingFreqRangeListNR

- Understanding 2: The SN transfers the candidateServingFreqRangeListNR at SN side to the MN and the MN includes both the MN candidate frequency ranges and the SN candidate frequency ranges in the MN candidateServingFreqRangeListNR. The UE report the IMD interference to the MN.

  - Problems of Each Understanding

Problem of the understanding 1a

For the understanding 1a, the main issue is that the MN (or SN) will misunderstanding the IDC interference type for that it only reports part frequency range. Then RAN2 need to discuss whether the network need to know the complete frequency range combination information for the IMD interference. As shown in the Fig 2 (according to the Table 1), the interference caused by combination of (fx_low, fx_high) and (fy_low, fy_high) are determined with the formula (|fy_low-fx_high|, |fy_high-fx_low|) for the lower range and (|fy_low+fx_low|, |fy_high+fx_high|) for the higher range.

Table 1: (2nd order) IMD frequency limits

| Band | n3 |  | n79 |  |
| --- | --- | --- | --- | --- |
| UL frequency | fx_low: 1710 | fx_high:1785 | fy_low:4400 | fy_high:5000 |
| 2nd order IMD products | ¦fy_low-fx_high¦ | ¦fy_high-fx_low¦ | ¦fy_low+fx_low¦ | ¦fy_high+fx_high¦ |
| IMD frequency limits (MHz) | 2615 | 3290 | 6110 | 6785 |

Fig 2:2nd order IMD frequency Range

Take the n3+ n79 as an example, if the UE reports the frequency range combination of n3 (1710~1785) and n79 (4400~5000), the network would determine the IMD interference location with the above formula and get two frequency ranges: (2615~3290) and (6110~6785). If the UE only indicates part of the frequency range that may cause IMD interference, the network can’t know the exact IMD interference location.

Observation 2: If the UE only indicates part of the frequency range that cause IMD interference, the network can’t know the exact IMD interference location and may misunderstand the interference type.

Proposal 1: Ran2 to confirm if the IMD interference is caused by the frequency ranges combinations, the UE shall report all of the frequency ranges.

If proposal 1 is confirmed, the understanding 1a can be precluded.

Proposal 1a: If the proposal 1 is confirmed, no spec change is needed and the understanding 1a shall be precluded.

Problem of the understanding 1b

For the understanding 1b, the main issue is that the MN (or SN) would receive the IDC report that involved a frequency range outside of its configured candidateServingFreqRangeListNR. Thus RAN2 need to confirm whether the UE is allowed to report the frequency range out of the candidateServingFreqRangeListNR of the corresponding cell group. In the previous meeting, it has been agreed that the frequency range reported by the UE shall at least overlap with the frequency range that configured by the network. But it says “configured by the network”, it doesn’t mention the cell group, thus some further clarification would be needed.

- The frequency range (centre frequency + bandwidth) reported by the UE shall at least overlap with the frequency range (centre frequency + bandwidth) configured by the network. - The centre frequency reported by the UE is within the frequency range (centre frequency + bandwidth indicated by network in the configuration) configured by the network.

Proposal 2: Ran2 to confirm that the frequency range reported by the UE shall at least overlap with the frequency range that included in the candidateServingFreqRangeListNR of the corresponding cell group.

If the proposal 2 is confirmed, the “candidateServingFreqRangeListNR” shall be further clarified as “ candidateServingFreqRangeListNR of the cell group” and thus the understanding 1b shall be precluded.

Proposal 2a: If the proposal 2 is confirmed, add cell group restriction to the “candidateServingFreqRangeListNR ” (i.e. candidateServingFreqRangeListNR of the cell group) and thus the understanding 1b shall be precluded.

Problem of the understanding 2

For the understanding 2, the MN would include both the MN candidate frequency ranges and the SN candidate frequency ranges in the MCG candidateServingFreqRangeListNR. However, the problem is that when the UE get the candidateServingFreqRangeListNR from the MCG, it doesn’t know whether it includes candidate frequency ranges from both nodes, thus the UE can’t distinguish the MR-DC IMD interference from the UL CA IMD interference. So if going to the understanding 2, the “UL CA or MR-DC combination” shall be changed to “UL CA or MR-DC combination” for that the UE can’t distinguish the MR-DC IMD interference from the UL CA IMD interference.

Proposal 3: If understanding 2 was confirmed to be the right one, change “UL CA or MR-DC combination” to “UL CA or MR-DC combination” for that the UE can’t distinguish the MR-DC IMD interference from the UL CA IMD interference.

Based on the above proposals and the observations, the spec can be simply changed as below:

#### 5.7.4.2 Initiation 1> if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig of a cell group: 2> if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information: 3> if on one or more frequency ranges included in candidateServingFreqRangeListNR, the UE is experiencing IDC problems that it cannot solve by itself; or 3> if on one or more supported UL CA or MR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself: 4> initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information; #### 5.7.4.3 Actions related to transmission of UEAssistanceInformation message 2> if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR of the cell group, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR of the cell group, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself: 3> include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems; 3> for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth; 3> for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

A complete TP based on the latest running CR was also provided as in Annex.

Proposal 4: Agree the TP as in Annex.

- Conclusion and proposals

With the above analysis, we have the following proposals:

Observation 1: According to the current spec, there are 2 different understandings on how to Config/Report MR-DC reporting.

- Understanding 1: The MN and SN configure the candidateServingFreqRangeListNR separately and the UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR.

- Understanding 1a: The UE report the IMD interference that cross the MCG candidateServingFreqRangeListNR and SCG candidateServingFreqRangeListNR by only reporting the frequency range(s) that are included in the candidateServingFreqRangeListNR of the corresponding Cell Group.

- Understanding 1b: The UE report the IMD interference that cross the MCG configured candidateServingFreqRangeListNR and SCG configured candidateServingFreqRangeListNR by reporting the frequency range(s) that are included in the MCG candidateServingFreqRangeListNR and/or the SCG candidateServingFreqRangeListNR

- Understanding 2: The SN transfers the candidateServingFreqRangeListNR at SN side to the MN and the MN includes both the MN candidate frequency ranges and the SN candidate frequency ranges in the MN candidateServingFreqRangeListNR. The UE report the IMD interference to the MN.

Observation 2: If the UE only indicates part of the frequency range that cause IMD interference, the network can’t know the exact IMD interference location and may misunderstand the interference type.

Proposal 1: Ran2 to confirm if the IMD interference is caused by the frequency ranges combinations, the UE shall report all of the frequency ranges.

Proposal 1a: If the proposal 1 is confirmed, no spec change is needed and the understanding 1a shall be precluded.

Proposal 2: Ran2 to confirm that the frequency range reported by the UE shall at least overlap with the frequency range that included in the candidateServingFreqRangeListNR of the corresponding cell group.

Proposal 2a: If the proposal 2 is confirmed, add cell group restriction to the “candidateServingFreqRangeListNR ” (i.e. candidateServingFreqRangeListNR of the cell group) and thus the understanding 1b shall be precluded.

Proposal 3: If understanding 2 was confirmed to be the right one, change “UL CA or MR-DC combination” to “UL CA or MR-DC combination” for that the UE can’t distinguish the MR-DC IMD interference from the UL CA IMD interference.

Proposal 4: Agree the TP as in Annex.

- Reference

- R2-2306925 38.331 running CR for introduction of IDC Xiaomi

- R2-2306595 Discussion on inter-node coordination solution	Huawei, HiSilicon	discussion

- Annex- TP

#### 5.7.4.2	Initiation

/****************************omit the unchanged part*************************************/

1>	if configured to provide IDC assistance information based on idc-FDM-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-FDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequency ranges included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or MR-DC combination comprising of frequency ranges included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-FDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

1>	if configured to provide IDC assistance information based on idc-TDM-AssistanceConfig of a cell group:

2>	if the UE did not transmit a UEAssistanceInformation message with idc-TDM-Assistance since it was configured to provide IDC assistance information:

3>	if on one or more frequencies or frequency ranges, the UE is experiencing IDC problems that it cannot solve by itself; or

3>	if on one or more supported UL CA or MR-DC combination comprising of carrier frequencies or frequency ranges, the UE is experiencing IDC problems that it cannot solve by itself:

4>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

2>	else if the current idc-TDM-Assistance information for the cell group is different from the one indicated in the last transmission of the UEAssistanceInformation message:

3>	initiate transmission of the UEAssistanceInformation message in accordance with 5.7.4.3 to provide IDC assistance information;

/****************************omit the unchanged part*************************************/

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

/****************************omit the unchanged part*************************************/

2>	if there is at least one affected frequency range overlapping with one frequency range included in candidateServingFreqRangeListNR of the cell group , and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeList with an entry for each affected frequency range;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include centerFreq and affectedBandwidth;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one supported UL CA or MR-DC combinations comprising of frequency ranges included in candidateServingFreqRangeListNR of the cell group, and each affected frequency range in the UL CA or MR-DC combination overlapping with one frequency range included in candidateServingFreqRangeListNR of the cell group, and the center frequency of the affected frequency range is within the frequency range included in candidateServingFreqRangeListNR of the cell group, the UE is experiencing IDC problems that it cannot solve by itself:

3>	include the field affectedCarrierFreqRangeCombList with an entry for each supported UL CA or MR-DC combination comprising of frequency ranges that is affected by IDC problems;

3>	for each affected frequency range included in the field affectedCarrierFreqRangeCombList, include centerFreq and affectedBandwidth;

3>	for each UL CA or MR-DC combination included in the field affectedCarrierFreqRangeCombList, include interferenceDirection and optionally victimSystemType, and set it accordingly;

2>	if there is at least one carrier frequency or frequency range or one supported UL CA or MR-DC combination comprising of carrier frequencies or frequency ranges, the UE is experiencing IDC problems that it cannot solve by itself, and affectedCarrierFreqList or affectedCarrierFreqCombList or affectedCarrierFreqRangeList or affectedCarrierFreqRangeCombList is included, and idc-TDM-AssistanceConfig is set to setup:

3>	include Time Domain Multiplexing (TDM) based assistance information as indicated by idc-TDM-Assistance that could be used to resolve the IDC problems;

/****************************omit the unchanged part*************************************/