3GPP TSG-RAN WG2 Meeting #123		   R2-2307547

Toulouse, France, August 21-25, 2023

Agenda item:	7.9.2

Source:	vivo

Title:	Remaining issues on U2U discovery and relay (re)selection

Document for:		Discussion & Decision

1. Introduction

In this contribution, we will further discuss the following issues related to U2U discovery and relay (re)selection:

- Resource pool for integrated discovery

- How to determine when to switch to direct link

- Simultaneous relay UE reselection triggering at both remote UEs

- Other miscellaneous issues, e.g. whether to use dedicated signalling for the discovery configuration, additional U2U relay (re)selection triggering

2. Discussion

2.1 Resource pool for integrated discovery

In RAN2 #120 meeting, it was agreed that:

Proposal 6 (modified): RAN2 to confirm that SL-SRB0 is reused for DCR message if discovery is integrated into PC5 unicast link establishment procedure.

While in RAN2 #119bis, it was also agreed:

Proposal 4.1:		Both shared and dedicated resource pool can be used for U2U discovery transmission and Rel-17 pool selection principle is re-used.

To us, it is not clear whether the DCR message with integrated discovery should use the dedicated pool or not.

- On one hand, as SL-SRB0 are not carrying discovery messages, according to current pool selection procedure, it should use the shared pool for transmission;

- On the other hand, the DCR message with integrated discovery serve the purpose to ‘discover’ other UEs, so it seems one UE has to monitor it anyway.

Back in Rel-17 when dedicated pool for U2N was introduced, one important argument is that we should consider the UEs which are interested in discovery only and the dedicated pool can achieve some power saving gain[1]. In U2U relay, if we make the DCR message with integrated discovery to be only transmitted in shared pool, then the UE may always monitor the shared pool even if it is only interested in discovery. Therefore, it seems the dedicated pool would not be useful anymore.

Observation 1: DCR message with integrated discovery will use the shared pool for transmission as we agreed to use SL-SRB0 for it.

Observation 2: A UE should always monitor the shared pool even it is only interested in discovery, trying not to miss the DCR message with integrated discovery, which makes the power saving gain brought by dedicated discovery pool not exist anymore.

To solve this concern, there may be different solutions. E.g.

- Either we revert our agreement to use SL-SRB4 for DCR message with integrated discovery, so it can use the dedicated discovery pool, or,

- We explicitly clarify that when the message is DCR message with integrated discovery it can use the dedicated discovery pool as other discovery messages.

We propose to discuss whether we are going to solve this issue and also the different options.

Proposal 1: RAN2 to discuss whether we should support DCR message with integrated discovery to use the dedicated discovery pool.

2.2 Switch to direct link in relay reselection

Another issue we think needs to be resolved is whether/how to switch back to direct link in U2U relay reselection. This issue has been touched in RAN2 #122 offline summary and a proposal was given (although not discussed due to lack of time):

[ToDis] Proposal 11: RAN2 to discuss whether AS criterion is needed for switching back from indirect to direct link

Some of the companies thought we should e.g. prioritize the direct link over the indirect link, or leave it to UE implementation if both direct and indirect link is available. However, as discussed in our previous contribution, the issue is actually similar to the discussion in Rel-17 when both cell (re)selection and relay (re)selection can happen at the same time for U2N relay.

TS 38.331 NOTE 3: For L2 U2N Remote UEs in RRC_IDLE/INACTIVE and L3 U2N Remote UEs, the cell (re)selection procedure and relay (re)selection procedure run independently. If both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE. Furthermore, L3 U2N Remote UE's selection on both cell and NR sidelink U2N Relay UE is also based on UE implementation.

Observation 3: In Rel-17 U2N relay, if both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE.

However, for U2U relay, it is not clear how the UE can switch between indirect link and direct link, as the L2 IDs for them are different based on SA2 LS:

R2-2301933 Question3b: Whether the Layer2 ID of the remote UE can be the same for U2U Relay service and Non-relay service? Answer 3b: No. The Source Layer-2 IDs of 5G ProSe End UE used for 5G ProSe UE-to-UE Relay Discovery and Communication would be different from the Source Layer-2 IDs used for Non-relay discovery and Communication.

Observation 4: The L2 ID pairs between remote UE1 and remote UE2 are different when they are communicating using direct link and using indirect link (i.e. via a U2U relay UE).

Therefore, it is not clear if it is even possible that the UE would know the direct link and indirect link are towards the same UE. The following is an example:

In this figure, it is not clear whether the remote UE1 would know the L2 ID=N and the L2 ID=C, are belonging to a same UE. So when the reselection happens, there are two interpretations about how to understand the impact of different L2 ID and about whether change from direct link to indirect link or vice visa, is possible:

Interpretation 1: When relay reselection happens, it is possible that remote UE1 can receive the message of L2 ID=C and find the measured RSRP is quite high (which means the direct link between remote UE1 and UE2 is good). Then, it may choose to communicate with remote UE2 by using a direct link.

Interpretation 2: When relay reselection happens, from AS layer’s perspective, remote UE1 and remote UE2 can only reselect to a new relay UE or no relay UE (which means the communication would be terminated), but not switch to the direct link, because the L2 ID pairs are different for direct link and indirect link. The remote UE1 would never know how good is the direct link between remote UE1 and remote UE2, because the direct link between them are not using L2 ID A and L2 ID C (but using L2 ID M and L2 ID N) in the above example.

Based on different interpretations, the relay reselection procedure and the switch between direct link and indirect link can have different modelling. We propose to clarify which is the correct understanding.

Proposal 2: RAN2 to discuss which interpretation is the correct understanding and send a LS to SA2 to confirm:

- Interpretation 1: UE can compare or select from direct link and indirect link based on two PC5-RSRPs even the two links are using different L2 ID pair;

- Interpretation 2: UE cannot compare or select from direct link and indirect link based on two PC5-RSRPs because the two links are using different L2 ID pair.

If reselection towards direct link is supported during relay reselection, then RAN2 can further discuss whether AS criterion is needed for switching back from indirect to direct link. Our understanding is that both indirect link quality and direct link quality should be considered. E.g. when the RSRP on indirect link is below a configured threshold and when the RSRP on the direct link is above a configured threshold, the UE may switch from the indirect to direct link.

Proposal 3: If it is confirmed that reselection towards direct link is supported during relay reselection, the following AS criterion for that can be discussed:

- When the PC5 RSRP on indirect link is below a configured threshold and when the PC5 RSRP on the direct link is above a configured threshold, the UE may switch from the indirect to direct link.

2.3 Simultaneous relay UE reselection triggering at both remote UEs

Another issue is how to deal with the case that simultaneous relay UE reselection can be triggered at both remote UEs at the same time. This issue has been touched in RAN2 #122 offline summary and a proposal was given (although not discussed due to lack of time):

[ToDis] Proposal 13: RAN2 to discuss whether/how to handle the case that remote UE and its peer remote UE may select two different relay UEs simultaneously for communicating with each other. Send LS to SA2 if necessary.

Briefly speaking, for a pair of remote UEs (e.g. remote UE1 and remote UE2), each UE may trigger relay reselection based on current hop or on the second hop based on RLF indication. Then it is possible that both UEs trigger the relay reselection at the same time.

It is not clear if both remote UE1 and UE2 triggers relay reselection at the same time, how the suitable relay UE would be selected. As the relay selection and reselection procedure are mainly decided in SA2, and it is possible that each remote UE’s upper layer alone may trigger relay reselection when upper layer criteria are not satisfied, we propose to leave this simultaneously triggering case to the SA2 for further study.

Proposal 4: Leave the decision to SA2 of whether/how to handle the case that relay reselection is triggered simultaneously at both remote UE1 and remote UE2. Send a LS to SA2 to inform this if agreed.

2.4 Other miscellaneous issues for discovery and relay (re)selection

There are some left [ToDis]-Proposals in [3] and we think some of them can be further progressed. These proposals are summarized as follows with our proposal:

| Left proposal in RAN2 #122 U2U summary which we think can be further progressed | Our analyses and proposals |
| --- | --- |
| [ToDis] Proposal 6: For RRC_CONNECTED U2U relay/remote UE, U2U relay, dedicated signalling is used for the discovery configuration. | There is already a majority view to support this proposal. And it is aligned with legacy design. So we suggest to agree this proposal. |
| [ToDis] Proposal 9: Remote UE can trigger U2U relay selection when PC5 RLF of the direct link between the remote UE and the peer remote UE is detected. | We already agreed about the PC5 RSRP on direct link to determine when U2U relay selection should be triggered. It is already enough. Secondly, the PC5 RLF will anyway be informed to upper layer so we can just rely on upper layer indication to trigger relay selection and no need for PC5 RLF to be an AS criterion. So we suggest not to agree this proposal. |
| [ToDis] Proposal 10: Remote UE can trigger relay reselection if the link quality of the second hop between the relay UE and peer remote UE is blow a threshold even the link quality of the first hop is good. FFS for the content of the link quality indication of the second hop. | We agreed that each Remote UE can trigger Relay reselection based at least on current hop quality. In case of UE1<->Relay<->UE2, it seems the second hop does not need to be considered by UE1 because the second hop quality is already considered by remote UE2 as its current hop quality. So we suggest not to agree this proposal. |

As the proposals are already given in last meeting summary document and they are not complexed, we do not give separate proposals for them again in this contribution.

3. Conclusion

In this contribution, we discussed about some left issues for common L2L3 parts for U2U relaying.

We have the following observations and proposals:

Observation 1: DCR message with integrated discovery will use the shared pool for transmission as we agreed to use SL-SRB0 for it.

Observation 2: A UE should always monitor the shared pool even it is only interested in discovery, trying not to miss the DCR message with integrated discovery, which makes the power saving gain brought by dedicated discovery pool not exist anymore.

Observation 3: In Rel-17 U2N relay, if both suitable cells and suitable NR sidelink U2N Relay UEs are available, it is up to NR sidelink U2N Remote UE implementation to select either a cell or a NR sidelink U2N Relay UE.

Observation 4: The L2 ID pairs between remote UE1 and remote UE2 are different when they are communicating using direct link and using indirect link (i.e. via a U2U relay UE).

Proposal 1: RAN2 to discuss whether we should support DCR message with integrated discovery to use the dedicated discovery pool.

Proposal 2: RAN2 to discuss which interpretation is the correct understanding and send a LS to SA2 to confirm:

- Interpretation 1: UE can compare or select from direct link and indirect link based on two PC5-RSRPs even the two links are using different L2 ID pair;

- Interpretation 2: UE cannot compare or select from direct link and indirect link based on two PC5-RSRPs because the two links are using different L2 ID pair.

Proposal 3: If it is confirmed that reselection towards direct link is supported during relay reselection, the following AS criterion for that can be discussed:

- When the PC5 RSRP on indirect link is below a configured threshold and when the PC5 RSRP on the direct link is above a configured threshold, the UE may switch from the indirect to direct link.

Proposal 4: Leave the decision to SA2 of whether/how to handle the case that relay reselection is triggered simultaneously at both remote UE1 and remote UE2. Send a LS to SA2 to inform this if agreed.

4. References

R2-2109430, Summary report of [Post115-e][611][Relay] Discovery shared/dedicated pool issue (Qualcomm), 3GPP TSG RAN WG2 Meeting #116-e

R2-2108835, RAN2 #115e chairman notes

R2-2306555, Summary of AI 7.9.2 on UE-to-UE relay