3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307551

Toulouse, France, 21st – 25th Aug, 2023

Source:              CATT

Title:	             Discussion on U2U Relay

Agenda Item:	7.9.2

Document for:	Discussion and Decision

Introduction

In this contribution, the below aspects are further discussed in detail:

- SRAP layer design;

- Control plane procedures;

- QoS split;

- Leftover issues of U2U relay discovery and (re)selection.

Discussion

SRAP layer design

In RAN2#122 meeting, regarding to the short ID, the following agreements were reached:

Agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

Regarding to the short ID design, the below two issues should be discussed:

- Issue 1: FFS if the short ID in U2U relay should be assigned hop-by-hop or globally.

For relay UE, it has more generalized information about the pair information, the relay UE can assign the single ID globally instead of hop-by-hop. Different to Rel-17 U2N relay, the gNB is not involved for U2U relay, RAN2 had agreement that the assignment work is in charge of relay UE. For the issue that when to assign the local ID for U2U relay, the U2U relay UE knows the L2 addresses of both L2 U2U Remote UEs during the PC5 link setup procedure in each hop between remote UEs and relay UEs, after that the relay can assign the short ID. Meanwhile, it is left to UE implementation to avoid collision on the usage of short ID. At last, after the relay UE assign the short ID, it can use the legacy PC5-RRC signalling to transfer the local ID to end remote UE.

Proposal 1: The short ID should be assigned unique within one U2U relay.

Figure 1: High-level signalling flow for L2 U2U relay short ID assignment

- Issue 2: Down selection of short ID design options

For option a), a single ID can identify the source and destination combination. While for option (b), two short IDs should be introduced, one for source L2 ID and another for destination L2 ID. Considering the pairs of end remote UEs connected via the U2U relay will not be very large, hence, for option a), the length of the short ID can be short. Hence, it is obvious that option a) can reduce the SRAP header overhead compared with Option b). From this perspective, option a) is slightly preferred.

Proposal 2: For U2U relay, suggest to use one single ID to identify the source and destination remote UEs.

Control plane procedures

    - End-to-end SL-SRBs configuration

In RAN2#121bis-e meeting, the below agreements for E2E PC5-RRC connection establishment were reached:

End-to-end PC5 RRC connection between source remote UE and target remote UE is supported, in addition to PC5-RRC connections between each remote UE and the relay UE.  This does not imply support of all PC5-RRC procedures between the remote UEs.

A one-to-one correspondence between end-to-end PC5 RRC connection and end-to-end PC5 unicast link is supported as legacy.

E2E PC5-RRC connection is considered to be established after a corresponding E2E PC5 unicast link is established.  FFS how configurations for e2e SL-SRBs are supported.

In RAN2#122 meeting, further progress was made on SL-SRB relevant configuration:

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

In order to solve the FFS part (the SRAP and PC5 RLC channel configuration for SL-SRB), further analysis is given as below: currently, the specified configuration for SL-SRB is utilized for direct communication in PC5. As for U2U relay, the existing mechanism can be reused. Besides, to enable the exchange of E2E PC5-S signaling (for DCR/DCA and SMC messages) following per-hop link establishment, the AS-layer settings for the corresponding SL-SRBs need to be prepared beforehand. It is a nature way-out to adopt the legacy approach by utilizing the specified configuration for SRAP and RLC.

- Regarding the SRAP configuration, since Remote UE E2E Radio Bearer ID should be included in the adaptation layer in first and second PC5 hop, meaning the specified configuration for SRAP can be employed for the SL SRBs. It is prefer to introduce one new SRAP configuration.

- Regarding the PC5 RLC channel configuration, considering the E2E RB-to-PC5 hop mapping should also be extended to two PC5 hops, some enhancement is also needed. In Rel-17 U2N relay, for the delivery of remote UE’s SRB0 RRC message, specified (fixed) configuration is used for the configuration of PC5 RLC channel. For the delivery of remote UE’s SRB1 RRC message such as RRCResume and RRCReestablishment message, default configuration is used for the configuration of PC5 RLC channel which can be reconfigured by network. Since RAN2 decided that explicit end-to-end PC5 RRC establishment is not needed, then in order to transfer the E2E PC5-S messages for PC5-S connection establishment and the first E2E PC5-RRC signalling, all the related E2E SL-SRBs(SL-SRB0,1,2,3) should be specified, then the following end-to-end DRBs can be further configured based on E2E SL-SRB3.In order to distinguish with the existed SL-RLC, new per-hop SL-RLCs (e.g. SL-RLC 4/5/6/7) should be introduced.

Proposal 3: For the E2E SL-SRB configuration of U2U relay, specified SRAP configuration is used.

Proposal 4: In order to support the specified configuration for E2E SL-SRBs, the mapping between the two PC5 RLC channels used for the E2E SL-SRBs should be specified.

Proposal 5: In order to support the specified configuration for E2E SL-SRBs, new per-hop SL-RLCs (e.g. SL-RLC4/5/6/7) should be introduced.

    - End-to-end SL-DRBs configuration

In RAN2#122 meeting, the agreements on E2E SL-DRB and PC5 RLC channel configuration were reached as below:

For OOC U2U relay/remote UE, pre-configuration is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For RRC_IDLE/INACTIVE U2U relay/remote UE, SIB is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

There is one leftover issue that for RRC_CONNECTED U2U relay/remote UE, whether dedicated signaling can be used for SL-DRB and PC5 RLC channel configuration. The current controversy point is that RAN2 had reached agreement that in RAN2#119bis-e meeting to simplify the gNB involvement in U2U relay as compared to the U2N case. From our point of view, for IC RRC_CONNECTED U2U relay/remote UE, it is reasonable to configure the SL-DRB and PC5 RLC channel. It brings more flexibility and reusability from gNB points of view, hence it is preferred to use dedicated signaling for the E2E SL-DRB and per-hop PC5 RLC channel configuration for U2U relay.

Proposal 6: For IC U2U relay/remote UE, dedicated signaling can be used for the E2E SL-DRB and per-hop PC5 RLC channel configuration for U2U relay.

QoS Split

For U2U relay QoS part, the below issue should be discussed:

- Which QoS parameter(s) should be split between the two hops?

Refer to Rel-17 U2N relay, the end-to-end PDB parameter was split between the two hops. PDB defines an upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF. With the introduction of U2U relay, the End-to-end link is split into two PC5 links, it is obvious the PDB should be split between the two PC5 links to ensure the overall PDB requirement.

Besides, PER defines an upper bound for the rate of PDUs (e.g. IP packets) that have been processed by the sender of a link layer protocol (e.g. RLC in RAN of a 3GPP access) but that are not successfully delivered by the corresponding receiver to the upper layer (e.g. PDCP in RAN of a 3GPP access). In order to meet the end-to-end PER requirement, the target PER of at least one of the PC5 should be lower than the end-to-end PER. Although in SA2’s TR23.752, it is recorded in the Solution#31 that PER is also needed to be split for U2U relay case. But in the final version of SA2’s TS23.304, PER is not captured for U2U relay. That is to say, the PER is not adopt. Hence, we propose that:

Proposal 7: The end-to-end PDB parameter needs to be split between two PC5 links for U2U relay.

Leftover issues of U2U relay discovery and (re)selection

In this section, we focus on some leftover issues of U2U relay discovery and (re)selection.

    - Discovery configuration for Connected UE

In RAN2#119bis-e meeting, the below agreement was reached for discovery configuration:

In UE-to-UE relay, the remote/relay UE in RRC_IDLE/RRC_INACTIVE or OOC can acquire discovery configuration as in Rel17 (i.e., cell-specific configuration/preconfiguration).  FFS if any restrictions specific to UE-to-UE relay are introduced for in-coverage UE in RRC_CONNECTED.

Based on the above agreement, there is still one leftover issue on how to configure the discovery message for the RRC_CONNECTED UE.

In Rel-17 U2N relay, the below agreements were reached in RAN2#116-e meeting,

Relay UE and remote UE (IC) in RRC CONNECTED can use the discovery configuration provided via dedicated signalling if available.

Relay UE and remote UE (IC) in RRC IDLE or RRC INACTIVE shall use the discovery configuration provided via SIB if available.

L2 relay UE will always use the discovery configuration provided by gNB (either via SIB or dedicated signalling).

It is shown that for the relay UE and remote UE (IC) in RRC CONNECTED can use the discovery configuration provided via dedicated signalling if available for Rel-17 U2N relay. For Rel-18 U2U relay, it is reasonable to follow the similar principle as in Rel-17. On the other hand, if there is any restrictions specific to U2U relay are introduced for IC UE in RRC CONNECTED, more specification effort should be introduced.

Proposal 8: For RRC_CONNECTED U2U relay/remote UE, discovery configuration can be configured by dedicated signalling.

    - Co-existence issue

For the issue that whether a UE be involved in acting as both U2N (as U2N Relay or Remote UE) and U2U (as U2U Relay or Remote UE) at the same time? The SA2 replied a positive answer [2] that “A UE can take any role as above based on its 5G ProSe Capability and configured/provisioned 5G ProSe policy/parameters”. That is to say, the co-existence between U2N relays and U2U relays can be discussed. But considering there is still a lot of specification effort on the control plane design for U2U relay, it is suggested to deprioritize it in the current release.

Proposal 9: RAN2 deprioritize the discussion on U2N relay and U2U relay co-existence.

    - Remote UE behavior when there are multiple suitable candidates U2U relay UEs which meet both the AS-layer and high layer criteria

In RAN2#121 meeting, RAN2 reached the following agreement:

For relay UE selection, the remote UE uses SL-RSRP measurements towards peer remote UE to trigger relay UE selection when there is data transmission on direct link.

For relay UE reselection, the remote UE uses SL-RSRP measurements towards the relay UE to trigger relay UE reselection when there is data transmission on the indirect link.

In both cases, it is left to remote UE implementation whether to use SL-RSRP or SD-RSRP for relay (re)selection trigger evaluation in case of no data transmission.

Further, in case there is more than one candidate U2U relay UEs are detected, which one is chosen can be left to remote UE implementation following the same principle in Rel-17 U2N relay.

Proposal 10: If multiple suitable candidates U2U relay UEs which meet both the AS-layer and higher layer criteria are available, it is up to remote UE implementation to choose which U2U relay UE is used.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: The short ID should be assigned unique within one U2U relay.

Proposal 2: For U2U relay, suggest to use one single ID to identify the source and destination remote UEs.

Proposal 3: For the E2E SL-SRB configuration of U2U relay, specified SRAP configuration is used.

Proposal 4: In order to support the specified configuration for E2E SL-SRBs, the mapping between the two PC5 RLC channels used for the E2E SL-SRBs should be specified.

Proposal 5: In order to support the specified configuration for E2E SL-SRBs, new per-hop SL-RLCs (e.g. SL-RLC4/5/6/7) should be introduced.

Proposal 6: For IC U2U relay/remote UE, dedicated signaling can be used for the E2E SL-DRB and per-hop PC5 RLC channel configuration for U2U relay.

Proposal 7: The end-to-end PDB parameter needs to be split between two PC5 links for U2U relay.

Proposal 8: For RRC_CONNECTED U2U relay/remote UE, discovery configuration can be configured by dedicated signalling.

Proposal 9: RAN2 deprioritize the discussion on U2N relay and U2U relay co-existence.

Proposal 10: If multiple suitable candidates U2U relay UEs which meet both the AS-layer and higher layer criteria are available, it is up to remote UE implementation to choose which U2U relay UE is used.

Reference

- TR 38.836, Study on NR sidelink relay

- R2-2301933 Reply LS on Differentiation of Layer2 ID and Coexistence of U2N/U2U

- R1-2304211  Reply LS on comparison of SL-RSRP and SD-RSRP measurements