3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307553

Toulouse, France, 21st – 25th Aug 2023

Source:	CATT

Title:	Discussion on Multi-path

Agenda Item:	7.9.4‎

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, some basic agreements on multi-path were made. In this contribution, we further discuss the leftover issues of multi-path for scenario 1/2 and give our preference.

Discussion

  - Cell Group for MP for scenario 1/2

In RAN2#122 meeting, inter-DU case was discussed for MP relay and the working assumption was made.

WA: For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

RAN2 had agreed that the PCell only on the direct path when a remote UE is configured with multi-paths. Thus, the direct path should be always configured in MCG of the remote UE when the remote UE is configured with multi-path.

Proposal 1: RAN2 to confirm the WA:

For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

  - Details of Multi-path

Failure report for scenario 1/2

In RAN2#121bis-e meeting, it was agreed that remote UE should inform Uu RLF of relay UE to gNB upon reception of Uu RLF indication from the relay UE when SRB1 is available on the direct path and not suspended:

Agreements: WA: For a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/RRC_INACTIVE. A remote UE in RRC_CONNECTED, upon reception of Uu RLF indication from the relay UE, suspends transmissions on the indirect path and informs the network if SRB1 is available on the direct path and not suspended, otherwise triggers re-establishment. FFS whether to apply the same behaviour 1) when the relay UE informs the remote UE of HO; 2) When the relay UE moves to IDLE following expiry of dataInactivityTimer, if the timer is supported for the relay UE. This agreement does not imply any conclusion on non-split SRB1 on indirect path. A remote UE in multipath that is released to RRC_IDLE/RRC_INACTIVE can apply legacy cell/relay selection behaviour, thus moving to single-path operation on either path according to implementation.

In RAN2#122 meeting, the above WA was confirmed. And further agreements were made:

Agreements: For Scenario-1/2, when reporting direct-path failure via indirect-path, use MCGFailureInformation message. FFS on whether additional IE needs to be introduced. For Scenario-1/2, if MCGFailureInformation is agreed for direct path failure recovery in P5, reuse T316 timer for the direct path failure recovery. For Scenario-1/2, confirm the WA that: for a remote UE and relay UE in RRC_CONNECTED, the network is expected to release the multipath configuration related to this relay at the remote UE before it releases the relay UE to RRC_IDLE/INACTIVE. No spec impact is foreseen. For Scenario-1/2, not pursue remote UE notifying network upon reception of notification message indicating relay UE handover. FFS whether rely on network to release configuration of relay UE at remote UE before relay UE handover, or rely on remote UE to suspend the indirect path upon reception of notification message indicating relay UE handover. For Scenario-1/2, no specification effort to handle the case when the relay UE moves to RRC_IDLE following expiry of dataInactivityTimer, i.e., not pursue relay UE notifying remote UE, and remote UE notifying network.

It was agreed that MCGFailureInformation message is used to report Uu-RLF in direct path by remote UE. Which message is used to carry PC5-RLF and Uu-RLF of relay UE is FFS. There are three options were proposed in the last meeting：

- Option 1: SidelinkUEInformationNR message.

- Option 2: MCGFailureInformation message.

- Option 3: A new message

For option 1, SidelinkUEInformationNR message is not suitable to be used for remote UE reports indirect path failure since indirect path failure includes two cases: 1) PC5-RLF between remote UE and relay UE, and 2) Uu-RLF of relay UE. SidelinkUEInformationNR message can be only used for PC5-RLF report since Uu-RLF of relay UE is non-sidelink event.

In Rel-17 U2N relay, SidelinkUEInformationNR message is sent by the relay UE while PC5-RLF is detected. The remote UE does not send SidelinkUEInformationNR message since there isn’t RRC connection when PC5 connection with the relay UE is RLF.

Furthermore, UE shall report all sidelink information in the SidelinkUEInformationNR message when it is initiated no matter it is changed or not, the signaling overhead of option 1 is higher than option 2. Consideration on SidelinkUEInformationNR message can’t be used to report indirect path failure for scenario 2, MCGFailureInformation or a new message can be used for both scenario 1 and scenario 2.

Proposal 2: For Scenario-1/2, remote UE reports indirect path failure via MCGFailureInformation or a new message.

In addition, new failure types of SL RLF (for scenario 1), ideal connection RLF (for scenario 2) and Uu RLF of relay UE (for scenario 1/2) need to be introduced in MCGFailureInformation or the new message.

Proposal 3: Introducing new failure types (SL RLF, ideal connection RLF, Uu RLF of relay UE) in MCGFailureInformation or the new message.

Path activation/deactivation for scenario 1/2

In RAN2#121bis-e meeting, it was agreed to reuse primary path and primary RLC entity for MP split bearer.

Agreements: The concept of the ‘primary path and primary RLC entity’ is adopted for each MP split bearer configuration according to the existing definition.

For legacy CA, SCells are configured by gNB via RRC signaling, and can be activated or deactivated via MAC CE in order to achieve power saving benefit. However, for multi-path scenario, direct path and indirect path can be in the same cell. Hence, the cell activation/deactivation concept is not suitable any more. Therefore, it is needed to introduce path activation/deactivation mechanism. Since Uu MAC CE can’t be transmitted to the remote UE via the indirect path. Path activation/deactivation can be done via MAC CE over direct path.

Proposal 4: Path activation/deactivation is introduced for multi-path for scenario 1/2.

Proposal 5: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of indirect path.

Whether split and non-split SRBs can be configured on the indirect path for scenario 1

In RAN2#122 meeting, the WA is primary path of the split SRB1 and SRB2 is always configured on direct path.

WA: For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. This does not preclude having the case where the UE switches the primary path to the indirect path for reporting after direct path failure. Agreements: For scenario 1, SRB1 and SRB2 are not decoupled in terms of support of non-split SRB on indirect path; i.e., if SRB1 can be supported on indirect path, so can SRB2.

RAN2 can confirm the WA.

Proposal 6: RAN2 to confirm the WA:

For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. The remote UE switches the primary path to the indirect path to report direct path failure after direct path failure is detected.

For the useful of non-split SRBs over indirect path, indirect path may be also in the PCell. Non-split SRB1 over indirect path doesn’t conflict with the PCell on direct path. Furthermore, which path is more reliable depends on the channel quality. When the remote UE is located in the cell edge, indirect path may be more reliable than direct path.

Proposal 7: For multi-path scenario 1, non-split SRBs can be configured on the indirect path.

Trigger of the RRC_IDLE/RRC_INACTIVE target relay UE to establish/resume the RRC connection for Scenario-1.

Similar as Rel-17 U2N relay, the serving relay UE of the remote UE configured with multi-path should be in RRC_CONNECTED. Additionally, when the remote UE performs path switch, the target relay UE can be in any RRC state. If the target relay UE is in RRC_IDLE /RRC_INACTIVE, it should enter RRC_CONNECTED triggered by the first RRC message received from the remote UE. In Rel-18 multi-path relay, the target relay UE in RRC_IDLE /RRC_INACTIVE can’t enter into RRC_CONNECTED if the SRB1 of the remote UE configured on direct path only.

In RAN2#121bis-e meeting, this issue was discussed, the agreements were listed below:

Agreements: When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 1. When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 1. FFS on need for additional condition. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path When split SRB1 with duplication is configured, the remote UE sends the RRCReconfigurationComplete message to gNB via both paths for Scenario 2. When one of the following conditions is met, the remote UE sends the RRCReconfigurationComplete message to gNB via the direct path for Scenario 2. - when primary RLC entity of split SRB1 is on direct path - when non-split SRB1 is configured on direct path

For the agreement of “A PC5-RRC trigger is specified at least for other cases.”, since the solution is only used for triggering IDLE/INACTIVE relay UE to enter CONNECTED state, the remote UE doesn’t know relay UE’s RRC state. If PC5-RRC is used, to avoid redundancy PC5-RRC message, the remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB. Furthermore, consideration on the compatibility, gNB should configure SRB1 on the indirect path if the target relay UE is a Rel-17 UE in IDLE/INACTIVE. Since gNB does not know the release/capability of IDLE UE, even if UE release will be introduced in UE capability, remote UE should acquire the release of the relay UE and indicate it to gNB.

In summary, the additional issues should be discussed for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- How to avoid the new trigger in PC5-RRC message cannot be interpreted by Rel-17 relay UE;

- How to avoid redundancy trigger message to CONNECTED relay UE.

Therefore, the additional enhancements should be considered:

- Remote UE should acquire the release/capability of the relay UE and indicate it to gNB;

- Remote UE can acquire the target relay UE’s RRC state from the relay UE or gNB.

Proposal 8: RAN2 discuss the following enhancements for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- Remote UE should acquire the release/capability of the relay UE and indicate it to gNB;

- Remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB.

T304/T420:

In RAN2#122 meeting, it was agreed T304-like and T420-like timer for MP configuration procedure.

Agreements: For Scenario-1, use T304-like timer for direct path addition and change. FFS on expiry behavior. For Scenario-1, use T420-like timer for indirect path addition and change. FFS on stop condition and expiry behavior. FFS if these two timers are new or reuse the existing timers.

In TS 38.331 the description of T420 timer is below:

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |
| T420 | Upon reception of the RRCReconfiguration message including sl-PathSwitchConfig | Upon successfully sending RRCReconfigurationComplete message (i.e., PC5 RLC acknowledgement is received from target L2 U2N Relay UE) | Perform the RRC re-establishment procedure as specified in 5.3.7. |

The stop condition and UE behavior after expiry of T420 are not suitable for MP. Since RRCReconfigurationComplete may be sent on direct path, there isn’t RRCReconfigurationComplete message on the indirect path. There are 3 options for stopping the T420:

- Option1: Upon successfully sending RRCReconfigurationComplete message on indirect path.

- Option2: Upon PC5 connection establishment successfully.

- Option3: Upon successfully receiving RRCReconfigurationCompleteSidelink

- Option4: Upon successfully receiving RRCReconfigurationSidelink message with sl-RLC-ChannelToAddModListPC5-r17 message..

For option 1, it reused the stop condition of T420 in Rel-17. However, it can be used for the case that duplicated split SRB1 is configured or primary path of split SRB1 or non-split SRB1 is configured on the indirect path. And it can’t be used for the cases that RRCReconfigurationComplete message is sent on the direct path only.

For option 2, there is the risk that indirect path setup failure even if PC5-S connection establishment is successfully.

For option 3, since the remote UE shall send RRCReconfigurationSidelink message to the relay UE to establish PC5 Relay RLC channels when reception of the RRCReconfiguration message to add the indirect path. This message can be used to trigger the IDLE/INACTIVE relay UE enter RRC_CONNECTED state. However, the relay UE will send RRCReconfigurationCompleteSidelink message to the remote UE only depending on the PC5 configuration, even if Uu connection has not established.

In addition, PC5 connection establishment and RRCReconfigurationSidelink interaction triggered by the remote UE may be happened before RRCReconfigurationComplete message, it will impact the stop condition of T420 of legacy Rel-17 remote UE. If option 2 or 3 is used, T420 can’t be reused.

For option 4, the target relay UE will send RRCReconfigurationSidelink with sl-RLC-ChannelToAddModListPC5-r17 message when it received RRCReconfiguration message from gNB to configure the SL RLC channel of remote UE. Therefore, both SL and Uu link of indirect path are established when the remote UE receives RRCReconfigurationSidelink with sl-RLC-ChannelToAddModListPC5-r17 message from the relay UE.

Proposal 9: For Rel-18 multi-path, the remote UE shall stop T420-like timer upon successfully receiving RRCReconfigurationSidelink with sl-RLC-ChannelToAddModListPC5-r17 message from the relay UE.

Proposal 10: Reuse T420 for MP indirect path addition or indirect path modification configuration procedure.

For indirect path addition procedure, when T420 expired, indirect path addition failure. The remote UE shall continue using the configuration used prior to the reception of RRCReconfiguration message. For indirect path modification procedure, target indirect path establishment failure. The remote UE can continue using the direct path configuration before. But for the source indirect path, it may be released. Therefore, for both indirect path addition and modification procedure, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message. Furthermore, the remote UE should inform network about the reconfiguration failure.

Proposal 11: For Rel-18 multi path, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration failure when T420 expired.

For direct path addition/modification procedure, T304 can be reused.

| Counter | Reset | Incremented | When reaching max value |
| --- | --- | --- | --- |
| T304 | Upon reception of RRCReconfiguration message including reconfigurationWithSync for the MCG which does not include sl-PathSwitchConfig, or upon reception of RRCReconfiguration message including reconfigurationWithSync for the SCG not indicated as deactivated in the NR or E-UTRA message containing the RRCReconfiguration message or upon conditional reconfiguration execution i.e. when applying a stored RRCReconfiguration message including reconfigurationWithSync. | Upon successful completion of random access on the corresponding SpCell For T304 of SCG, upon SCG release | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. For T304 of SCG, inform network about the reconfiguration with sync failure by initiating the SCG failure information procedure as specified in 5.7.3. |

Proposal 12: Reuse T304 timer for MP direct path addition or indirect path modification configuration procedure.

For both direct path addition and modification procedure, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message. Furthermore, the remote UE should inform network about the reconfiguration with sync failure.

Proposal 13: For Rel-18 multi path, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T304 is expired.

Indirect path change of scenario 2 (Case G):

In the study phase, the following cases were discussed for multi-path scenario 2:

- A. The remote UE configured only on the direct path adds the indirect path under the same gNB;

- B. The remote UE configured only on the indirect path adds the direct path under the same gNB;

- C. The remote UE configured with multi-path releases the indirect path;

- D. The remote UE configured with multi-path releases the direct path;

- E. The remote UE configured with multi-path changes the serving cell of the remote UE for the direct path while keeping the serving relay UE for the indirect path under the same gNB;

- F. The remote UE configured with multi-path keeps the serving relay UE for the indirect path and the serving cell of the remote UE for the direct path while the serving relay UE changes the serving cell of the relay UE under the same gNB;

- G. The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB.

During the last RAN2 meetings, it has been agreed that Case A/C are supported, and Case B/D/E/F are not supported. The only remaining issue is whether Case G can be supported. In the RAN2#120 meeting, the following agreement was reached for Case G:

Agreements: For Scenario 2, whether to support Case G is discussed in normative phase, but RAN2 will not do additional work to enable it for Scenario 2 over Scenario 1.

In RAN2#122 meeting, there are some discussions on whether more than one relay UE can be supported for one remote UE in multi-path scenario 2. For scenario 2, the interface between relay UE and remote UE are ideal. Hence, they should be co-located or connected via fiber. In this case, there is no strong motivation to use more than one relay UE for a remote UE. Hence, it is unnecessary to support Case G.

Proposal 14: For scenario 2, more than one relay UE for a remote UE is not supported.

Proposal 15: For scenario 2, Case G (The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB) is not supported.

Conclusion

According to the above discussion, the proposals on the above issue are as follows:

Proposal 1: RAN2 to confirm the WA:

For Scenario-1/2, MP remote UE is configured with a single cell group, i.e., MCG, for the direct path, and SL configuration, for the indirect path.

Proposal 2: For Scenario-1/2, remote UE reports indirect path failure via MCGFailureInformation or a new message.

Proposal 3: Introducing new failure types (SL RLF, ideal connection RLF, Uu RLF of relay UE) in MCGFailureInformation or the new message.

Proposal 4: Path activation/deactivation is introduced for multi-path for scenario 1/2.

Proposal 5: Introduce Path Activation/Deactivation MAC CE over direct path to control the activation/deactivation of indirect path.

Proposal 6: RAN2 to confirm the WA:

For scenario 1, primary path of the split SRB1 and SRB2 is always configured on direct path. The remote UE switches the primary path to the indirect path to report direct path failure after direct path failure is detected.

Proposal 7: For multi-path scenario 1, non-split SRBs can be configured on the indirect path.

Proposal 8: RAN2 discuss the following enhancements for triggering the idle/inactive relay UE to RRC_CONNECTED via PC5-RRC message:

- Remote UE should acquire the release/capability of the relay UE and indicate it to gNB;

- Remote UE should acquire the target relay UE’s RRC state from the relay UE or gNB.

Proposal 9: For Rel-18 multi-path, the remote UE shall stop T420-like timer upon successfully receiving RRCReconfigurationSidelink with sl-RLC-ChannelToAddModListPC5-r17 message from the relay UE.

Proposal 10: Reuse T420 for MP indirect path addition or indirect path modification configuration procedure.

Proposal 11: For Rel-18 multi path, the remote UE shall fallback to continue using the direct path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration failure when T420 expired.

Proposal 12: Reuse T304 timer for MP direct path addition or indirect path modification configuration procedure.

Proposal 13: For Rel-18 multi path, the remote UE shall fallback to continue using the indirect path configuration used prior to the reception of RRCReconfiguration message and inform network about the reconfiguration with sync failure when T304 is expired.

Proposal 14: For scenario 2, more than one relay UE for a remote UE is not supported.

Proposal 15: For scenario 2, Case G (The remote UE configured with multi-path changes to a new relay UE for the indirect path while keeping the direct path under the same gNB) is not supported.

Reference

- Draft_R2_121bis_meeting_report_v1

- Draft_R2_122_meeting_report_v1