3GPP TSG-RAN WG2 Meeting #123                                                        R2-2307557

Toulouse, France, 21st - 25th Aug, 2023

Source:              CATT

Title:	             Other Remaining Issues in SL-U

Agenda Item:	7.15.4

Document for:	Discussion and Decision

Introduction

In this contribution, the following remaining issues will be discussed?

- Issue 1: How to maintain the sl-drx-HARQ-RTT-Timer in case of multiple PSFCH occasions for GC?

- Issue 2: Whether any enhancement in SL RLF is needed with the introduction of SL-U?

Discussion

Multiple PSFCH occasions

In the RAN2#122 meeting, the sl-drx-HARQ-RTT-Timer maintenance in case of multiple PSFCH occasions for UC is agreed as below:

Agreements on multiple PSFCH occasions

1: 	Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if HARQ A/N is successfully transmitted in one PSFCH occasion, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the corresponding PSFCH transmission carrying the SL HARQ feedback.” is agreed at least for UC.

2: 	Working assumption “In case of multiple PSFCH occasion per PSCCH/PSSCH, if LBT failure happens in all PSFCH occasions, Rx UE starts the sl-drx-HARQ-RTT-Timer for the corresponding Sidelink process in the first slot after the end of the last PSFCH occasion for the SL HARQ feedback.” is agreed at least for UC.

For GC, how to maintain the sl-drx-HARQ-RTT-Timer is still FFS. For GC, multiple UEs will provide HARQ feedback through the PSFCH occasions.  Some Rx UEs may perform successful PSFCH transmission, while the other Rx UE may fail due to LBT failure. In this case, in order to ensure Rx UE can receive the retransmission, Rx UE had better start the sl-drx-HARQ-RTT-Timer for the corresponding sidelink process in the first slot after the end of the last PSFCH occasion for SL HARQ feedback.

Proposal 1: For SL GC, Rx UEs start the sl-drx-HARQ-RTT-Timer for the corresponding sidelink process in the first slot after the end of the last PSFCH occasion for SL HARQ feedback.

SL RLF

According to the current spec, when numConsecutiveDTX reaches sl-maxNumConsecutiveDTX, UE will indicate HARQ-based Sidelink RLF detection to RRC. If cases of multiple PSFCH occasions are introduced per PSCCH/PSSCH, some PSFCH occasions may have no PSFCH transmission due to LBT, but according to the current MAC spec, the numConsecutiveDTX will still be accumulated. This will increase probability of SL RLF. In order to avoid unnecessary trigger of SL RLF, it is proposed that for multiple PSFCH occasions can be regarded as one unit, and only no HARQ FB is received on all the PSFCH occasions in this unit, the numConsecutiveDTX can be added by 1.

Proposal 2: Only when no HARQ FB is received on all the PSFCH occasions which corresponding to one PSCCH/PSSCH is received, the number of HARQ DTX can be added by 1.

In addition, in RAN2#121bis, it was agreed that:

Confirm the working assumption:

UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets.

In RAN2#122 meeting, the issue of whether new cause value should be introduced in the SUI for SL C-LBT failure was discussed in some contributions. In the following, we will analyze whether new cause value is needed when UE has triggered SL C-LBT failure in all RB sets.

In [1], it captured that:

SL-Failure-r16 ::= SEQUENCE { sl-DestinationIdentity-r16 SL-DestinationIdentity-r16, sl-Failure-r16 ENUMERATED {rlf,configFailure, drxReject-v1710, spare5, spare4, spare3, spare2, spare1} } sl-Failure Indicates the sidelink cause for the sidelink RLF (value rlf), sidelink AS configuration failure (value configFailure) and the rejection of sidelink DRX configuration (value drxReject-v1710) for the associated destination for unicast..

Since RAN2 has agreed that UE triggers SL RLF for all UC connections when UE has triggered consistent SL LBT failure in all RB sets, hence, the cause value “rlf” is enough.

Proposal 3: No new cause value should be introduced in SUI when the SL failure is caused by SL C-LBT failure.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: For SL GC, Rx UEs start the sl-drx-HARQ-RTT-Timer for the corresponding sidelink process in the first slot after the end of the last PSFCH occasion for SL HARQ feedback.

Proposal 2: Only when no HARQ FB is received on all the PSFCH occasions which corresponding to one PSCCH/PSSCH is received, the number of HARQ DTX can be added by 1.

Proposal 3: No new cause value should be introduced in SUI when the SL failure is caused by SL C-LBT failure.

Reference

- TS38.331  Radio Resource Control (RRC) protocol specification (Release 17)