3GPP TSG-RAN WG2 Meeting #123	 R2-2307571

Toulouse, France, 21 – 25 August, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.321 | CR | 1638 | rev | - | Current version: | 17.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Inter-UE Coordination Information MAC CE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh-Core |  |  |  |  |  | Date: |  |  | 2023-08-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In RAN2#118 meeting, when discussing R2-2205639, it was agreed that it is up to UE A implementation to decide whether and how to select a subset of RC(s) to be included in IUC INFO MAC CE. However, such agreement is not captured in current TS 38.321. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a note to capture the agreement that it is up to UE implementation to decide whether and how to select a subset of RC(s) to be included in Inter-UE Coordination Information MAC CE. Impact analysis Impacted 5G architecture options: NR SA, NR-DC Impacted functionality: Inter-UE coordination Inter-operability: If the network is implemented according to this CR while the UE is not, there is no inter-operability issue. If the UE is implemented according to this CR while the network is not, there is no inter-operability issue. If one UE is implemented according to this CR while the other UE is not, there is no inter-operability issue. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Whether and how to select a subset of RC(s) to be included in IUC INFO MAC CE is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START OF CHANGE

### 6.1.3	MAC Control Elements (CEs)

<<<<<<<<Skipped>>>>>>>>

#### 6.1.3.53	Inter-UE Coordination Information MAC CE

The Inter-UE Coordination Information MAC CE is identified by a MAC subheader with LCID as specified in Table 6.2.4-1. The priority of the Inter-UE Coordination Information MAC CE is fixed to '1' for Logical Channel Prioritization (LCP) procedure. It has a variable size with following fields:

-	RT: This field indicates the resource set type, i.e., preferred resource set or non-preferred resource set, as the codepoint value of the SCI format 2-C resourceSetType field as specified in TS 38.212 [9].

-	RSL: This field indicates the location of reference slot, as the codepoint value of the SCI format 2-C referenceSlotLocation field as specified in TS 38.212 [9]. The length of the field is 17 bits. If the length of referenceSlotLocation field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 17 bit, this field contains referenceSlotLocation field using the LSB bits;

-	LSIi: This field indicates lowest subchannel indices for the first resource location of each TRIV, as the codepoint value of the SCI format 2-C lowestIndices field as specified in TS 38.212 [9]. LSI0 indicates lowest subchannel indices for the first resource location of TRIV within the first resource combination, LSI1 indicates lowest subchannel indices for the first resource location of TRIV within the second resource combination and so on. The length of the field is 5 bits. If the length of lowestIndices field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 5 bit, this field contains lowestIndices field using the LSB bits;

-	RCi: This field indicates resource combination, as the codepoint value of the SCI format 2-C resourceCombination field as specified in TS 38.212 [9]. RC0 indicates the first resource combination, RC1 indicates the second resource combination and so on. The length of the field is 26 bits. If the length of resourceCombination field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 26 bit, this field contains resourceCombination field using the LSB bits;

-	First resource locationi-1: This field indicates first resource location, as the codepoint value of the SCI format 2-C firstResourceLocation field as specified in TS 38.212 [9]. First Resource Location0 indicates the first resource location for the second resource combination, First Resource Location1 indicates the first resource location for the third resource combination and so on. The length of the field is 13 bits. If the length of firstResourceLocation field in SCI format 2-C as specified in TS 38.212 [9] is shorter than 13 bit, this field contains firstResourceLocation field using the LSB bits;

-	R: Reserved bit, set to 0.

NOTE X:	It is up to UE implementation to decide whether and how to select a subset of RC(s) to be included in Inter-UE Coordination Information MAC CE.

Figure 6.1.3.53-1: Inter-UE Coordination Information MAC CE

END OF CHANGE