3GPP TSG-RAN WG2 Meeting #123	R2-2307575

Toulouse, France, 21st – 25th of August 2023

Agenda item:	7.25.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On RAN2 Impacts of Air-To-Ground (ATG) in Rel-18

WID/SID:	NR_ATG-Core

Document for:		Discussion and Decision

# Introduction

During RAN2#121bis-e it has been briefly discussed what are the requirements for Air-To-Ground (ATG). In particular whether SIB19 is relevant for signalling the base station’s (BS’s) position. As a result, the LS to RAN4 has been sent in [1]. RAN4 has considered RAN2 inputs during their last meeting (RAN4#107, held in May 2023) and replied in [2] and [3]. In this paper we analyze what are the RAN2 implications of the decisions made by RAN4 for supporting ATG.

# Discussion

## On the Base Station’s location accuracy

One of the main concerns expressed in [1] was related to disclosing the location of ground BS in ATG system. Due to that, RAN2 has asked RAN4 what kind of accuracy is required. Even though there is no solid number given in RAN4 response [2], it is clearly stated the BS is allowed not to provide its exact location.

Observation 1: RAN4 has confirmed the location of BS does not have to be exact and some inaccuracy is allowed.

However, it is not clear how inaccurate this location can be. As per [2], BS’s location does not need to be specified and RAN4 has not completed their study on the required accuracy.

Observation 2: RAN4 believes BS’s location accuracy does not have to be specified, while the actual requirements are still under discussion in RAN4.

Interestingly, RAN4 points out that the UE will use the BS’s signalled location for time and frequency pre-compensation calculations but will not consider the error due to misalignment between the indicated and the actual BS location. Thus, the misalignment is likely to have an impact on the PRACH formats that will be used in ATG. The larger the misalignment will be, the lower is the allowed number of PRACH formats (i.e. large inaccuracy between the actual and signalled BS’s location would require to use only these longer PRACH preamble formats).

Observation 3: The larger the misalignment between the actual and indicated BS position, the more restricted is the choice of PRACH formats.

However, it seems this can be addressed via RAN4 requirements, while RAN2 can just ensure this BS location is signalled via system information.

Proposal 1: RAN2 does not specify any BS location accuracy (e.g. via distanceThresh) but indicates just the reference location (e.g. using referenceLocation IE) to provide the approximate BS’s position.

## Other ATG-related SIB components

In addition to the BS’s location inaccuracy, [2] has also indicated that ATG system requires koffset signalling. It is also included in SIB19 (see: cellSpecificKoffset in TS 38.331). It is optionally signalled as a part of NTN-Config and consumes 10 bits.

Observation 4: koffset which is required by RAN4, can be provided using cellSpecificKoffset which is a part of NTN-Config.

Besides the LS in [2], RAN4 has also identified another ATG-related need, which shall be addressed by RAN2. As suggested in [3], system information for ATG should indicate which set of cell reselection requirements has to be applied. RAN4 agreed to consider either set 1 (Rel-15 NR requirements) or set 2 (Rel-17 HST requirements for cell reselections).

Observation 5: As indicated by RAN4, system information needs to indicate which set of cell reselection requirements applies for a particular ATG cell.

This information is needed for the neighbours, but apparently can be signalled by the serving cell, jointly for all neighbours.

Observation 6: Information on which set of cell reselection requirements applies for ATG needs to be signaled in serving cell’s SIB, commonly for all neighbours.

## Which SIB to use

It is tempting to conclude that SIB19 is a perfect candidate for signaling referenceLocation and cellSpecificKoffset. Most of the SIB19 content is OPTIONAL, so ATG version of SIB19 could be deprived of most of the unnecessary details.

Observation 7: Most of the IEs in SIB19 are OPTIONAL, so ATG version of SIB19 may not consume excessive number of bits.

On the other hand, SIB19 was defined as a system information block that is sent from the satellite and not from the ground BS. Furthermore, there could be Rel-17 UEs that will be able to read ATG-related SIB19 and could perceive this ATG base station as NTN-type of base station. The reception of SIB19 also triggers other specific procedures (as per TS 38.331), such as the start of the timer T430, which is probably not needed for ATG.

Observation 8: SIB19 was defined as a system information block that is sent from the satellite and not from the ground BS.

Observation 9: Another SIB19 which is not for NTN purpose could be also problematic for Rel-17 UEs.

Observation 10: The reception of SIB19 triggers specific procedural steps, such as the start of the timer T430, which is not applicable to ATG.

Thus, we think it is probably better to use other SIB than SIB19 for signalling ATG-related parameters, such as referenceLocation and cellSpecificKoffset.

Proposal 2: Use other SIB than SIB19 (e.g. SIB1) for providing referenceLocation and cellSpecificKoffset in ATG cell.

# Conclusion

The following observations and proposals were made in this document:

Observation 1: RAN4 has confirmed the location of BS does not have to be exact and some inaccuracy is allowed.

Observation 2: RAN4 believes BS’s location accuracy does not have to be specified, while the actual requirements are still under discussion in RAN4.

Observation 3: The larger the misalignment between the actual and indicated BS position, the more restricted is the choice of PRACH formats.

Proposal 1: RAN2 does not specify any BS location accuracy (e.g. via distanceThresh) but indicates just the reference location (e.g. using referenceLocation IE) to provide the approximate BS’s position.

Observation 4: koffset which is required by RAN4, can be provided using cellSpecificKoffset which is a part of NTN-Config.

Observation 5: As indicated by RAN4, system information needs to indicate which set of cell reselection requirements applies for a particular ATG cell.

Observation 6: Information on which set of cell reselection requirements applies for ATG needs to be signaled in serving cell SIB, commonly for all neighbours.

Observation 7: Most of the IEs in SIB19 are OPTIONAL, so ATG version of SIB19 may not consume excessive number of bits.

Observation 8: SIB19 was defined as a system information block that is sent from the satellite and not from the ground BS.

Observation 9: Another SIB19 which is not for NTN purpose could be also problematic for Rel-17 UEs.

Observation 10: The reception of SIB19 triggers specific procedural steps, such as the start of the timer T430, which is not applicable to ATG.

Proposal 2: Use other SIB than SIB19 (e.g. SIB1) for providing referenceLocation and cellSpecificKoffset in ATG cell.

# References

- R2-2304565 Reply LS on applicability of SIB19 for NR ATG 3GPP TSG RAN WG2#121bis-e Online meeting: April 17-26, 2023

- R4-2310058 Reply LS on applicability of SIB19 for NR ATG 3GPP TSG-RAN WG4 Meeting # 107 Incheon, KR, May 22 – May 26 2023

- R4-2310152 LS on signaling for NR ATG 3GPP TSG-RAN WG4 Meeting # 107 Incheon, KR, May 22 – May 26 2023