3GPP TSG-RAN WG2 Meeting #123	R2-2307577

Toulouse, France, 21st-25th of August 2023

Agenda item:	7.4.2.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On Reference Configuration, Subsequent LTM and L3 mobility

WID/SID:	NR_Mob_enh2-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

According to the agreements from RAN2#121 [1] and RAN2#121bis [2]:

- RAN2#121 Reference config can be empty In the RRC procedures, the candidate delta configuration is applied on top of the reference configuration to form a complete candidate configuration when the UE receives the LTM configuration (before the LTM cell switch). UE implementation can postpone that step to the reception of the LTM cell switch command. FFS Discuss early vs late compliance check. In the RRC procedures, the complete candidate configuration is applied and replacing the current UE configuration (at the time of reconfiguration execution/cell switch), by a RRC reconfiguration procedure that makes replacements of configuration but doesn’t necessarily reset MAC, RLC or PDCP. FFS whether we can rely on a modified version of the reconfiguration procedure with fullconfig flag set. FFS how to make sure the procedures work in case the LTM candidate configuration is a complete configuration. - RAN2#121bis […] Whether the Reference configuration is a complete configuration or not is up to the network implementation. Reference configuration + LTM candidate configuration (in combination) has to be a complete configuration. The reference configuration is always explicitly signalled (not automatically derived from any other config, e.g. current). […] The UE may perform early decoding and early validity check. FFS whether Early validity check triggers early re-establishment. FFS the possible timing, FFS subset of cells, FFS if need to specify anything or just up to UE impl, FFS if other signalling to notify network is needed.

This document discusses the aspects of the RRC Reconfiguration for LTM, with a particular focus on how reference and delta configuration should be configured and LTM interworking with L3 mobility.

# 2	Target Cell RRC Configuration Provision in LTM

## 2.1	RRC Reconfiguration Options Considering availability Of Reference

### 2.1.1 Reference Configuration is empty

When the UE receives the cell switch command, it applies the target cell configuration on top of the current RRC configuration. If the target cell configuration is delta, this could create problems at the time of cell switch in subsequent LTM since the target cell delta is built on top of the initial serving cell configuration and not on top of the current one. In such case the network would have to ensure that the delta configurations are able to be applied on top of each serving without any problems.

On the other hand, if a complete configuration is provided to the UE at the time of LTM preparation for the target cells, the UE can simply apply that on top of the serving in case of subsequent LTM. Obviously, the use of complete configuration in case of empty reference ensures smooth operation in dynamic switching (subsequent LTM).

Observation 1: Use of Complete Configuration in case of empty reference ensures smooth operation in dynamic switching (subsequent LTM).

### 2.1.2 Reference Configuration is a non-empty configuration

At the time of the LTM preparation the source cell provides to the UE the Reference configuration and the target cell configurations (which can be delta or not) of the prepared target cells.

In case of a non-empty Reference configuration there are two options, (a) the reference configuration is a complete configuration, thus containing all the needed RRC IEs for cell switch or (b) the reference is a non-complete configuration, thus containing parts of the needed RRC IEs for cell switch. The latter approach has the benefit of the reduction of the size of the contained RRCReconfiguration message.

In any case the UE will have to apply the target cell (delta) configuration on top of the reference in order to create a complete configuration for the Target cell and as agreed it is up to the network implementation whether a complete or non-complete Reference will be provided to the UE.

Compared to the case where Reference Configuration is empty, the non-empty reference has less signalling overhead and at the same time it builds on top of the delta configuration principles to ensure that the UE in case of subsequent LTM will always have the correct configuration for the target cell, which has been built at the time of the preparation of the target cell and is not subject to current UE RRC Configuration.

Observation 2: Reference configuration plus delta configuration for the target cell can ensure that the UE will have the correct configuration for the target cell which has been built at the time of preparation in case of subsequent LTM.

## 2.2 	Subsequent LTM

According to the WID we should:

- To specify mechanism and procedures of L1/L2 based inter-cell mobility for mobility latency reduction:

- Configuration and maintenance for multiple candidate cells to allow fast application of configurations for candidate cells [RAN2, RAN3]

- Dynamic switch mechanism among candidate serving cells (including SpCell and SCell) for the potential applicable scenarios based on L1/L2 signalling [RAN2, RAN1]

- […]

To our understanding the above description has two aspects, one relevant to the single cell change using L1/2 Triggered Mobility and one relevant to Dynamic Switching, what has been lately referred to as Subsequent LTM. In single cell change LTM, the UE will release the configurations upon the cell change and the target of this approach is the interruption reduction during the cell change, whereas in the subsequent LTM, the UE will maintain the target cell configurations for subsequent cell switches (without requiring new configurations provisions) with the cost of the resource reservation in the network side.

When comparing single cell change using LTM to subsequent LTM, the network has to do more target cell preparations, since each cell change requires a cell preparation. In subsequent LTM, the network maintains the target cell configurations for future use, and it updates them when they become outdated or when the UE find new more suitable target cells, thus one preparation can be used multiple times. On the other hand, subsequent LTM has the cost of increased resource reservation in the network side since the UE configuration is maintained for increased amount of time. Simulation evaluations in the literature ([3]) highlight the increased number of preparations in case of since cell change LTM compared to subsequent LTM, as well as the high resource reservation cost in case of Subsequent LTM.

Observation 3: In single cell change LTM, the target of this approach is the interruption reduction during the cell change which is achieved with the skipping of certain procedures (e.g., RACH for RACH-less cell change), whereas in the subsequent LTM the target is to maintain the target cell configurations for subsequent cell switches (without requiring new configurations provisions).

Observation 4: Subsequent LTM has higher cost in terms of resource reservation in the network side compared to single cell change LTM.

Proposal 1: Both options of single cell change LTM and subsequent LTM are supported by RAN2 since they have different objectives.

In case of subsequent LTM, the network needs to ask the UE to release the target cell configurations; this can be done with an RRC Reconfiguration message that explicitly asks the UE to release the target cell LTM configurations. However, this would not be efficient as it requires the network to send an RRC Reconfiguration to ask the UE to release the target cells. However, other approaches could be more efficient since the network can estimate the likelihood of a subsequent LTM (or of a ping pong) over time. Thus, the network could configure the UE to release the prepared cells after a specific time after the configuration or the cell change, without requiring an RRC Reconfiguration.

Proposal 2: In case of subsequent LTM, cell configurations for LTM can be maintained by the UE for a specific amount of time upon the reception of RRC Reconfiguration or the cell change.

## 2.3	LTM and L3 mobility interworking

When attached to a cell, the UE may be configured to provide L3 measurements to the serving gNB. Based on L3 measurement, the serving/source gNB can take the decision (and  send the Handover Request) to start L3 mobility to a target gNB. While preparing for L3 mobility, it is also possible that the target gNB prepares for LTM as well and provide the LTM configuration as part of L3 handover configuration (e.g. baseline handover or CHO configuration).

Observation 5: Nothing prevents a target gNB to provide LTM configuration as part of the target cell configuration in case of L3 (Baseline HO or Conditional HO) mobility.

In this case, the UE upon the performance of the L3 mobility to the target cell, it will directly apply the target cell configuration for that cell, which will include the LTM configuration for multiple other cells. This scenario is captured in the figure X below.

Figure 1: LTM and L3 mobility interworking scenario

As illustrated in Figure:

- Point 1: UE is configured with L3 mobility Baseline HO or CHO.

- Point 2: the UE applies the target cell configuration. In case of baseline HO, points 1 and 2 can be the same. In case of CHO the UE will evaluate the target cell configuration(s) and apply it.

- Point 3: the UE is triggered by the network to switch to cell 1.2. The UE is not needed to be again configured with cell 1.2 LTM configuration, since this is already provided to the UE at point 1.

Proposal 3: The target gNB can provide LTM configuration as part of the L3 mobility configuration.

# 3	Conclusion

This document has made the following observations:

Observation 1: Use of Complete Configuration in case of empty reference ensures smooth operation in dynamic switching (subsequent LTM).

Observation 2: Reference configuration plus delta configuration for the target cell can ensure that the UE will have the correct configuration for the target cell which has been built at the time of preparation in case of subsequent LTM.

Observation 3: In single cell change LTM, the target of this approach is the interruption reduction during the cell change which is achieved with the skipping of certain procedures (e.g., RACH for RACH-less cell change), whereas in the subsequent LTM the target is to maintain the target cell configurations for subsequent cell switches (without requiring new configurations provisions).

Observation 4: Subsequent LTM has higher cost in terms of resource reservation in the network side compared to single cell change LTM.

Observation 5: Nothing prevents a target gNB to provide LTM configuration as part of the target cell configuration in case of L3 (Baseline HO or Conditional HO) mobility.

And the following proposals:

Proposal 1: Both options of single cell change LTM and subsequent LTM are supported by RAN2 since they have different objectives.

Proposal 2: In case of subsequent LTM, cell configurations for LTM can be maintained by the UE for a specific amount of time upon the reception of RRC Reconfiguration or the cell change.

Proposal 3: The target gNB can provide LTM configuration as part of the L3 mobility configuration.

# References

[1] 		R2_121 Chair Notes 2023-03-03 EOM

[2]		R2_121bis-e Chair Notes EOM.docx

[3]	Alperen Gündogan, Akin Badalioglu, Panagiotis Spapis, Ahmad Awada, „On the Modelling and Performance Analysis of Lower Layer Mobility in 5G-Advanced”. WCNC 2023