3GPP TSG-RAN WG2 Meeting #123	R2-2307585

Toulouse, France 21st – 25th of August 2023

Agenda item:	7.8.5

Source:	Nokia, Nokia Shanghai Bell

Title:	On the Remaining Aspects for Supporting PC5-based BRID and DAA

WID/SID:	NR_UAV - Release 18

Document for:	Discussion and Decision

# 1	Introduction

During RAN2#121bis (April 2023) the LS to SA2 in [1] has been approved. In [1] RAN2 has informed that Detect and Avoid (DAA) in Rel-18 is supported using only the broadcast mode. In addition, [1] has also asked if A2X (Aircraft to Anything) communication can rely on the existing QoS framework defined for Sidelink when V2X was considered or whether new handling is needed. SA2 has addressed our questions during their May 2023 meeting and responded in [2]. In this paper we provide our further view on what it means for BRID and DAA related Rel-18 work in RAN2.

# 2	Discussion

## 2.1 	DAA over Unicast

Release 18 Work Item Description (WID) on Uncrewed Aerial Vehicles among the others, defines the following objective [3]:

3. Specify the support for UAV identification broadcast (BRID) in NR PC5. Support of DAA using the same framework as BRID without DAA specific enhancements can be considered [RAN2]. Note:. UAV use of NR PC5 is to be used only in designated bands as defined in regulation for BRID/DAA use.

In SA2 response the following was suggested [2]:

SA2 specified DAA mechanism using broadcast, as well as DAA mechanism using unicast over NR PC5 by reusing the unicast mode V2X communication as specified in TS 23.287. Because the contents related to DAA transmitted over PC5 reference point are transparent not only to the A2X layer but also to the AS layer, SA2's understanding is that DAA specific enhancements to the existing unicast communication are not needed and the work carried out in RAN2 supports what is defined in SA2

As per [3], DAA can be considered (supported), but only using the same framework as defined for BRID. As a reminder, BRID relies on broadcast via sidelink. So logically, if the same framework without any changes is to be used, then DAA should also use broadcast only.

Observation 1: Work Item Description indicates DAA can be supported if it reuses the same framework as agreed for BRID.

On the other hand, SA2 claims that in their understanding no AS layer enhancements are needed to support unicast-based DAA.

Observation 2: According to SA2, no AS layer enhancements are needed to support unicast-based DAA.

Thus, if supporting DAA via unicast is indeed limited to introducing a RAN2 capability for UAV UE which covers sidelink unicast, then this could be doable in Rel-18. However, if any additional RAN2 work is needed besides that, then we believe there is not capacity left in this WI.

Proposal 1: Unicast-based DAA can be supported in Rel-18, but only its introduction does not require additional RAN2 work besides defining a sidelink capability.

## 2.2 	QoS Requirements for A2X

Second part of the response LS [2] was focused on Quality of Service (QoS) and whether there will be separate QoS requirements for A2X (i.e. BRID and DAA). RAN2 has agreed to reuse the same framework as defined in the past for V2X. SA2 has responded the following:

SA2 answer: SA2 discussed that defining new PQI(PC5 5QI) value(s) is beneficial for BRID and DAA over NR PC5 so that the AS layer can distinguish A2X services from V2X services if differentiated handling is needed. The related CR work will be progressed in Rel-18. For BRID and DAA over LTE PC5, it is considered that the existing QoS framework can be used

So RAN2 may use the same framework, but expect that a specific, UAV-related value of PQI will be standardized by SA2.

Observation 3: BRID and DAA can rely on the same QoS framework as defined for V2X. A separate PQI is expected to be defined by SA2 for NR SL.

Thus, RAN2 can wait with specification updates until SA2 addresses this topic in one of their next WG meetings.

Proposal 2: RAN2 updates the NR specification (e.g. by pointing to a proper table with a list of PQIs), but only after the topic is resolved in SA2.

Another related open point in RAN2 is whether a separate pool of Tx sidelink resources should be defined for A2X or not. This was captured in RAN2#121:

FFS whether different resource pools are needed for UAV services

For NR PC5 resource pools are configured within BWP-specific configuration. For NR PC5 Mode 2 the following parameter would apply: sl-TxPoolSelectedNormal-r16. There can be up to maxNrofTXPool-r16 of such pools configured per BWP and the maximum value defined in NR RRC is 8. Related ASN.1 excerpt is provided below:

SL-BWP-PoolConfig information element -- ASN1START -- TAG-SL-BWP-POOLCONFIG-START SL-BWP-PoolConfig-r16 ::= SEQUENCE { sl-RxPool-r16 SEQUENCE (SIZE (1..maxNrofRXPool-r16)) OF SL-ResourcePool-r16 OPTIONAL, -- Cond HO sl-TxPoolSelectedNormal-r16 SL-TxPoolDedicated-r16 OPTIONAL, -- Need M sl-TxPoolScheduling-r16 SL-TxPoolDedicated-r16 OPTIONAL, -- Need N sl-TxPoolExceptional-r16 SL-ResourcePoolConfig-r16 OPTIONAL -- Need M } SL-TxPoolDedicated-r16 ::= SEQUENCE { sl-PoolToReleaseList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolID-r16 OPTIONAL, -- Need N sl-PoolToAddModList-r16 SEQUENCE (SIZE (1..maxNrofTXPool-r16)) OF SL-ResourcePoolConfig-r16 OPTIONAL -- Need N } SL-ResourcePoolConfig-r16 ::= SEQUENCE { sl-ResourcePoolID-r16 SL-ResourcePoolID-r16, sl-ResourcePool-r16 SL-ResourcePool-r16 OPTIONAL -- Need M } SL-ResourcePoolID-r16 ::= INTEGER (1..maxNrofPoolID-r16) -- TAG-SL-BWP-POOLCONFIG-STOP -- ASN1STOP

Observation 4: For NR PC5 Mode 2 the UE can be configured with up to 8 Tx resource pools per BWP.

However, one may think that if BRID and DAA will have a separate PQI, it shall also have a dedicated resource pool.

Proposal 3: RAN2 is asked to consider if separate PQI for BRID and DAA over NR SL justifies the introduction of a dedicated SL Tx resource pool.

# 3	Conclusion

The following observations and proposals have been made in this paper:

Observation 1: Work Item Description indicates DAA can be supported if it reuses the same framework as agreed for BRID.

Observation 2: According to SA2, no AS layer enhancements are needed to support unicast-based DAA.

Proposal 1: Unicast-based DAA can be supported in Rel-18, but only its introduction does not require additional RAN2 work besides defining a sidelink capability.

Observation 3: BRID and DAA can rely on the same QoS framework as defined for V2X. A separate PQI is expected to be defined by SA2 for NR SL.

Proposal 2: RAN2 updates the NR specification (e.g. by pointing to a proper table with a list of PQIs), but only after the topic is resolved in SA2.

Observation 4: For NR PC5 Mode 2 the UE can be configured with up to 8 Tx resource pools per BWP.

Proposal 3: RAN2 is asked to consider if separate PQI for BRID and DAA over NR SL justifies the introduction of a dedicated SL Tx resource pool.

# References

- R2-2304564 LS on BRID and DAA broadcast over LTE and NR PC5 	3GPP TSG-RAN WG2 Meeting #121bis e-Meeting, 17th April – 26th April 2023

- S2-2307781 LS on BRID and DAA broadcast over LTE and NR PC5 (S2-2306322/R2-2304564) 	3GPP TSG-WG SA2 Meeting #157	Berlin, Germany, May 22 – 26, 2023

- RP-230782 Revised WID: NR Support for UAV (Uncrewed Aerial Vehicles) 	3GPP TSG RAN Meeting #99 Rotterdam, Netherlands, March 20th – 23rd 2023.