3GPP TSG-RAN WG2 Meeting #123	R2-2307586

Toulouse, France, 21 – 25 August 2023

Agenda item:	7.25.4

Source:	Nokia, Nokia Shanghai Bell

Title:	On CP and UP Latency for IMT-2020 NTN Self Evaluation

WID/SID:	FS_IMT-2020_Sat_eval

Document for:	Discussion and Decision

# 1	Introduction

In [1], a new SID was approved on self-evaluation towards the IMT-2020 submission of the 3GPP Satellite radio interface technology.

Detailed objectives of this study item include:

a)	Complete all required submission templates as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc]

b)	Provide self-evaluation results against technical performance requirements for eMBB-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Peak data rate

•	Peak spectral efficiency

•	User experienced data rate

•	5th percentile user spectral efficiency

•	Average spectral efficiency

•	Area traffic capacity

•	Latency, including user plane latency and control plane latency

•	Energy efficiency, including both network and device

•	Mobility

•	Mobility interruption time

c)	Provide self-evaluation results against technical performance requirements for mMTC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Connection density

d)	Provide self-evaluation results against technical performance requirements for HRC-s as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2], including

•	Reliability

e)	Provide self-evaluation results for other requirements (including bandwidth) as defined in Report ITU-R M.2514 [RAN ITU-R Ad-Hoc, RAN1, RAN2, RAN4]

In RAN2#122 the following was agreed [2]:

Agreements:

- RAN2 will perform the evaluations of user plane latency, control plane latency, and mobility interruption time.

- Evaluate the control plane latency from RRC_INACTIVE to RRC_CONNECTED.

- Evaluate the control plane latency based on the 2-step RACH.

- For user plane latency evaluation, HARQ disabling should be assumed.

Agreements from Friday CB session:

- Evaluate the mobility interruption in beam mobility

- Confirm 0ms mobility interruption time is achieved by NR in beam mobility

- For RTD we consider the same scenario as considered by RAN1

Whereas RAN1 has agreed on the following assumptions [3]:

Proposal 1.1: The following working assumption is confirmed:

For peak spectral efficiency and peak data rate parameters:

- The parameters are chosen based on “ideal conditions”: 90degree elevation angle, 0dB atmospheric loss, 0dB shadow fading margin, 0dB scintillation loss, 0dB polarization loss, 0dB additional losses.

- Companies to provide in RAN1#113 realistic parameters, declaring the assumptions and evaluations leading to those parameters.

In this contribution we discuss some of the general assumptions and the CP and UP latencies.

# 2	Discussion on general assumptions

Within RAN2#122 submitted papers there were proposals not to consider the service and feeder link delays in the evaluation of the CP and UP latencies. Without these links, NTN is reduced to a TN and the existing IMT-2020 values from [4] can be reused. As the latencies introduced by the feeder and service links in NTN can be considerably large, we believe it is crucial that these links are considered.

Observation 1: Feeder and service link delays in NTN have an impact on user and control plane latencies which will be reflected in the system performance.

Proposal 1: Feeder and service link delays need to be taken into account in the user and control plane latency analysis for NTN.

As a starting point, LEO at 600 km can be considered with ideal conditions: 90 degree elevation angle, while also the GW/gNB can be assumed to be at nadir.

Proposal 2: For the feeder and service link delay consider LEO at 600 km with both UE and GW/gNB at nadir.

# 3	Control plane latency

The CP latency according to [5]:

Control plane latency refers to the transition time from a most “battery efficient” state (e.g. Idle state) to the start of continuous data transfer (e.g. Active state). Control plane latency should be equal to or less than 40 ms. This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the control plane latency. Table 2 in § 7.2.5 of Report ITU-R M.2412 provides an example of the elements in the calculation of the control plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 1.15 s, and may operate with a range of relevant satellite orbits.

The referred example Table in [6] looks as follows:

In order to satisfy the requirements, we can assume the system is set such that there are no HARQ retransmissions needed, as line of sight can be assumed.

Proposal 3: For the CP latency no HARQ retransmissions are assumed.

The message exchange relevant for NTN CP latency analysis is shown in Figure 1 [7]

Figure 1: Control Plane procedure latency elements

A number of companies proposed to set the service and feeder link latency to zero, but we believe that makes the whole purpose of this submission obsolete, so we believe the feeder link and service link need to be included in latency analysis, according to the discussion in the previous section.

Proposal 4: service and feeder link latencies need to be taken into account in the control plane evaluations.

In order to satisfy the requirements, the most optimistic (but still realistic) values can be taken into account and we tried to propose those in the table below.

| Step | Description | CP Latency for UL data transfer [ms] |
| --- | --- | --- |
| 1 | Delay due to RACH scheduling period | [0] (taken from [4]) |
| 2 | Transmission of RACH Preamble | Length of the preamble according to the PRACH format as specified in [6 38.211] + satellite processing + service delay+ feeder link delay |
| 3 | Preamble detection and processing in gNB | Tproc,2 (taken from [4]) |
| 4 | Transmission of RA response | Ts + satellite processing + service delay+ feeder link delay |
| 5 | UE Processing Delay (decoding of scheduling grant, timing alignment and C-RNTI assignment + L1 encoding of RRC Connection Resume Request) | NT,1+NT,2+0.5 ms [37.910] |
| 6 | Transmission of RRC Resume Request | Ts + satellite processing + service delay+ feeder link delay |
| 7 | Processing delay in gNB (L2 and RRC) | [3] (taken from [4]) |
| 8 | Transmission of RRC Resume | Ts + satellite processing + service delay+ feeder link delay |
| 9 | Processing delay in UE of RRC Resume including grant reception | [7] (taken from [4]) |
| 10 | Transmission of RRC Resume Complete and UP data | 0 (taken from [4]) |
|  | Total delay [ms] |  |
| NOTE 1: Processing times are set to 0 ms for the most optimistic view. |  |  |

Table 2: Control plane procedure latency

Observation 2: The control latency analysis can be based on the analysis in 37.910 where the feeder and service link latencies should added.

In 37.910 values are given for different PRACH lengths, different resource mappings, UE capabilities and different SCS. For NTN FR1 only 15 and 30 kHz are defined, so the 60 kHz and 120 kHz can be skipped.

Proposal 5: RAN2 to focus on FR1 and SCS 15 and 30 kHz and discuss which of the cases in  TR 37.910 to select and extend with the feeder and service link latencies.

# 4	User plane latency

The UP latency according to [3]:

User plane latency is the contribution of the radio network to the time from when the source sends a packet to when the destination receives it (in ms). It is defined as the one-way time it takes to successfully deliver an application layer packet/message from the radio protocol layer 2/3 SDU ingress point to the radio protocol layer 2/3 SDU egress point of the radio interface in either uplink or downlink in the network for a given service in unloaded conditions, assuming the mobile station is in the active state. User plane latency should be equal to or less than 10 ms, for both downlink and uplink, assuming unloaded conditions (i.e. a single user) and small IP packets (e.g. 0 byte payload + IP header). This requirement is defined for the eMBB-s usage scenario and the above target applies to handheld devices. The proponent should provide the elements and their values in the calculation of the user plane latency, for both UL and DL. Table 3 in § 7.2.6 of Report ITU-R M.2412 provides an example of the elements in the calculation of the user plane latency that should be followed with needed adaptions to e.g. include the assumed time-of-flight between the terminal and the base station. In addition to compliance with the above requirement, the proponent may provide indication that the Satellite Radio Interface is able to support larger latencies, e.g. up to 650 ms, and may operate with a range of relevant satellite orbits

The referred example Table in [6] looks like shown below and takes into account the HARQ retransmission.

In TR 37.910 for the user plane latency 0 and 10% retransmission rates were assumed for TN. We suggest similar retransmission rates can be assumed for evaluating NTN user plane latencies.

Proposal 6: For the UP latency 0 and 10% retransmission rates are assumed.

The procedures for UL and DL data transmission can be seen in Figures 2a and 2b [7].

Figure 2a: Downlink (DL) user plane (UP) data transfer procedure latency

Figure 2b: Uplink (UL) UP data transfer procedure latency

Also for the user plane latencies the feeder link and service link need to be included, according to the discussion in the previous section.

Proposal 7: Service and feeder link latencies need to be taken into account for the User plane latency evaluation.

[4] contains the results for a large number of cases for TN which can be adjusted with the service and feeder link. Table 7.5.1.1.1-2 contains the DL latencies for FDD cases while Table 7.5.1.1.2-2 contains the values for the uplink cases for FDD.

Proposal 8 RAN2 to decide which of the cases in 37.910 Tables 7.5.1.1.1-2 and 7.5.1.1.2-2 to use and adjust for feeder and service link delays.

# 5	Conclusion

This document has made the following observations and proposals:

Observation 1: Feeder and service link delays in NTN have an impact on user and control plane latencies which will be reflected in the system performance.

Proposal 1: Feeder and service link delays need to be taken into account in the user and control plane latency analysis for NTN.

Proposal 2: For the feeder and service link delay consider LEO at 600 km with both UE and GW/gNB at nadir.

Proposal 3: For the CP latency no HARQ retransmissions are assumed.

Proposal 4: service and feeder link latencies need to be taken into account in the control plane evaluations.

Observation 2: The control latency analysis can be based on the analysis in 37.910 where the feeder and service link latencies should added.

Proposal 5: RAN2 to focus on FR1 and SCS 15 and 30 kHz and discuss which of the cases in TR 37.910 to select and extend with the feeder and service link latencies.

Proposal 6: For the UP latency 0 and 10% retransmission rates are assumed.

Proposal 7: Service and feeder link latencies need to be taken into account for the User plane latency evaluation.

Proposal 8 RAN2 to decide which of the cases in 37.910 Tables 7.5.1.1.1-2 and 7.5.1.1.2-2 to use and adjust for feeder and service link delays.

# References

- RP-230736 – “New SID: Study on Self-Evaluation towards the 3GPP submission of an IMT-2020 Satellite Radio” Interface Technology, Ericsson, Qualcomm, Thales, MediaTek Inc.

- Report of 3GPP TSG RAN WG2 meeting #122, Incheon, Korea.

- R1-230603, “Feature lead summary #1 on evaluation methodology for IMT-2020 Satellite”, 3GPP TSG-RAN WG1 Meeting #113.

- 3GPP TR37.910, “Study on self evaluation towards IMT-2020 submission”, Rel 17, 2022.

- Report ITU-R M.2514 (09/2022) - Vision, requirements and evaluation guidelines for satellite radio interface(s) of IMT-2020 – PDF, Word

- Report ITU-R M.2412-0 (10/2017) -Guidelines for evaluation of radio interface technologies for IMT-2020.

- R2-2305410, “Discussion on IMT-2020 Satellite self-evaluation for Latency and Mobility”, Thales, May 2023.