3GPP TSG-RAN WG2 Meeting #123					 			 		  R2-2307591

Toulouse, France, August 21-25, 2023                     Revision of R2-2305960

Agenda item:		7.22.2

Source: 			ZTE Corporation, Sanechips

Title: 	RAN2 impacts of LP-WUS in idle or inactive mode

Document for: 	Discussion and Decision

- Introduction

In RAN#97, a study on low-power Wake-up Signal and Receiver for NR has been agreed in SID [1]. And the objective about LP-WUS is the following:

The study item includes the following objectives: - Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] - Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this paper, based on the RAN1 progress, we will further discuss the topics which need RAN2 decision and the corresponding RAN2 impacts for using LP-WUS in idle/inactive mode.

- Discussion

For idle/inactive mode, in RAN1# 110b meeting, it has been agreed to introduce ultra-deep sleep mode as below:

Agreement Take the following power model for main radio for evaluation in LP-WUS/WUR SI, - For IoT and wearable cases, reuse TR38.875 power model as baseline. - For eMBB and other cases, reuse TR38.840 power model as baseline. - Introduce ‘Ultra-deep sleep’ power state for main radio of UEs with LP-WUS receiver - FFS: The details of ‘Ultra-deep sleep’ power state

And in RAN2#121bise meeting, the understanding about the Ultra-deep sleep mode has reached an agreement as below:

=>Ultra-deep-sleep = R2 understands for now that this is a power saving state (introduced by R1) to denote a state when the Main Receiver (MR) may sleep/turn off.

Shortly to say, in idle/inactive mode, UE could enter into ultra-deep sleep mode for power saving. During the ultra-deep sleep mode, UE may turn off Main radio (MR), and monitor LP-WUS with LP-WUR (LR).

2.1 Activation/Deactivation procedures of LP-WUS

In RAN2#122 meeting, some agreements about the Activation/Deactivation procedures of LP-WUS were reached as below:

- RAN2 expect that different coverage LR/MR may have RAN2 impact, e.g. UE need to stop using LP WUS when moving out of LR coverage, other aspects FFS. What to cover (if anything) in TS 38.304 is FFS. - For UE in RRC_IDLE/RRC_INACTIVE state, it is FFS to what extent the network is or need to be aware of which receiver the UE uses MR/LR or both (for paging reception etc). A potential drawback of not knowing could be increased LP WUS load, a potential drawback of awareness is increased signalling. - RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. (Other control methods not precluded)

RAN1 is discussing the condition of LP-WUS in recent meetings. For example, if the channel environment for a UE dynamically changes, e.g., UE may be in an accelerated movement, it may cause the LP-WUS no longer to be available. In RAN2, in order to figure out the entry/exit conditions for using LP-WUS, RAN2 needs to consider the conditions of using LP-WUS for a LP-WUS capable UE based on RAN1’ outputs.

From network perspective, in order to ensure the alignment between gNB and UE, gNB should have some control on the utilization of LP-WUS. On one hand, only if gNB activates LP-WUS for a LP-WUS capable UE, UE’ MR is allowed to enter into ultra-deep sleep mode and UE’LR could monitor the corresponding LP-WUS. On the other hand, if gNB deactivates LP-WUS for a LP-WUS capable UE, UE’ MR cannot enter into ultra-deep sleep mode.

Proposal 1: On one hand, only if gNB activates LP-WUS for a LP-WUS capable UE, UE’ MR is allowed to enter into ultra-deep sleep mode and UE’LR could monitor the corresponding LP-WUS. On the other hand, if gNB deactivates LP-WUS for a LP-WUS capable UE, UE’ MR cannot enter into ultra-deep sleep mode.

In general, there are two ways to activate or deactivate a LP-WUS for a LP-WUS capable UE in idle or inactive mode:

- Alt1: gNB could indicate a LP-WUS capable UE to enter into ultra-deep sleep mode and activate LP-WUS via RRC release message. Upon reception of this message, the UE’ MR could enter into ultra-deep sleep mode immediately and UE’ LR starts to monitor the common or grouped or dedicated LP-WUS. And gNB could transmit the corresponding LP-WUS to wake up the UE which has been activated for monitoring LP-WUS.

- Alt2: gNB could activate LP-WUS via SIB. If SIB broadcasts the configuration of LP-WUS, a LP-WUS capable UE’ MR always enters into ultra-deep sleep mode and UE’ LR monitors the common or grouped LP-WUS. And gNB always transmits LP-WUS to wake up any LP-WUS capable UE.

In RAN2, both of two ways could be considered.

Proposal 2: RAN2 needs to consider activating or deactivating LP-WUS for a LP-WUS capable UE via RRC dedicated message or SIB.

In last RAN2 meeting, an agreement has been achieved that RAN2 assumes that UE uses LP WUS when pre-configured condition(s) are fulfilled. This can be helpful to avoid that UE keep using LP-WUS even when conditions become unsuitable and then avoid the loss of paging. According to this agreement, we understand, when activating the LP-WUS for the LP-WUS capable UE(s), some pre-configured condition(s) can also be provided or the pre-configured condition(s) can be used as kind of implicit activating indication.

However, for UE in idle or inactive mode, because gNB couldn’t obtain the information about the change of channel condition in UE, gNB cannot exactly know whether the condition for monitoring LP-WUS is satisfied in UE side. If gNB can’t know whether the condition of LP-WUS is satisfied, it couldn’t know whether to transmit LP-WUS to the UE. Moreover, if UE’MR enters into the ultra-deep sleep mode and woke up by LP-WUS, it requires at least a ramp up time (400 ms or 500 ms) for UE’ MR to turn on and receive paging message then. Since gNB also can’t know when UE is able to receive paging message, it couldn’t determine when to transmit paging message.

One solution is to allow UE inform gNB whether UE satisfied the condition of LP-WUS. But UE has to set up RRC connection frequently which will increase the UE power consumption and so is undesired. So we suggest not to pursue this solution.

Proposal 3: There is no need for UE in idle/inactive to inform gNB whether UE satisfies the condition of using LP-WUS in idle/inactive mode.

Another approach to address this issue is that, after the LP-WUS is activated by either RRC or SIB for a LP-WUS capable UE, gNB always transmits LP-WUS when it needs to wake up the UE and further transmits the paging message at a fixed time point after LP-WUS. By doing so, if UE’MR enters into the ultra-deep sleep mode, UE can detect the LP-WUS and wake up accordingly. But if UE’MR exits from the ultra-deep sleep mode, UE can directly monitor paging message at a fixed time point after LP-WUS. This fixed time point can be determined by the ramp-up time and the time for synchronization.

Proposal 4: If the LP-WUS is activated by RRC or SIB for a LP-WUS capable UE, gNB always transmits LP-WUS when it needs to wake up the UE. And gNB also transmits the paging message at a fixed time point after LP-WUS.

- 2.2 RRM measurement

In idle/inactive mode, UE needs to perform serving cell measurement and neighbor cell measurement. According to TS 38.133, clause 4.2.2, UE shall wake up MR to measure SSB of serving cell at least once every M1*N1 DRX cycles. Moreover, if the condition of triggering neighbor cell measurement is satisfied according to TS 38.304, UE shall wake up MR to measure SSB of neighbor cell at least every Tmeasure,NR. It can be seen that the serving cell and neighbor cell measurement contributes a lot to power consumption of UE in idle/inactive mode.

In RAN1, there is a common understanding that RRM measurements performed by MR would jeopardize power saving benefit from LP-WUS. Hence, in order to keep power saving benefit from LP-WUS, a new serving cell and/or neighbor cell(s) measurement scheme in idle/inactive mode that requires lower power consumption would be beneficial. Considering the low power of LP-WUS, RAN1 is discussing RRM measurements performed by LP-WUR instead of RRM measurements performed by SSB.

In RAN1#113 meetings, some agreements related to RRM measurement are reached as below:

Agreement - For Idle/Inactive mode, study offloading of RRM measurements of serving cell to LP-WUR under certain conditions, if any, and relaxation of serving/neighboring cell RRM measurements in MR considering - Periodic reference signal(s) is/are used for LR measurements. - FFS: reference signal(s) to measure, e.g. PSS/SSS/PBCH DMRS, LP-WUS waveform sequence, LP-SS - FFS: periodicity, content - MR performs measurements - Alt2: with relaxed periodicity if RRM measurement in MR is relaxed. - FFS: Condition for relaxation if any - Can apply for both neighboring and serving cell - Alt3: only when reference signal(s) based measurements by LP-WUR satisfy certain condition(s), e.g. are below threshold. - FFS threshold. - Above MR measurement under certain conditions can apply for both neighboring and serving cell - Potentially with relaxation methods for MR neighboring cell measurement - Other alternatives are not precluded - FFS: Feasibility of RRM measurements of neighbor cells by LP-WUR

As above, some relaxed serving cell RRM measurement is under discussion. From RAN2 perspective, RAN2 needs to consider the potential impacts, e.g., RAN2 needs to study the condition of the relaxed serving cell RRM measurement performed by LP-WUR.

Proposal 5: RAN2 needs to consider the condition of serving cell RRM measurement if it’s performed by LP-WUR in idle/inactive mode.

Moreover, RRM measurements performed by LP-WUR can be performed only if the condition of using LP-WUS is satisfied. Hence, the condition for using LP-WUS also directly affects the condition of RRM measurements performed by LP-WUR.

In addition, according to TS 38.304, if the condition of neighbor cell measurement, e.g. Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, is satisfied, UE starts the neighbor cell measurement performed by SSB. If the condition of the relaxed neighbor cell measurement, e.g. (SrxlevRef – Srxlev) < SSearchDeltaP, is satisfied, UE starts the relaxed neighbor cell measurement performed by SSB. If UE starts to measure the SSB in neighbor cell, UE will turn on MR, and it is natural for UE to also measure SSB in the serving cell (that may means serving cell RRM measurements performed by LP-WUR can be disabled). Hence, it can be deduced that the condition of neighbor cell measurement and the condition of relaxed neighbor cell measurement may have some relationship with the condition of serving cell RRM measurements performed by LP-WUR. RAN2 may need to jointly consider them.

Proposal 6: RAN2 needs to consider the relationship between serving cell RRM measurements performed by LP-WUR and the (relaxed) neighbor cell measurement performed by SSB.

Furthermore, in RAN1#113 meetings, some agreements related to metrics of RRM serving cell measurement performed by LP-WUR are reached as below:

Agreement - For at least RRM serving cell measurement performed by LP-WUR based on reference signal(s), RAN1 identified at least the following metrics for further study and evaluation (including feasibility, complexity, power consumption, etc) - LP-RSSI or Energy detection: linear average of total received power over a RSSI resource. - FFS RSSI resource. - LP-RSRP: linear average of received power of resource of reference signal(s) or signal(s) parts. - FFS resource of reference signal(s) or signal(s) parts - LP-SINR = LP-RSRP/(power of interference and noise) - FFS how to define “power of interference and noise” - LP-RSRQ= [N x] LP-RSRP/LP-RSSI, where N is the factor of resource size difference for evaluation LP-RSRP and LP-RSSI. - Accounting AGC accuracy, ADC of at least 4 bits is required. - Note: Reference signal for performing measurements can be e.g. SSB (PSS/SSS/PBCH DMRS), LP-WUS-waveform sequence, LP-SS - Note: The definition of metrics could be further refined based on future study

We can see some new metrics for measurement performed by LP-WUR are being evaluated in RAN1. If those measurement metrics could replace the RSRP or RSRQ based on SSB, RAN2 could further consider the impacts on the criteria for cell reselection.

Proposal 7: RAN2 needs to consider the impacts on the criteria for cell reselection if RRM measurements is performed by LP-WUR.

- 2.3 Paging mechanism

In current TS 38.304, UE monitors one PEI occasion per DRX cycle in IDLE/INACTIVE mode. PEI could indicate the available POs and subgroups of a paging occasion. Based on PEI, UE could determine whether to monitor the coming PO for paging. If LP-WUS is introduced, LP-WUS is transmitted to wake up UE from the ultra-deep sleep mode before PO. Hence, the UE behavior after LP-WUS should be discussed.

After the discussion in RAN2#121bise and 122 meeting, some agreements have been reached as below:

- In scope: Use LPWUS with Idle / Inactive UE camping with reception of paging and other necessary transmissions (from serving cell), reusing if possible/reasonable concepts from earlier releases, where the LPWUS either wakes the UE to receive by MR, or it conveys information by itself, or both. - RAN2 assumes that using subgrouping for LP-WUS could be beneficial to reduce false alarms rate (depend on L1 capacity to carry payload).

As above, the possible UE behavior after UE wakes up could be summarized as below:

- Option 1: perform PEI monitoring, and afterwards follow legacy procedures

- Option 2: perform PO monitoring, and afterwards follow legacy procedures

- Option 3: transmit PRACH for initial access, and follow legacy procedures

In order to clarify these options clearly, the corresponding procedures are illustrated in the following Fig.1.

Fig.1

Generally, the paging mechanism with LP-WUS could takes into account three key factors: access latency, false wake up and the complexity of design of LP-WUS. And the design of LP-WUS is mainly within the scope of RAN1. From RAN2 side, the extended access latency by LP-WUS is more severe than legacy paging mechanism. And the extended access latency will increase the access failure. Therefore, RAN2 could consider how to decrease the access latency in a paging mechanism with LP-WUS.

For option 1, its access latency is largest, but its possibility of the false wake up is relatively low. If UE detects LP-WUS, UE’MR will wake up, obtain synchronization, monitor PEI, and determine whether to monitor the coming PO based on PEI. The procedure of monitoring PEI would increase the access latency, but decrease the false wake up. However, UE may miss its PEI in the nearest DRX cycle after UE wakes up, as illustrated in Fig.1. In this case, UE has to monitor the PEI in the next DRX cycle. Hence, it’s highly possible the UE needs to wait for some additional time after the ramp up time. This is the reason why longer paging latency is caused.

For option 2, its access latency is medium, but its possibility of the false wake up is relatively high. If UE detects LP-WUS, UE’MR will wake up, obtain synchronization, and monitor the coming PO. No need to monitor PEI would decrease the access latency. However, similar as Option 1, UE may miss its PO in the nearest DRX cycle after UE wakes up and has to monitor the PO in the next DRX cycle, as illustrated in Fig.1. Hence, it’s also highly possible the UE needs to wait for some additional time after the ramp up time, which will cause additional paging latency. In addition, in order to decrease the possibility of the false wake up, LP-WUS is required to carry some information to indicate a condition that UEs in a specific PO or group need to wake up. In this way, UE that matches with the condition will wake up, obtain synchronization and monitor its own PO. But the information carried in LP-WUS increases the complexity of LP-WUS.

For option 3, its latency is shortest, but it requires LP-WUS to wake up a specific UE. In general, the UE specific LP-WUS is allocated by gNB in connected mode. And UE monitors its LP-WUS during ultra-deep sleep mode. If UE detects its LP-WUS, UE will wake up and initiate a RACH directly. No need to monitor PEI and PO would decrease the access latency, so the paging latency in Option 3 is mainly caused by the ramp up time. However, in idle/inactive mode, UE may move out of the serving cell, UE specific LP-WUS that allocated in serving cell may became invalid. Therefore, RAN2 needs to consider these issues if UE specific LP-WUS in idle/inactive mode would be supported.

Proposal 8: RAN2 needs to make choice among the following options for paging mechanism with LP-WUS, e.g., the UE behavior after UE wakes up by a LP-WUS, with consideration on how to decrease the paging/access latency and also reduce the false wake up:

- Option 1: perform PEI monitoring, and afterwards follow legacy procedures

- Option 2: perform PO monitoring, and afterwards follow legacy procedures

- Option 3: transmit PRACH for initial access, and follow legacy procedures

- 2.4 UE capability

UE capability about LP-WUS is essential for gNB. Initially, gNB needs to activate LP-WUS for a UE only if the UE supports it. Moreover, when gNB pages a UE, it needs to firstly transmit LP-WUS to wake up the UE who supports LP-WUS and has LP-WUS activated. However, gNB couldn’t store UE context of a UE in idle mode. gNB needs to obtain UE capability about LP-WUS when it pages the UE, e.g., from the core network.

Proposal 9: RAN2 needs to consider the UE capability signaling for LP-WUS.

- Conclusion and proposals

Based on the analysis in previous sections, the following observations and proposals are given:

Proposal 1: On one hand, only if gNB activates LP-WUS for a LP-WUS capable UE, UE’ MR is allowed to enter into ultra-deep sleep mode and UE’LR could monitor the corresponding LP-WUS. On the other hand, if gNB deactivates LP-WUS for a LP-WUS capable UE, UE’ MR cannot enter into ultra-deep sleep mode.

Proposal 2: RAN2 needs to consider activating or deactivating LP-WUS for a LP-WUS capable UE via RRC dedicated message or SIB.

Proposal 3: There is no need for UE in idle/inactive to inform gNB whether UE satisfies the condition of using LP-WUS in idle/inactive mode.

Proposal 4: If the LP-WUS is activated by RRC or SIB for a LP-WUS capable UE, gNB always transmits LP-WUS when it needs to wake up the UE. And gNB also transmits the paging message at a fixed time point after LP-WUS.

Proposal 5: RAN2 needs to consider the condition of serving cell RRM measurement if it’s performed by LP-WUR in idle/inactive mode.

Proposal 6: RAN2 needs to consider the relationship between serving cell RRM measurements performed by LP-WUR and the (relaxed) neighbor cell measurement performed by SSB.

Proposal 7: RAN2 needs to consider the impacts on the criteria for cell reselection if RRM measurements is performed by LP-WUR.

Proposal 8: RAN2 needs to make choice among the following options for paging mechanism with LP-WUS, e.g., the UE behavior after UE wakes up by a LP-WUS, with consideration on how to decrease the paging/access latency and also reduce the false wake up:

- Option 1: perform PEI monitoring, and afterwards follow legacy procedures

- Option 2: perform PO monitoring, and afterwards follow legacy procedures

- Option 3: transmit PRACH for initial access, and follow legacy procedures

Proposal 9: RAN2 needs to consider the UE capability signaling for LP-WUS.

- References

- RP-222644, Revised SID: Study on low-power Wake-up Signal and Receiver for NR, RAN#97e, September, 2022