3GPP TSG RAN WG2 #123                                                                         R2-2307595

Toulouse, France, August 21-25, 2023

Agenda Item:	7.19.2

Source:	ZTE Corporation, Sanechips

Title:	Remaining issues of enhanced eDRX in RRC_INACTIVE

Document for:	Discussion and Decision

Introduction

For R18 eRedCap WID, long eDRX supporting in RRC_INACTIVE has been discussed for several meetings and the agreements for most issues been made. But there is still the following remaining issue from RAN2#121bise meeting:

If this is even a valid case (we will decide later): In an overlapped PH: Within CN PTW and outside RAN PTW, T = min {CN configured DRX cycle, default paging cycle broadcast in system information}.

In this contribution, we will discuss this issue and give our proposals.

Discussions

Considering that the Rel-18 RRC_INACTIVE extended eDRX cycle is generally less than or equal to the RRC_IDLE eDRX cycle, the CN PTW may include one or several RAN eDRX cycle as shown in the following figure (e.g. one CN PTW may include two RAN PTWs and two “outside RAN PTW duration”). Thus, the case of “Within CN PTW and outside RAN PTW” is valid.

Figure 1. The case “Within CN PTW and outside RAN PTW”

Observation 1: Based on the eDRX cycle value range, the case of “Within CN PTW and outside RAN PTW” is valid.

Furthermore, it has been agreed that “RAN PTW length is mandatorily present within Rel-18 INACTIVE eDRX’s configuration.” (Agreement from RAN2#121bise). That means, the RAN PTW length may be different from the CN PTW length. We also see no any technical reason to restrict that the RAN PTW length must be longer than or equal to the CN PTW length.

Observation 2: There is no reason to restrict that the RAN PTW length must be longer than or equal to the CN PTW length.

Proposal 1: RAN2 confirm that the case of “Within CN PTW and outside RAN PTW” is valid.

About “overlapped PH” definition, some clarification is necessary. The description of PH in the TS 38.304 running CR are as following highlight yellow texts [1]:

…………….PH, PTW_start and PTW_end are given by the following formula: The PH for CN is the H-SFN satisfying the following equations: H-SFN mod TeDRX_CN= (UE_ID_H mod TeDRX_CN), where - TeDRX_CN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_CN = 2, …, 1024 Hyper-frames) configured by upper layers. The PH for RAN is the H-SFN satisfying the following equations: H-SFN mod TeDRX_RAN= (UE_ID_H mod TeDRX_RAN), where - TeDRX_RAN: UE-specific eDRX cycle in Hyper-frames, (TeDRX_RAN = 2, …, 1024 Hyper-frames) configured by RRC. For CN configured PTW: PTW_start denotes the first radio frame of the PH for CN that is part of the PTW and has SFN satisfying the following equation: SFN = 128 * ieDRX_CN, where - ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8 PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation: SFN = (PTW_start + L*100 - 1) mod 1024, where - L = Paging Time Window (PTW) length (in seconds) configured by upper layers For RAN configured PTW: PTW_start denotes the first radio frame of the PH for RAN that is part of the PTW and has SFN satisfying the following equation: SFN = 128 * ieDRX_CN, where - ieDRX_CN = floor(UE_ID_H /TeDRX_CN) mod 8 PTW_end is the last radio frame of the PTW and has SFN satisfying the following equation: SFN = (PTW_start + L*100 - 1) mod 1024, where - L = Paging Time Window (PTW) length (in seconds) configured by RRC UE_ID_H is defined as follows: UE_ID_H : 13 most significant bits of the Hashed ID. Hashed ID is defined as follows: ……………………………………………

Based on the TS 38.304 specification, the PH means the H-SFN that contains the PTW_start, it only part of the PTW, NOT all the paging Hype frame that includes PTW. Thus, if PTW spans more than one H-SFN, the “overlapped PH” may cause misunderstanding.

For example, in above figure 1, based on the current TS 38.304, hf2 are hf4 are not overlapped PH, but in which UE should also monitor CN paging by using T = min {CN configured DRX cycle, default paging cycle broadcast in system information} for the calculation of PF.

Observation 3: Based on the TS 38.304, the PH only means the paging H-SFN that contains the PTW_start, NOT includes all the paging Hype frame. If PTW spans more than one H-SFN, the “overlapped PH” description will cause misunderstanding.

In fact, whether in overlapped PH or non-overlapped PH does not matter for the T calculation, we only need to care about whether it’s inside PTW or outside PTW.

Proposal 2: RAN2 does not need to describe “overlapped PH” and “non-overlapped PH” in MAC specification. To only describe “inside PTW” and/or “outside PTW” is enough.

Conclusions

This contribution presents some of our views on the remaining issue of eDRX enhancements for Rel-18 eRedCap. The following proposals are given:

Proposal 1: RAN2 confirm that the case of “Within CN PTW and outside RAN PTW” is valid.

Proposal 2: RAN2 does not need to describe “overlapped PH” and “non-overlapped PH” in MAC specification. To only describe “inside PTW” and/or “outside PTW” is enough.

References

[1] R2-2306734, 38304_(Rel-18)_Running 38.304 CR for eRedCap. RAN2#122, June 2023