3GPP TSG RAN WG2 Meeting #123                	R2-2307601

Toulouse, France, August 21-25, 2023

Agenda item:	7.7.3

Source:	Samsung

Title:	UE support of Network Verified UE Location Feature

WID/SID:	NR_NTN_enh-Core

Document for:	Discussion and Decision

# Introduction

In RAN2#122 meeting, RAN2 briefly discussed whether the new feature of UE location verification would require defining additional UE behavior. RAN2 agreed the following [1]:

Working Assumption: - A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network

In this contribution, we further discuss one of the main remaining open point on location verification:

- Handling of an NTN UE that does not support location verification.

# Discussion

## Handling of NTN UE that does not support location verification:

According to the revised WID on NR NTN (Non-Terrestrial Networks) enhancements, the following are the detailed objectives for Network verified UE location [2]:

4.1.3 Network verified UE location Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied. […] Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved Note 6 : The enhancements should take into account the mirror-image ambiguity Note 7 : Network verified UE location is an optional UE feature

Observation 1: RAN1 will work on multi-RTT for performing verification of the UE location.

Observation 2: the new feature of location verification is an optional UE feature.

In RAN2#122 meeting, RAN2 briefly discussed the issue of how to handle UEs that may or may not support the new feature of network verified UE location in NR NTN. The idea is that a certain UE behavior needs to be considered in this case. Eventually, RAN2 agreed the need to introduce a new UE capability to support this new feature [1]:

Working Assumption: - A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network

The Multi-RTT method that is selected to support the network verified UE location in NR NTN is an optional method in NR, introduced in Rel-16 that is not mandatory for a Rel-17 NR NTN UE.

However, considering the RAN2 conclusion on the need for a new Rel-18 UE capability to support the new feature of network verified UE location in NR NTN, our understanding is that a Rel-18 NTN UE needs to support the Multi-RTT method before accessing the NTN cell.

Observation 3: A Rel-18 UE should support multi-RTT as part of its NR NTN capability.

Proposal 1: RAN2 to change the WA on “A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network” to a full agreement.

Coming back to the open point on how the network would handle NTN UEs that do not support the new feature of network verified UE location in NR NTN. In our view, for example, there could be three options:

- Option 1 (no UE access to all services): the network should not allow the UE to access all services. This is reasonable considering the understanding in SA2 [3] and RAN3 [4] that allowing the UE to obtain any services before verifying the UE’s reported location, may violate regulatory requirements, if the UE manages to obtain a service at a location in which the UE is not allowed any services.

LS from SA2 [3] Q1 Is there any constraint on the latency (from trigger to result) of the verification procedure? Answer: In Release 17 and 18, …. However, a long period of location verification is not preferred because it could interfere with power saving for UEs which need to access a PLMN for only very short periods, and would allow a UE that was not at an allowed location to obtain service from the PLMN that might violate regulatory requirements. Hence, SA2 requests that location verification be capable of being completed within a period of approximately 1 minute maximum and 30 seconds preferably.

RAN3#119bis-e [4]: Moderator’s Summary : Most companies agree that if UE location verification is decided and initiated by 5GC, no services can be provided to the NTN UE until its location has been verified at initial network attach. Also, many other services if not all, e.g, mobility, etc, should also not be provided to the NTN UE until its location has been verified. ...

- Option 2 (UE access to limited services): the network may allow the UE access to limited services.

- Option 3 (UE access to all services with no restrictions): the network may allow the UE to access all services (without limitations) then perform verification of the reported UE location. This option may raise concern regarding a malicious UE gaining access to the network (and services) before the verification of the UE location. For example, the malicious UE may gain access to services and complete transmission session then leaves the network before the network have the chance to complete the verification [5]. This way the malicious UE may repeat this behavior and avoid location verification.

Proposal 2: RAN2 to discuss how the network handles the access to NR NTN cells for UEs that do not support the new Rel-18 NR NTN capability to support the new feature of network verified UE location.

# Conclusions

In this contribution, we discussed the remaining issue on network verified UE location in NTN:

- Handling of an NTN UE that does not support location verification.

The following are the observations and proposals in this contribution:

Observation 1: RAN1 will work on multi-RTT for performing verification of the UE location.

Observation 2: the new feature of location verification is an optional UE feature.

Observation 3: A Rel-18 UE should support multi-RTT as part of its NR NTN capability.

Proposal 1: RAN2 to change the WA on “A Rel-18 UE capability is needed for indicating whether UE supports the feature of network verified UE location in NR NTN network” to a full agreement.

Proposal 2: RAN2 to discuss how the network handles the access to NR NTN cells for UEs that do not support the new Rel-18 NR NTN capability to support the new feature of network verified UE location.

# References

- [1] RAN2#122 Chair notes

- [2] RP-223534 Revised WID: NR NTN (Non-Terrestrial Networks) enhancements

- [3] R2-2300062 LS Response on Latency impact for NTN verified UE location (SA2 reply LS).

- [4] R3-231893 Summary of Offline Discussion on CB: # NTN2_LocationVerification

[5] R3-231759 Discussion on network verified UE location, Huawei, Ericsson, CATT

[6] R2-2303962 Discussion on the network verfied UE location, Huawei, HiSilicon, RAN2#121bis-e.

# Annex C (SA2 reply LS [3])

Title: LS Response on Latency impact for NTN verified UE location Response to: S2-2210193 / R2-2211044 Release: Rel-18 Work Item: 5GSAT_ARCH Source: SA2 To: RAN2 Cc: SA1, RAN1, RAN3, RAN Contact person: Stephen Edge sedge AT qti.qualcomm.com Send any reply LS to: 3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org Attachments: # 1 Overall description SA2 thanks RAN2 for their LS on Latency impact for NTN verified UE location. SA2 has the following answers to the 2 questions from RAN2. Q1 Is there any constraint on the latency (from trigger to result) of the verification procedure? Answer: In Release 17 and 18, location verification for regulatory services (e.g. Public Warning System, Charging and Billing, Emergency calls, Lawful Intercept, Data Retention Policy in cross-border scenarios and international regions, Network access) can occur when a UE performs some access to an AMF or MME at a NAS level, such as for initial PLMN Registration or Attach, Registration update or TAU, Service Request, PDU session or PDN connection establishment. The associated NAS procedure is first completed and then the serving AMF or MME can initiate location verification for the UE from an LMF or E-SMLC, respectively. Because the initial NAS procedure is first completed, there is no real time restriction on the latency of the location verification. Hence a latency of more than 10 seconds could be tolerated. However, a long period of location verification is not preferred because it could interfere with power saving for UEs which need to access a PLMN for only very short periods, and would allow a UE that was not at an allowed location to obtain service from the PLMN that might violate regulatory requirements. Hence, SA2 requests that location verification be capable of being completed within a period of approximately 1 minute maximum and 30 seconds preferably. Q2 Can the verification procedure be run independently from the targeted services (e.g. in parallel to prevent any set-up delay)? If not, what is the estimate of set-up delay? Answer: As indicated above, location verification is started after an initiating NAS procedure has been completed and would then run in parallel with any other UE related activity. SA2 is not aware of any constraint at a 5GC level that might impede or delay the location verification once started. # 2 Actions To RAN2 ACTION: SA2 asks RAN2 to take the above answers into account when defining support for UE location verification in Release 18.