3GPP TSG-RAN WG2 Meeting #123                                                                 R2-2307604

Toulouse, France, August May 21 – 25, 2023

Agenda item: 		7.12.2.1

Source: 		Qualcomm Incorporated

Title: 	Enhancements for mobile IAB connected mode mobility

Document for:	Discussion

# 1	Introduction

In the last meeting, RAN2 discussed RACHless and conditional handover for UEs during DU migration. The following agreements were achieved [1]:

Handover Enhancements RAN2 think that to have a fast handover from UE point of view for legacy UEs it is important that the target cell is known to the UE (detected and measured). For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report). for the UL grant and HO completion in RACH-less HO: - 1. Both type-1 configured grant and dynamic grant are supported - 2. FFS handling of supervision timer and when HO is considered successfully complete (expect to align with other WI).

This paper discusses remaining issues for connected-mode mobility during DU migration.

# 2	Discussion

## 2.1	RACH-less handover during DU migration

RAN2 has not yet decided if RACH-less handover should be supported during DU migration. At this state of the WI, RAN2 should conclude whether to support RACH-less handover or not. This decision should be based on the benefits expected from RACH-less handover for this use case and the expected specification effort.

The following benefits can be considered for RACH-less handover:

- Reduced handover failure: Handover failure may happen if handover execution occurs too late. This issue does not apply for UE handover during DU migration since the two logical cells are expected to be collocated, i.e., the source and target channels are the same, and since both cells can be concurrently active without for an extended period of time.

- Reduced interruption time: The interruption time due to the RA procedure is significant. During DU migration, RACH-less handover can therefore provide similar interruption time improvements as for other scenarios, e.g., NTN or LTM, where RACH-less handover is considered.

- Reduced overhead: RACH-less handover can spare the signaling needed for the RA procedure. This may be of significant benefit for DU migration in case a large number of UEs have to be handed over within a small timeframe.

Observation 1: For DU-migration, RACH-less handover can provide significant interruption time reduction, and also significant signalling reduction in case many UEs have to be handed over within a small timeframe.

The specification effort needed for RACH-less HO of mobile IAB can be inferred from the enhancements discussed and agreed for RACH-less HO pursued by other WIs. The specification effort for RACH-less HO of mIAB can further be reduced where enhancements introduced for RACH-less HO by other WIs can be adopted.

Presently, NTN and LTM pursue RACH-less handover. RACH-less HO for DU migration can leverage enhancements introduced for NTN RACH-less HO since both efforts apply to inter-CU handover on L3. LTM is different in that respect as it applies handover to intra CU on L1/L2.

Observation 2: RACH-less handover for DU migration, if supported, may leverage enhancements of NTN RACH-less HO since both handover procedures are conducted on L3 and may occur between different CUs.

The following lists the agreements for NTN RACH-less HO from R2#121, R2#121bis-e, and R2-122, and it highlights those agreements that could be adopted for mobile-IAB RACH-less HO:

R2#121: RACH-less handover agreements for NTN

Agreements:

- Support RACH-less Handover in Rel-18.

- RACH-less Handover in NR NTN is a L3 mobility procedure (FFS if this is combined with the unchanged PCI approach, if supported) and uses the LTE’s RACH-less Handover procedure as a baseline. FFS on TA acquisition

- In NTN RACH-less handover, network indicates (implicitly or explicitly) whether NTA in the target cell is identical to the source cell or explicitly provided by the NW.

- Support dynamic grant from the target cell for RACH-less PUSCH transmission to reduce random access congestion in the target cell. FFS whether to limit the solution to same feeder link/gateway scenario

Comment: This was already agreed for mobile IAB RACH-less HO.

R2#121bise: RACH-less handover agreements for NTN

Agreements:

- In Rel-18 we don’t aim at RACH-less HO for NTN-TN mobility

- For initial UL transmission in RACH-less HO, support pre-allocated grant in RACH-less HO command

Comment: This was already agreed for mobile IAB RACH-less HO.

Agreements via email – from offline 109:

- NTN RACH-less HO is supported for Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB;

- NTN RACH-less HO can be supported for intra-satellite handover with different feeder links, i.e., with gateway/gNB switch, inter-satellite handover with gateway/gNB switch, and inter-satellite handover with same gateway/gNB.

- RAN2 confirms the general UE procedure for NTN RACH-less HO

1.	receive a RACH-less HO command which can include pre-allocated grant optionally. FFS N_TA is optional. (RRC)

2.	start timer T304 for the target cell (RRC)

3.	perform DL and UL synchronization, and start timer T430. FFS how to perform RACH-less UL synchronization to NTN target cell. (RRC, MAC)

4.	start time alignment timer (MAC)

5.	monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

6.	send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

7.	consider RACH-less HO is completed upon receiving NW confirmation. FFS how to confirm RACH-less HO is successfully completed. (RRC, MAC)

8.	stop timer T304 for the target cell. (RRC)

FFS whether to release UL grant if pre-allocated after RACH-less HO completion

FFS RACH-less HO failure handling, e.g. whether UE fallback to RACH-based HO to the target cell

FFS procedure for RACH-less HO combined with PCI unchanged or CHO if supported

4.	The pre-allocated grant is provided as type-1 CG

5.	Send an LS to RAN1 informing RAN2 agreements on NTN RACH-less HO and check RAN1 views on the following aspects:

1. whether the pre-allocated grant is provided with association to SSBs; if so, whether a RSRP threshold is configured for SSB selection.

2. to monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam indication can be provided in RACH-less HO command.

3. power control for initial UL transmission

Comment:

  - This can be adopted for mobile IAB RACH-less HO.

  - This was already agreed for mobile IAB RACH-less HO.

  - These issues also apply to mobile IAB RACH-less HO.

Agreements online:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field. FFS if anything else is needed for dynamic grant

- Consider to support combining RACH-less HO with time-based CHO for NTN, taking into account the 1) validity of pre-allocated grant and potential waste of reserved resource; 2) when/how to provide dynamic grant in PDCCH.

Comment: This can be adopted for mobile IAB RACH-less HO.

R2#122: RACH-less handover agreements for NTN

Agreements:

- In NTN RACH-less handover, NW either indicates NTA in the target cell is identical to the source cell, or the NTA explicitly provided by the NW is 0. RAN2 will not discuss the case where NTA does not equal to 0

Agreements:

- From RAN2 perspective synchronization among source and target cells is not an issue in NTN RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant. FFS any enhancement to the confirmation of RACH-less HO completion, e.g. the NW does not send the UE Contention Resolution Identity MAC CE, and sends PDCCH/PDSCH addressed to C-RNTI

- Remove “FFS how to perform RACH-less UL synchronization to NTN target cell”, RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire NTA (FFS on the spec impact , if any)

Comments:

  - This may not be relevant for mobile IAB RACH-less HO since RAN2 agreed that delta-TA = 0 is assumed.

  - This can be adopted for mobile IAB RACH-less HO.

Observation 3: A large number of agreements for NTN RACH-less HO can be adopted for mIAB RACH-less HO.

The following identifiers RAN2 agreements for mobile-IAB RACH-less HO, which have presently not been considered in the NTN effort but need to be considered on top:

For RACH-less, if supported, there would need to be a beam indication (in RRC HO command), which seems feasible in this release from R2 perspective. R2 assumes that the network can know/select the beam, either from network impl specific knowledge or from UE measurement report (legacy report).

This enhancement is minor and does not require a lot of specification effort.

Observation 4: There are only minor additional enhancements needed for mIAB RACH-less HO on top of NTN RACH-less HO such as including a beam indication in the HO command.

Based on observations 1 to 4, RACH-less handover to be supported for DU-migration.

Proposal 1: RACH-less HO to be supported for DU migration.

Further, based on the above, the following steps for RACH-less HO procedure can be adopted from NTN for mIAB:

Proposal 2: RACH-less HO for mIAB to adopt the following steps of the agreed NTN RACH-less HO procedure:

- 1. Receive a RACH-less HO command which can include pre-allocated grant optionally

- 2. Start time T304 for the target cell (RRC)

- 3. Perform DL and UL synchronization.

- 4. Start time alignment timer (MAC)

- 5. Monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

- 6. Send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

- 7. Consider RACH-less HO is completed upon receiving NW configuration.

- 8. Stop timer T304 for the target cell (RRC).

These steps do not include any of the FFS in the NTN agreements. They further do not include the starting of timer T430, which is specific to NTN..

The following further RAN2 agreements on NTN RACH-less HO can be adopted by mIAB:

Proposal 3: RACH-less HO for mIAB to adopt the following agreements on NTN RACH-less HO procedure:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field.

- From RAN2 perspective synchronization among source and target cells is not an issue in mIAB RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant.

- RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire TA.

Based on Observation 4, there are RAN2 agreements on RACH-less HO for mIAB, which do not (yet) apply to NTN.  To ensure that one comprehensive solution to RACH-less HO is developed for NTN and mIAB, both WIs should handle this topic in a common session.

Proposal 4: NTN and mIAB to have common session on RACH-less handover.

# Conclusion

This paper discussed remaining issues for connected-mode mobility during DU migration. The following observations and proposals have been made:

Observation 1: For DU-migration, RACH-less handover can provide significant interruption time reduction, and also significant signalling reduction in case many UEs have to be handed over within a small timeframe.

Observation 2: RACH-less handover for DU migration, if supported, may leverage enhancements of NTN RACH-less HO since both handover procedures are conducted on L3 and may occur between different CUs.

Observation 3: A large number of agreements for NTN RACH-less HO can be adopted for mIAB RACH-less HO.

Observation 4: There are only minor additional enhancements needed for mIAB RACH-less HO on top of NTN RACH-less HO such as including a beam indication in the HO command.

Proposal 1: RACH-less HO to be supported for DU migration.

Proposal 2: RACH-less HO for mIAB to adopt the following steps of the agreed NTN RACH-less HO procedure:

- 1. Receive a RACH-less HO command which can include pre-allocated grant optionally

- 2. Start time T304 for the target cell (RRC)

- 3. Perform DL and UL synchronization.

- 4. Start time alignment timer (MAC)

- 5. Monitor target cell PDCCH for dynamic grant if pre-allocated grant is not configured in RACH-less HO command (MAC, PHY)

- 6. Send initial UL transmission including RRCReconfigurationComplete message using the available UL grant (RRC, MAC, PHY)

- 7. Consider RACH-less HO is completed upon receiving NW configuration.

- 8. Stop timer T304 for the target cell (RRC).

Proposal 3: RACH-less HO for mIAB to adopt the following agreements on NTN RACH-less HO procedure:

- At least for pre-allocated grant, for the confirmation of RACH-less HO completion we reuse of LTE approach, i.e., UE Contention Resolution Identity MAC CE is used but UE ignores the content of this field.

- From RAN2 perspective synchronization among source and target cells is not an issue in mIAB RACH-less HO

- Release pre-allocated UL grant after RACH-less HO completion

- LTE approach (of confirming the HO completion) is reused for both pre-allocated grant and dynamic grant.

- RAN2 assumes the UL sync handling in the target cell is the same in RACH-based HO and RACH-less HO, except how to acquire TA.

Proposal 4: NTN and mIAB to have common session on RACH-less handover.

# References

- Chair Notes; TSG RAN WG2 Meeting #122, Incheon, Korea, May, 2023