3GPP TSG-RAN WG2 Meeting #123                                                                 R2-2307605

Toulouse, France, August 21 – 25, 2023

Agenda item: 		7.12.2.2

Source: 		Qualcomm Incorporated

Title: 	Enhancements for mobile IAB idle/inactive mode mobility

Document for:	Discussion

# 1	Introduction

In the last meeting, RAN2 achieved the following agreements [1]:

R2 considers that UEs can use the mIAB-cell indication, to prioritize (cell and/or freq) when the UE is camped on the mIAB cell, and FFS to prioritize when the UE is not yet camped on the mIAB cell. FFS if it can be specified the detailed condition for when to apply such prioritization (for either case), RAN2 considers condition based on cell dwelling timer or Mobility state. R2 direction (solution agreements at later stage, no other directions will be considered): - RAN2 acknowledges following two problems to be addressed for idle/inactive UEs: - - Problem 1: For a UE that is physically on a moving vehicle but not camped on its mobile IAB-cell yet (i.e. the UE is camped on a stationary cell), how to help such UE(s) to identify a neighbour mobile IAB-cell, prioritize mobile IAB-cell (frequency and cell) and to be “pulled” into this mobile IAB-cell, especially for inter-frequency scenario where the mobile IAB-cell’s frequency priority is low. - - Problem 2: After the UE physically on a moving vehicle is camped on the mobile IAB cell, how to avoid it reselecting other non-mIAB-(stationary) cells. - - Such UE may prioritize a highest ranked cell at a frequency, if it broadcasts a mIAB-cell type indicator in SIB1 for cell reselection. UE may use the SIB4 assistance information to identify the presence of such mobile IAB-cell(s), if broadcasted. A SIB4 assistance information may include mIAB-cell frequencies. FFS on stage-2/3 to clarify the UE in problem 1 and 2.

This paper discusses remaining issues for idle/inactive-mode mobility for mobile IAB.

# 2	Discussion

RAN2 agreed to introduce an mobile-IAB-cell-type indicator into SIB1 under the conditions that a UE behavior upon detection of this indicator could be agreed. In the last meeting, RAN2 converged on two problems scenarios, where such indicator may be helpful. These problem scenarios are:

- Problem scenario 1: The idle/inactive UE is on a vehicle with an mIAB-node, but it is still camping on a stationary cell. It should prioritize inter-frequency cell re-selection to the mobile-IAB-cell, but it should not prioritize inter-frequency cell re-selection of a stationary cell.

- Problem scenario 2: The idle/inactive UE is on a vehicle with an mIAB-node, and it is camping on the mIAB cell. In this case, in case of cell reselection, e.g., due to DU migration, the mIAB-node should prioritize (intra- or inter-frequency) cell-reselection of mIAB cells.

In both problem scenarios, the desirable behavior can be achieved if the UE prioritizes cells broadcasting the mIAB-node indicator. In problem scenario 1, this prioritization applies to inter-frequency cell re-selection. The UE’s current cell can assist the is prioritization by providing mIAB-specific frequency information.

Proposal 1: The following behavior is defined for a UE receiving the mIAB-cell-type indicator:

- A UE camping on a non-mIAB-cell may prioritize inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

- The UE camping on a mIAB-cell may prioritize intra- and inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

Proposal 2: The non-mIAB cell may provide SIB4 assistance information to the UE to identify the presence of a mIAB-cell, where the assistance information may include the mIAB-cell’s frequency.

The problem scenarios and the corresponding UE behavior should be captured on ST2 in TS 38.304.

Proposal 3: Capture in TS 38.304 the problems identified by RAN2 for the use of the mIAB-cell-type indicator in case it is camping on a non-mIAB-cell (problem 1) and in case it is camping on a mIAB-cell (problem 2). For problem 1, TS 38.304 to also capture the purpose of the assistance information in SIB4.

# Conclusion

This paper discussed remaining issues for idle/inactive-mode mobility for mobile IAB. The following observations and proposals have been made:

Proposal 1: The following behavior is defined for a UE receiving the mIAB-cell-type indicator:

- A UE camping on a non-mIAB-cell may prioritize inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

- The UE camping on a mIAB-cell may prioritize intra- and inter-frequency cell reselection of a cell broadcasting the mIAB-cell-type indicator.

Proposal 2: The non-mIAB cell may provide SIB4 assistance information to the UE to identify the presence of a mIAB-cell, where the assistance information may include the mIAB-cell’s frequency.

Proposal 3: Capture in TS 38.304 the problems identified by RAN2 for the use of the mIAB-cell-type indicator in case it is camping on a non-mIAB-cell (problem 1) and in case it is camping on a mIAB-cell (problem 2). For problem 1, TS 38.304 to also capture the purpose of the assistance information in SIB4.

# References

- Chair Notes; TSG RAN WG2 Meeting #122, Incheon, Korea, May, 2023