3GPP TSG-RAN2 Meeting #123	R2-2307638

# Toulouse, France, Aug 21-25, 2023

Agenda Item:	7.11.2.1

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Service continuity, RRC state transitions and notifications

Document for:	Discussion/Decision

# Introduction

Rel-17 MBS added support of multicast services reception by UEs in CM_CONNECTED/ RRC_CONNECTED state only. One of the objectives of Rel-18 WI on enhancements of NR MBS is as follows (highlighting added), see RP-221458:

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - Specify support of multicast reception by UEs in RRC_INACTIVE state [RAN2, RAN3] - PTM configuration for UEs receiving multicast in RRC_INACTIVE state [RAN2] - Study the impact of mobility and state transition for UEs receiving multicast in RRC_INACTIVE. (Seamless/lossless mobility is not required) [RAN2, RAN3] - <<skip>> Note: collaboration with SA2 is expected in due course for the above objectives.

Previous RAN2 meetings made several agreements related to service continuity, state transitions and notifications. In this contribution, we provide further views on the outstanding issues for multicast reception in RRC_INACTIVE.

# Transition from RRC_CONNECTED to RRC_INACTIVE

## PTM configuration via RRC Release Msg vs via MCCH

RAN2#121bis-e agreed the following:

Similar to Rel-17 broadcast reception procedure, UE acquires new SIB and multicast MCCH to get PTM configuration after cell reselection.

When a UE reselects to a cell for which PTM configuration is not available in multicast MCCH, the UE initiates RRC resumption procedure for an active multicast session it is interested to receive or continue receiving.

The above agreements apply for the case of cell reselection. RAN2#121bis-e also agreed the following:

UE in RRC CONNECTED state is not required to read multicast MCCH to be able to receive multicast MBS service i.e. the UE receives the PTM configuration via dedicated signalling. <<skip>>

RAN2#119bis-e had previously agreed:

Dedicated RRC signalling (i.e. RRC release message with suspendConfig) is used for switching a multicast receiving UE from RRC_CONNECTED to RRC_INACTIVE and continue multicast reception (details FFS).

RAN2#122 also agreed:

The multicast MCCH configuration takes the broadcast MCCH configuration structure (i.e., mcch-Config-r17) as baseline.

For PTM configuration structure on the multicast MCCH, Rel-17 broadcast PTM configuration structure is taken as baseline.

As a baseline, The PTM configuration in the RRCRelease message with suspendconfig has the same structure as the PTM configuration in multicast MCCH.

The above agreements clarify about the structure of the PTM configuration signaling, and as baseline, the agreements aim to keep harmonization of the signaling structure between the MCCH-provided configuration and RRCRelease-provided configuration.

Previous RAN2 agreements aim to keep harmonization of the PTM configuration signaling structure between the MCCH-provided configuration and RRCRelease-provided configuration.

Consider a case where UE in RRC_CONNECTED receives PTM configuration in a RRC Release message. At the same time network is already providing the service for UEs in RRC_INACTIVE state and multicast SIB and MCCH is already being broadcasted. RAN2 previously agreed that same or different PHY resources can be used to deliver same service in RRC_CONNECTED vs RRC_INACTIVE in a cell. RAN2 also agreed how the MCCH change will be notified.

To notify the multicast MCCH change, change notification mechanism for Rel-17 broadcast MCCH is the baseline.

However, there are still several questions to be addressed. E.g.,

- How does the UE know whether the same or different configurations are provided in multicast SIB/MCCH vs the dedicated configuration in the Release message?

- If they are different, when does the MCCH-provided configuration takes precedence over the dedicatedly provided configuration?

It has been agreed that if the UE changes the cell, UE must read MCCH in the new cell. However, if the UE does not change the cell, the UE is not required to read MCCH in RRC_CONNECTED but it is not clear when to read the MCCH after the UE transits from RRC_CONNECTED to RRC_INACTIVE.

Therefore, RAN2 should discuss when the dedicatedly provided configuration ceases to be valid upon transition of the UE from RRC_CONNECTED to RRC_INACTIVE state and when the UE is required to read the MCCH to get the PTM configuration, if different. Without any indication, the UE may keep using the dedicated PTM configuration even if it is not valid anymore and miss the multicast reception.

If the UE does not change cell but transits from RRC_CONNECTED to RRC_INACTIVE, it is unclear when the dedicatedly provided configuration ceases to be valid and when the UE is required to start using MCCH-provided PTM configuration.

It is beneficial if the UE knows whether the current serving cell is broadcasting MCCH or not. And if MCCH is present, it is beneficial to know whether the PTM config is same between the Release msg and MCCH when the UE moves to RRC_INACTIVE.

For RRC_CONNECTED UEs being released to RRC_INACTIVE, the RRC Release message indicates whether MCCH is being broadcasted in the cell. If MCCH is being broadcasted, RRC Release message indicates whether MCCH-provided PTM configuration is same or different from dedicatedly provided configuration.

When the UE transits from RRC_CONNECTED to RRC_INACTIVE, discuss when the PTM configuration provided in RRC Release message stops being applicable, and when the UE is required to start using the MCCH-provided PTM configuration.

## Assistance info for gNB to move a UE to RRC_INACTIVE

RAN2 previously agreed the following in RAN2#119e:

It is up to gNB to decide whether a multicast session may be received by UE(s) in INACTIVE. FFS what information gNB may be provided to form such decision (related to SA2 discussion).

It is assumed the network can choose which UEs receive in RRC INACTIVE and which in RRC Connected and can move UEs between the states for Multicast service reception.

Regarding the “FFS what information gNB may be provided to form such decision (related to SA2 discussion)”, there are two aspects of this:

- Information provided from 5GC to gNB

- Information provided from UE to gNB

### Assistance info from 5GC to gNB

Regarding assistance information provided from 5GC to gNB, SA2 and RAN3 has made some conclusions.

SA2 sent LS to RAN2 in R2-2301934 (S2-2303407), which indicated the following:

- SA2 agrees that the MBS assistance information for the MBS session sent to NG-RAN consists of an indication that the UE is preferred to be kept in connected when receiving the related MBS session data.

And RAN3 agreed the following:

As specified in TS 23.247 [45], the gNB may receive from the 5GC MBS for a UE Assistance Information associated with a multicast MBS session, which assists the gNB in configuring the UE properly. The MBS Assistance Information indicates that the UE is expected to require dedicated resources very frequently. Based on this information, the gNB may decide the RRC state of the UE.

As indicated in above SA2 and RAN3 conclusions, indication about ‘special UE’ (i.e. indication that the UE is preferred to be kept in connected when receiving the related MBS session data and the UE is expected to require dedicated resources very frequently) will be provided to NG-RAN node by the CN, and based on this the gNB “may” decide the RRC state of the UE, i.e. the final decision is up to the gNB.

According to SA2 and RAN3 conclusions, the final decision on the RRC state of the “special UE” is up to the gNB.

RAN2 has previously agreed the following in RAN2#121bis-e:

A "special UE" identified by MBS assistance information from 5GC can be released to RRC_INACTIVE (e.g., when the session is deactivated). FFS how can network enable such UE to resume to RRC_CONNECTED upon session activation

To complement the first part of the above agreement, we propose the following.

It is up to the gNB whether the “special UE” is resumed to RRC_CONNECTED upon session activation or data transmission resumed.

Now, let’s discuss on the FFS part of the above agreement, i.e. how network can resume such ‘special UE’ if it so decides. RAN2#121bis-e agreed the following:

Legacy group paging (i.e., Rel-17 group paging) can be used to resume UE to RRC_CONNECTED state.

UE-specific paging (i.e. PagingRecordList) can be used to move specific MBS multicast UE(s) to RRC_CONNECTED (i.e. legacy UE behavior).

When both enhanced group paging and unicast paging are received by the UE (and targeted for this UE), the UE follows unicast Paging and goes to RRC CONNECTED.

RAN2#122 agreed:

Introduce a new indication per tmgi in the group paging which informs Rel-18 UEs having a valid PTM configuration to receive the multicast in RRC_INACTIVE.

Based on the above agreements, network already has mechanism to resume ‘special UE’ to RRC_CONNECTED upon session activation or data transmission resumed while keeping other UEs in RRC_INACTIVE. This can be done by using group paging with the Rel-18 per TMGI flag included, and unicast paging for the ‘special UE’.

Based on previous RAN2 agreements, network can resume the “special UE” selectively by using unicast paging while keeping the other UEs in RRC_INACTIVE with the Rel-18 group paging.

So, no additional special handling would be needed regarding ‘special UE’.

No additional enhancements are needed specifically for moving “special UEs” back to RRC_CONNECTED.

### Assistance info from UE to gNB

Regarding information provided from UE to gNB, in previous RAN2 meetings some companies proposed UE to provide its preference whether to receive a multicast service in RRC_CONNECTED vs RRC_INACTIVE state. In our view, decision on whether a UE should receive multicast service in RRC_INACTIVE state is totally up to the RAN decision (while taking into account the recommendation about special UE from the CN if applicable) as long as the UE is capable of receiving multicast in RRC_INACTIVE.

UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE. Other than that, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e., it is up to network decision).

# Temporary no data while UE is in RRC_INACTIVE

In Rel-17, upon receiving multicast deactivation indication from 5GC, RAN will release all radio resources and inform UEs by releasing MRB configuration via dedicated RRC signaling procedure.

In case of Rel-18, for RRC_INACTIVE UEs, RAN2#122 agreed the following:

MCCH is used for notifying MC session deactivation for multicast reception in RRC_INACTIVE to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

However, note the FFS from RAN2#121bis-e:

FFS which option to take: enhanced group paging or enhanced MCCH, to enable Rel-18 UE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI upon events like session deactivation/temporary no data.

In other words, RAN2#122 agreed to use MCCH to notify the UE for the case of session deactivation but didn’t explicitly conclude anything for the case of temporary no data.

RAN2#122 agreed to use MCCH to notify the UE for the case of session deactivation but didn’t explicitly conclude anything for the case of temporary no data.

So, to conclude the remaining FFS, we propose the following.

MCCH is used for notifying temporary no data for a multicast session to enable Rel-18 UE in RRC_INACTIVE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.

# Summary

Based on the discussion above, we have following observations:

Observation 1.	Previous RAN2 agreements aim to keep harmonization of the PTM configuration signaling structure between the MCCH-provided configuration and RRCRelease-provided configuration.

Observation 2.	If the UE does not change cell but transits from RRC_CONNECTED to RRC_INACTIVE, it is unclear when the dedicatedly provided configuration ceases to be valid and when the UE is required to start using MCCH-provided PTM configuration.

Observation 3.	It is beneficial if the UE knows whether the current serving cell is broadcasting MCCH or not. And if MCCH is present, it is beneficial to know whether the PTM config is same between the Release msg and MCCH when the UE moves to RRC_INACTIVE.

Observation 4.	According to SA2 and RAN3 conclusions, the final decision on the RRC state of the “special UE” is up to the gNB.

Observation 5.	Based on previous RAN2 agreements, network can resume the “special UE” selectively by using unicast paging while keeping the other UEs in RRC_INACTIVE with the Rel-18 group paging.

Observation 6.	RAN2#122 agreed to use MCCH to notify the UE for the case of session deactivation but didn’t explicitly conclude anything for the case of temporary no data.

Based on these observations and discussion above, we have the following proposals:

Proposal 1.	For RRC_CONNECTED UEs being released to RRC_INACTIVE, the RRC Release message indicates whether MCCH is being broadcasted in the cell. If MCCH is being broadcasted, RRC Release message indicates whether MCCH-provided PTM configuration is same or different from dedicatedly provided configuration.

Proposal 2.	When the UE transits from RRC_CONNECTED to RRC_INACTIVE, discuss when the PTM configuration provided in RRC Release message stops being applicable, and when the UE is required to start using the MCCH-provided PTM configuration.

Proposal 3.	It is up to the gNB whether the “special UE” is resumed to RRC_CONNECTED upon session activation or data transmission resumed.

Proposal 4.	No additional enhancements are needed specifically for moving “special UEs” back to RRC_CONNECTED.

Proposal 5.	UE indicates capability on whether it supports MBS multicast reception in RRC_INACTIVE. Other than that, UE does not provide additional indication to RAN about its preference to receive multicast in RRC_INACTIVE state (i.e., it is up to network decision).

Proposal 6.	MCCH is used for notifying temporary no data for a multicast session to enable Rel-18 UE in RRC_INACTIVE to stay in RRC_INACTIVE and stop monitoring corresponding G-RNTI.