3GPP TSG-RAN2 Meeting #123	R2-2307640

# Toulouse, France, Aug 21-25, 2023

Agenda Item:	7.11.3

Work Item:	NR_MBS_enh-Core

Source: 	Qualcomm Incorporated

Title: 	Shared processing for MBS broadcast and unicast reception

Document for:	Discussion/Decision

# Introduction

Rel-18 WI on enhancements of NR MBS includes the following objective (highlighting added), see [1]:

This Work Item is to further enhance the NR Multicast/Broadcast functions based on Rel-17 MBS. The objectives for Rel-18 include: - <<skip>> - Specify Uu signalling enhancements to allow a UE to use shared processing for MBS broadcast and unicast reception, i.e., ‎including UE capability and related assistance information reporting regarding simultaneous unicast reception in RRC_CONNECTED and MBS broadcast reception from the same or different operators [RAN2] - <<skip>> Note: collaboration with SA2 is expected in due course for the above objectives.

Figure 1 Framework for support of simultaneous reception of MBS + NR unicast operation

The scenario under consideration is a UE receiving NR unicast from a unicast gNB, while receiving (or being interested to receive) NR MBS broadcast from a non-serving gNB (which may be collocated or non-collocated, and may belong to same or different operator), as shown in Figure 1.

When operating with simultaneous NR unicast and broadcast, the amount of available resources for broadcast would depend on the NR unicast configuration. For example, a UE (capable of multiple carriers) configured with a single carrier for unicast may have more leftover baseband resources available for broadcast than the UE configured with multiple NR carriers for unicast.

Related to this objective, RAN2#119e agreed the following:

RAN2 focuses on solutions taking multi-Rx UEs (i.e. no specific enhancements for 1Rx UEs).

RAN2#119bis-e agreed the following:

For shared processing we adopt the following as a baseline:

- 1) new IE is added in system information to control whether MBSInterestIndication for shared processing can be sent or not;

- 2) MBSInterestIndication message content and related procedure is updated for shared processing.

New IE to control whether MBSInterestIndication for shared processing can be sent or not is added to SIB1.

In MBSInterestIndication, for a broadcast service that the UE is receiving or is interested to receive, at least the following information can be signalled: broadcast frequency, subcarrier spacing, and bandwidth. FFS details/exact parameters and other information. FFS in which scenarios the UE reports this information (e.g. intra-PLMN case, inter-PLMN case)

FFS whether UE capability is needed to enable shared processing.

There was no discussion on this agenda item in RAN2#120. Then RAN2#121 agreed the following:

Indicate the capability of receiving MBS broadcast from a non-serving cell. FFS whether the granularity is at FeatureSetDownlink or FeatureSetDownlinkPerCC level.

FFS Whether to include additional information in MII can be controlled by the network. Should consider whether this would be two-step procedure or one-step procedure (e.g. having more info in SIB1)

Then there was no discussion on this agenda item in RAN2#121bis. Further, RAN2#122 agreed the following:

The granularity for capability of receiving MBS broadcast from a non-serving cell is at FeatureSetDownlinkPerCC level. This capability does not imply simultaneous reception on multiple CCs.

No additional signalling is introduced to control information to be reported by the UE (on top of what we have already agreed).

When sending MII, UE reports the whole information (i.e. at least frequency, bandwidth, SCS) when indicated by SIB1 of its unicast serving cell. FFS whether there are cases where this information is not available at the UE and what happens then.

FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII)

No additional information is added to MII on top of what has been already agreed.

This contribution discusses shared processing for MBS broadcast and NR unicast reception and addresses some FFSes above.

# Discussion

## Broadcast frequency, BW, SCS not known to the UE

RAN2 has discussed that UE may receive the information about the available services from the MBS broadcast gNB and decide on which MBS broadcast services it is receiving or is interested to receive from the MBS gNB. Subsequently, RAN2 agreed that when sending MII to the unicast gNB, UE reports the whole information (i.e. at least frequency, bandwidth, SCS). However, there is an outstanding FFS:

FFS whether there are cases where this information is not available at the UE and what happens then.

In the last meeting, some companies explained (e.g., see [2]) that there may be cases where the UE may not be able to acquire SCS and BW from non-serving cell but may only know about the frequency information, e.g., if the UE is not able to read SIB1 from the non-serving cell. The same paper also explained that the UE may report all the information in the MII if available. But if the additional information is not available, the UE can only report the interested broadcast frequency in the MII.

We agree that in such case, UE should be allowed to report whatever information is available (i.e., at least the frequency, and optionally SCS and/or BW if available). Then, UE should be allowed to acquire the additional information from non-serving cell and report it to serving cell once available using an MII update.

In case additional information (SCS, bandwidth) is not available at the time of sending the MII to the unicast serving cell (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE reports whatever is available information at that time (i.e. at least the frequency, and optionally SCS and/or BW as available). UE reports updated MII after acquiring additional information from the non-serving cell.

## Non serving cell updates the configuration relevant to MII

Another outstanding FFS from last meeting is the following:

FFS if any special handling is needed when the non-serving cell updates the configuration (which is relevant for MII)

The discussion in section 2.1 is relevant for this case also. If the non-serving cell updates the configuration, once the UE is aware of such update UE should send updated MII to the unicast serving cell with updated information. No special handling other than allowing the UE to send updated MII is needed to be specified.

In case the non-serving cell updates its configurations relevant to the MII already sent by UE to the unicast serving cell, UE reports updated MII after acquiring the updated information from the non-serving cell. No additional special handling is needed.

# Summary and Proposals

As discussed above, we have the following proposals:

Proposal 1.	In case additional information (SCS, bandwidth) is not available at the time of sending the MII to the unicast serving cell (e.g. the UE is not able to read SIB1 from the non-serving cell), the UE reports whatever is available information at that time (i.e. at least the frequency, and optionally SCS and/or BW as available). UE reports updated MII after acquiring additional information from the non-serving cell.

Proposal 2.	In case the non-serving cell updates its configurations relevant to the MII already sent by UE to the unicast serving cell, UE reports updated MII after acquiring the updated information from the non-serving cell. No additional special handling is needed.

# References

[1] RP-221458, Revised WID: Enhancements of NR Multicast and Broadcast Services, CATT, CBN, June 2022

[2] R2-2304821, Discussion on shared processing for MBS broadcast and unicast reception, Huawei, HiSilicon, May 2023