3GPP TSG-RAN WG2 #123	R2-2307642

Toulouse, France, 21 – 25 August 2023	revision of R2-2305649

Agenda item:		7.4.2.2 RRC

Title:		Further discussion on cell switch

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the past RAN2 meetings, there were many agreements on the LTM cell switch. In RAN2#122, there was one important agreement overring some earlier agreements on MAC reset [1]:

If the TA maintenance etc for candidate cell(s) in the UE is needed, the TA(s) associated with candidate cell(s) can be maintained during LTM (TDB exactly which cells decide stage-3). For non-TA parts, we do MAC reset, which overrides earlier agreements on partial MAC reset. As earlier agreed RLC-AM can continue at LTM cell switch (intended for intra-DU).

During the post RAN2#122 Email discussion [2], the running 38.331 CR has been updated. The running CR reflects this point. In this contribution, we discuss on detailed L2 handling for cell switch, e.g. L2 reset determination and PDCP SDU discard for SRBs,and provide our views.

# 2. Discussion

2.1	L2 reset determination

In the current running CR [2], it is explained that the UE determines whether the UE should perform no L2 reset for an LTM candidate cell upon LTM cell switch based on “ltm-ServingCellNoResetID” and “ltm-NoResetID”. “ltm-ServingCellNoResetID” is replaced by the ID of new serving cell upon LTM cell switch. Both fields are set by the source RRC configuration. We also assume this way would work but see some issues.

At first, it is not yet clear what the L2 reset is. We assume it includes  the MAC reset and the RLC re-establishment, while for PDCP data recovery may or may not be performed (e.g., depending on AM DRBs or UM DRBs). In addition, referring to the previous agreement, the MAC is always reset in the LTM and thus we can consider RLC and PDCP as a part of L2 reset. For RLC, it is obvious that at least the RLC re-establishment is applied for inter-DU LTM. A question is who decides for intra-DU LTM. The simplest way would be that no L2 reset is applied to LTM between any pair of cells managed by one DU and the L2 reset is applied otherwise. In this way, the CU can manage based on differentiation between LTM configuration prepared by source DU and LTM configurations prepared by candidate DUs. However, this way may unnecessarily restrict on the network implementation.

Other way is that the L2 reset is decided by corresponding DU, i.e. source DU decides for intra-DU LTM and the candidate DU(s) decide for LTM among candidate cells of each DU for subsequent LTM. This way gives more flexibility for the network and helps at least if some networks want to have an option of not having RLC re-establishment even in intra-DU LTM. We think this is more reasonable.

Yet another way is that the L2 reset is decided by the CU (i.e. its implementation). However, considering that the RLC is managed by the DU, the DU should have control for the L2 reset especially in a multi-vendor network. Based on the observations, the second way seems better.

Proposal 1: RAN2 to agree that the L2 reset is decided by corresponding DUs as follows:

- Source DU decides for a) LTM between source cell and candidate cells of the S-DU and also b) subsequent LTM among candidate cells of the S-DU,

- Candidate DU decides for subsequent LTM among candidate cells of the candidate DU

If the P1 is agreeable, RAN2 needs to inform the agreement to RAN3 so that they can take necessary action for F1 and Xn interfaces.

2.2	PDCP SDU discard for SRB

Another issue in the L2 handling is the SRB discard. In case of reconfigurationwithSync without security key change, PDCP re-establishment for SRBs is not performed, but those PDCP SDUs/PDUs stored in the PDCP entity are actually RRC signalling towards the source cell. Those RRC message may cause problems if the UE transmits them to the target cell as these messages may be SpCell specific. Therefore, PDCP SDU discard procedure for SRB is performed in order todiscard those PDCP SDU/PDUs to avoid any unnecessary confusion to the network. Currently, UE performs it based on explicitly indication discardOnPDCP received from the network. In the LTM, the security key is not changed regardless of intra-DU or inter-DU and therefore PDCP SDU discard for SRBs shall always be performed.

Next, we discuss how to trigger the UE to always perform the PDCP SDU discard for SRBs during LTM cell switch. Since it was agreed that “The RadioBearerConfig IE can be optionally supported in an LTM candidate configuration”, always configuring discardOnPDCP in the LTM candidate configuration would results in a mandatory supported RadioBearerConfig IE in LTM candidate configuration. To solve this, the UE can just implicitly perform (i.e. without RRC IE) PDCP SDU discard for SRBs during LTM. And this can be applied to each of LTM cell switch (i.e. initial as well as subsequent LTM).

Proposal 2: During the LTM cell switch, the UE implicitly performs (i.e. without RRC IE) PDCP SDU discard for SRBs.

# 3. Conclusion

In this contribution we discussed some more details on cell switch focusing on L2 handling issues and made the following proposals.

Proposal 1: RAN2 to agree that the L2 reset is decided by corresponding DUs as follows:

- Source DU decides for a) LTM between source cell and candidate cells of the S-DU and also b) subsequent LTM among candidate cells of the S-DU,

- Candidate DU decides for subsequent LTM among candidate cells of the candidate DU

Proposal 2: During the LTM cell switch, the UE implicitly performs (i.e. without RRC IE) PDCP SDU discard for SRBs.

# References

[1] RAN2#122, Chair notes

[2] R2-230xxxx, RRC running CR for LTM (outcome of [Post122][055])