3GPP TSG-RAN WG2 #123	R2-2307645

Toulouse, France, 21 – 25 August 2023

Agenda item:		7.3.5	Connected mode mobility

Title:		Discussion on CHO enhancement

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the past RAN2 meetings, there were some agreements related to the Connected mobility below [1][2].

Agreements @RAN2#121bis-e

-	RAN2 agree to make enhancement in CHO procedure based on that the source cell entering “NES mode”.  FFS further details

-	For source cell CHO framework, RAN2 assumes a reference scenario where the UE has already performed CHO conditions evaluation by the time the source cell starts some “NES-mode”

-	As a baseline, UE initiates CHO evaluation upon receiving the CHO configuration.  FFS what trigger is used for execution of CHO

Agreements @RAN2#122

1.	We will have a CHO solution that considers NES mode of at least source cell.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

3.	We will not introduce new L1 signalling for the purpose of CHO

4.	Event A3, A4, A5 can be configured as a CHO execution condition in the NES scenario.   We will study the time based mechanism

In addition, in this meeting RAN1 LS has been received [3], which captures the latest RAN1 agreement below. It seems RAN1 considers the L1 signaling for (de)activation of cell DTX/DRX is feasible, which may be related to the FFS about source cell NES mode.

- Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disabling the feature) from RAN1 perspective. - RAN1 supports the group common L1 signaling using PDCCH for cell DTX/DRX activation and deactivation without HARQ feedback. - subject to UE capability - RAN1 asks RAN2 to consider the additional support of a MAC CE based indication.

In this contribution, we discuss further details on CHO enhancement based on the source cell NES mode and provide our views.

# 2. Discussion

2.1	What is source cell NES mode?

In this WI, there are some aspects related to the network energy saving. However, it is still not yet clear what “NES mode” is. On one hand, the NES mode may not necessarily be defined, because how the network performs the energy saving is basically up to the network implementation along with existing and new functions introduced in this WI. Note that many RAN1 (L1) aspects are also involved. On the other hand, it is useful to assume or roughly define the NES mode at least for technical discussion purpose and maybe also in the specification. One example expecting benefit is the CHO enhancement as per the previous agreement, i.e. make enhancement in CHO procedure based on that the source cell entering “NES mode”.

Observation 1: It may be useful to roughly define a NES mode to make sure a feature related to the NES mode, at least for technical discussion purpose.

Focusing on RAN2 scope, the Cell DTX/DRX is one of the functions achieving network energy saving. Regardless of actual ON/OFF period, once the Cell DTX/DRX is activated, it can be considered as the NES mode. During Cell DTX/DRX, it will be reasonable to serve limited number of UEs in the cell and other UEs are handed over to other cell(s).

Regarding the Cell DTX/DRX activation/deactivation, RAN1 confirms that it is feasible to support the group common L1 signaling using PDCCH and is asking about possibility of having additional MAC CE based indication. This MAC CE possibility should be discussed separately under Cell DTX/DRX sub-agenda. However, we understand that RAN2 can assume at least L1 signaling is feasible for Cell DTX/DRX activation/deactivation.

Proposal 1: RAN2 to confirm that the network is in NES mode while it activates the Cell DTX/DRX.

Proposal 2: RAN2 to confirm that at least L1 signaling can be used for Cell DTX/DRX activation/deactivation.

Other NES functions should be also considered. From RAN2 perspective, the network may update the SIB contents to perform the energy saving. For example, the network may change the cell reselection parameters so that the legacy UEs tend to select other cells or change the SSB or SIB periodicities from shorter value to longer value. This consideration has one issue. The network may or may not intend the NES mode or operation even if similar configurations are applied. In one case, the network actually intends to be in NES mode, while in other case the network does not. If the network needs to let the UE know whether the network is in NES mode or not, a certain indication is necessary in SIB1.

Observation 2: Further discussions on NES mode are necessary, where SIB parameters update may be considered.

2.2	CHO enhancement based on source cell NES mode

Next, we discuss the CHO enhancement based on the previous agreements. In the last meeting, there were some proposals on CHO enhancement e.g., different CHO threshold configurations or event configurations depending on source cell NES mode. For example, the UE changes the condition (e.g. by using different thresholds) upon a certain network signallig or upon entering the NES mode at the network side. Then, it was agreed to have a specific NES CHO execution condition based on source cell NES mode. For this agreement, there are two FFS, i.e. 1) how the UE determines is in NES mode and 2) how this is achieved in RRC.

Observation 3: The previous agreement implies that a specific NES CHO execution condition is applied when the source cell is in the NES mode.

Before discussing the FFS, we would like to discuss the actual meaning of the specific NES CHO execution condition. The agreement can be interpreted into 2 ways:

- Approach 1: specific NES CHO execution condition is validated based on source cell NES mode

- Approach 2: specific NES CHO execution condition is evaluated based on source cell NES mode

It is assumed that a specific NES CHO execution condition is configured to a UE (potentially together with other legacy CHO execution condition). The approach 1 means that the UE initiates the evaluation for the specific condition upon CHO configuration but does not judge whether the condition is met or not until the source cell is in NES mode. This sounds a bit strange, because the evaluation already includes the meaning of judgement of the condition. On the other hand, the approach 2 sounds very natural. The UE initiates the evaluation for the specific condition upon the source cell is in NES mode.

Another difference is on the measurements. In the approach 1 the UE initiates corresponding measurements upon NES CHO configuration, while in the approach 2 the UE initiates corresponding measurements upon moving NES mode. This might cause some delay in measurements in the approach 2. However, the intention of this NES CHO configuration is not as urgent as legacy (C)HO. In addition, measurements on the target frequency (or neighbour cell) may be performed for other purposes e.g., legacy (C)HO. There should be no critical problem.

Regarding the CHO evaluation timing, it had been agreed “as a baseline”, UE initiates CHO evaluation upon receiving the CHO configuration. However, the CHO evaluation needs to be performed for an intended condition. If the agreement should be realized as discussed above (i.e. approach 2), the earlier agreement can be updated so that the UE initiates the CHO evaluation for the specific NES CHO execution condition upon entering the NES mode in the source cell (e.g. when the UE determines it is in NES mode). This would mean to delay in initiating the execution condition evaluation.

Proposal 3: RAN2 to agree that the UE initiates the CHO evaluation for the NES specific execution condition when the UE determines the source cell is in NES mode.

Next, we discuss the remaining FFS.

2.	We can have a specific NES CHO execution condition based on source cell NES mode.   FFS how the UE determines is in NES mode.   FFS on how this is achieved in RRC

For the first FFS above, this highly depends on the discussions in section 2.1 and its outcome or conclusion. With assuming that both the P1 and the P2 are agreeable, one way to achieve this is that the UE determines when it receives the activation indication of the Cell DTX/DRX. Although it was agreed that “We will not introduce new L1 signalling for the purpose of CHO”, the Cell DTX/DRX (de)activation indication is the separate function and thus can be reused also for the CHO enhancement.

Another way is to add an indication in SIB1 which says the network intends the energy saving somehow, as discussed in 2.1. However, this needs more discussions independent from CHO enhancement. Here we focus on the Cell DTX/DRX and propose as follows.

Proposal 4: The UE determines that the serving cell is NES mode, when it receives the activation indication of Cell DTX/DRX. FFS for other possibilities e.g., SIB update with NES indication.

For the second FFS above, the NES specific execution condition can be configured like the legacy CHO. With assuming the P3 is agreeable, one difference from legacy is that the UE stores such the configuration but not uses right after reception. RRC ASN.1 should include a certain information which is indicated explicitly with one bit or implicitly via ASN.1 structure itself. A question is whether other legacy CHO execution condition (if it is also configured) is ignored or kept the target of CHO execution evaluation. Before going into details further, it would be good to solve the aspects discussed above at first and then move to discussions on RRC details.

Observation 4: There are still many aspects to be concluded before discussing RRC details.

# 3. Conclusion

In this contribution we discussed further details on CHO enhancement based on the source cell NES mode and made the following proposals.

Proposal 1: RAN2 to confirm that the network is in NES mode while it activates the Cell DTX/DRX.

Proposal 2: RAN2 to confirm that at least L1 signaling can be used for Cell DTX/DRX activation/deactivation.

Proposal 3: RAN2 to agree that the UE initiates the CHO evaluation for the NES specific execution condition when the UE determines the source cell is in NES mode.

Proposal 4: The UE determines that the serving cell is NES mode, when it receives the activation indication of Cell DTX/DRX. FFS for other possibilities e.g., SIB update with NES indication.

# References

[1] R2-2304203, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

[2] R2-2306543, Report from Session on NES, UAV, Rel-15-17 UP, Rel-17 Small Data, IIoT/URLLC, and RACH partitioning

[3] R1-2306246=R2-2307017, Reply LS on Cell DTX/DRX activation/deactivation, RAN1