3GPP TSG-RAN WG2#123			R2-2307646

Toulouse, France, Aug 21-25, 2023

Title:	[Draft] Reply LS on applicability of pre-configured measurement gaps for RedCap UE

Release:	Rel-17

Work Item:	          NR_redcap-Core

Source:	RAN2

To:	RAN3

Cc:	RAN4

Contact Person:

#### Name:	Linhai He

E-mail Address:	linhaihe@qti.qualcomm.com

Attachments:

1. Overall Description:

RAN2 thanks RAN3 for the LS regarding “applicability of pre-configured measurement gaps for RedCap UE”.

RAN2 discussed following RAN3 question:

Based on “gapToAddModList IE” definition in MeasGapConfig, RAN3 would like to ask RAN2 whether it is possible that this IE can be used to configure different Pre-MGs for different BWPs, since the RedCap UE will require a MG whenever it performs a measurement outside its active BWP.

RAN2 Answer:

RAN2 would like to inform RAN3 that due to RAN4 restrictions, only one pre-configured measurement gap is allowed per gap type (i.e, UE type or per FR type), by using “gapToAddModList IE” in MeasGapConfig. Nevertheless, the design of preConfGapStatus per BWP includes multiple bits, thus allowing signaling support for activating/deactivating multiple pre-configured gaps per BWP. That helps ensure forward compatibility with future requirements.

RAN2 would like to inform RAN3 that pre-configured MGs are applicable to both RedCap and non-RedCap UEs.

2. Actions:

To RAN3:

ACTION: 	RAN2 kindly asks RAN3 to take above response into account for RAN3 signalling design work.

3. Date of Next TSG-RAN WG3 Meetings:

TSG-RAN WG2#123bis                   09 – 13 October 2023			Xiamen, China

TSG-RAN WG2#124                       13 – 17 November 2023		Chicago, USA