3GPP TSG-RAN WG2 Meeting #123	R2-2307652

Toulouse, France, August 21-25, 2023

Agenda item:	    7.21.2

Source: 		Qualcomm Incorporated

Title: 	UL Coverage Enhancements-Control Plane

Document for:	Discussion

# Introduction

In this contribution we discuss some views on Rel-18 Further UL Coverage Enhancements. The WID [1] asks RAN2 to handle the objective below:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. The detailed objectives of the work item are as follows: - Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable.)

The following was agreed in the last RAN2 meeting [2]:

- MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg3-based SI request - FFS for MSG1 repetition can be applicable to the 4-step CBRA procedure initiated by Msg1-based SI request. - RAN2 intends to support CFRA for msg1 repetition for ReconfigurationWithSync case, FFS for other cases - RAN2 to agree to configure multiple RSRP thresholds for different repetition numbers - The RSRP threshold(s) for triggering Msg1 repetition are configured per-BWP - RAN2 to further discusss fallback from lower number of MSG1 repetition to higher number which is also FFS for now. We need to understand how to signal this and how this impacts MAC procedure.

A long email discussion is ongoing [3] with the following extracted proposals from Phase 1:

Proposal 1: MSG1 repetition can be applicable to 4-step CBRA procedure initiated by Msg1-based SI request (4/2). Proposal 2: Each RSRP threshold is configured separately by RRC, which is associated with a repetition number if configured (5/4) . Proposal 3: A single feature priority for MSG1 repetition is configured by RRC, i.e. all the MSG1 repetition numbers use the same feature priority (8/1). Proposal 4: Send LS to RAN1 to ask the feasibility and necessity on the support of PDCCH order based CFRA with MSG1 repetition. Proposal 5: Send LS to RAN1 to ask the feasibility and necessity on the support of CFRA with MSG1 repetition for BFR. Proposal 6: NW indicates ONE MSG1 repetition number applicable for CFRA MSG1 repetition by RRC for Reconfiguration with sync. (7/2)

In this contribution, we provide our views on the outstanding issues in the control plane

# Criteria for Msg 1 Repetition

Currently, RAN1 and RAN2 have agreed that RSRP would be the criteria for the UE to select the correct repetition number. However, one point that needs RAN2 further consideration is whether other criteria need to be considered when selecting a repetition number. To this end, we think UE power class should also be an additional criterion that can be taken into consideration when determining whether to do Msg 1repetition.

Observation 1: Using a single Msg-1 repetition RSRP threshold for all UE power classes can be suboptimal in terms of coverage enhancements design.

Proposal 1: RAN2 to consider other criteria for Msg-1 repetition, namely, UE power class.

# RRC Signalling

RRC signalling has been discussed extensively in previous two meetings with no resolution on how to signal Msg-1 repetition parameters in the RRC. The rapporteur nicely summarized the options in [3] as follows:

- Option 1: use three reserved bits in featureCombiantion IE for indicating the MSG1 repetition number 2, 4 and 8.

- Option 2: use one reserved bit in featureCombiantion IE for indicating MSG1 repeititon as a feature and introduce a new paratmer in featureCombinationPreamble IE for indicating the associated MSG1 repetition number 2, 4 or 8.

- Option 3: introduce a new paratmer outside featureCombiantion IE for indicating the associated MSG1 repetition number 2, 4 or 8. No change to featureCombiantion IE.

In our view, using three bits in featureCombiantion is an overkill and not future compatible. Thus, it is enough to use a single bit in featureCombiantion and indicate featureCombinationPreamble the different repetition numbers.

Proposal 2: A single bit in featureCombination is used to indicate Msg-1 repetition and the different repetition numbers are indicated in featureCombinationPreamble.

## Fallback Modelling

In the User Plane email discussion [4], there is widespread support for modelling fallbacks using the option 2.2

- Option 2.2: All repetitions are treated as a single feature, but within the feature, different repetition numbers are treated as different RACH type - In this case Msg1 repetition is considered as a feature; - But, RACH resources for all repetitions are considered as RACH type within the same feature; - In this option, the fallback can be supported within the RACH partition and the different Msg1 repetitions are treated as different RACH types within the same feature (i.e. similar to 2-step and 4-step RACH today, where we allow fallback from 2-step to 4-step RACH); - Requires big RRC spec change, e.g. to introduce msg1-Repetition-r18 in FeatureCombination-r17, and to introduce multiple preamble index ranges (and/or RO mask indexes) within FeatureCombinationPreambles-r17, each one associated with a specific repetition number;

Since the direction of RAN2 is to support fallback using RRC enhancements, it would be appropriate within the control plane discussions to discuss how RRC can be modified to enable fallbacks between repetition numbers and how to inform the UE of the conditions and procedure of fallback between repetition numbers.

Observation 2: Fallback Option 2.2 which has seen some support across companies needs some RRC enhancements to support fallbacks.

To that end, RAN2 can discuss how fallbacks can be configured and signalled to the UE.

Proposal 3: RAN2 to discuss RRC enhancements needed to signal feedback to the UE.

# SIB1 Configuration

In our companion contribution, we have proposed that some types of fallbacks should be supported. However, if the gNB was to encode detailed information, e.g., about each RSRP threshold the UE must use to determine repetition for each attempt, the size of SIB1 can get very large making the feature unattractive for deployment.

Observation 3: Fallback between repetitions number, while useful, may cause too much SIB1 overhead if not encoded carefully.

Thus, we propose that RAN2 should discuss ways to indicate fallback efficiently to UEs.

Proposal 4:  RAN2 to discuss how SIB1 can efficiently indicate fallback to a higher repetition number to UE, e.g., defining a default behaviour or indicating an RSRP step decrement between RACH attempts.

# Conclusion

Observation 1: Using a single Msg-1 repetition RSRP threshold for all UE power classes can be suboptimal in terms of coverage enhancements design.

Proposal 1: RAN2 to consider other criteria for Msg-1 repetition, namely, UE power class.

Proposal 2: A single bit in feature combination is used to indicate Msg-1 repetition and the different repetition numbers are indicated in featureCombinationPreamble.

Observation 2: Fallback Option 2.2 which has seen some support across companies needs some RRC enhancements to support fallbacks.

Proposal 3: RAN2 to discuss RRC enhancements needed to signal feedback to the UE.

Observation 3: Fallback between repetitions number, while useful, may cause too much SIB1 overhead if not encoded carefully.

Proposal 4:  RAN2 to discuss how SIB1 can efficiently indicate fallback to a higher repetition number to UE, e.g., defining a default behaviour or indicating an RSRP step decrement between RACH attempts.

# References

[1] RP-221858 Revised WID on Further NR coverage enhancements

[2] RAN2 122 Chair notes

[3] Summary of [Post122][801][R18CEenh-CP] CP open issues (Huawei)

[4] Summary of [Post122][802][R18CEenh-UP] UP open issues (ZTE)