3GPP TSG-RAN WG2 Meeting #123	R2-2307654

Toulouse, France, August 21-25, 2023

Agenda item:	    7.23

Source: 		Qualcomm Incorporated

Title: 	Timing Synchronization Status and Reporting

Document for:	Discussion

# Introduction

In this contribution we discuss the 5GS network timing synchronization status and reporting relating to the new Timing Resiliency and URLLC Enh WI introduced in [1] as follows:

1. 5GS network timing synchronization status and reporting [RAN3, RAN2]: a. AMF providing clock quality reporting control information per-UE to the gNB. [RAN3] b. gNB delivering 5G Clock quality information to the UE in RRC_CONNECTED state, based on the clock quality reporting control information and gNB capability. [RAN2, RAN3] Note 1: Details of the 5G clock quality information will be decided by RAN3. c. UE in RRC_IDLE and RRC_INACTIVE state determining that the 5G Clock quality information has changed via information received in the broadcast signalling. [RAN2] d. gNB reporting node-level RAN timing synchronization status information towards the AMF, based on RAN timing synchronization status reporting configuration and gNB capability. [RAN3]

RAN2 has discussed this item in RAN2 #122 with the following agreements [2]

- RAN2 to confirm that there is no need to support “group of cells across gNBs” for scope of the report ID. The scenario, as per RAN3 will be supported - different cells within gNB that are served by different DUs and different clock quality is possible. - Confirm the following SA2’s conclusions regarding NG-RAN procedures on 5G clock quality information: - RRC_INACTIVE/IDLE UE to be informed of timing synchronization status via an event ID in SIB9 by comparison with the one maintained locally. - RRC_CONNECTED UE to be informed of 5G clock quality information via dedicated RRC message - Postpone the UAC until CT1’s feedback on UAC framework to randomize UE access to network in the time domain - 5G Clock Quality information is carried in DLInformationTransfer message. - FFS if update of event ID is informed to UE by SI modification procedure - For Idle/Inactive mode RAN2 assumes that the NAS layer triggers the RRC connection procedure based on inputs from AS. - The AS layer of the UE determines if there a change of event ID and it indicates a change in the RAN timing synchronization status to NAS layers. FFS to double check for connected mode and inactive

And further discussions in RAN2 #123 resulted in the following agreements:

Agreements

- Update of event ID is informed to UE by normal SI modification procedure.

- Confirm the AS layer of the UE determines if there a change of event ID and/or gNB ID. If there is a change, the AS layer notifies the change in the RAN timing synchronization status to NAS layer. For both IDLE and INACTIVE mode, NAS layer may requests the RRC layer to move to RRC_CONNECTED

- For RRC_CONNECTED mode UEs, the NW has the necessary information to determine whether to send detailed clock quality information to the UE, (i.e. it may choose to always send update or only when needed) and the details can be left to NW implementation

- Event ID is optional.  Under the same gNB, UE considers the change of timing synchronization if event ID field from SIB9 is different.   FFS if gNB always broadcasts event ID

- If the UE is in RRC_INACTIVE, the UE can acquire clock quality information using the SDT procedure, if it supports and/is configured with SDT procedure.

# UAC Framework

RAN2 had already sent an LS to CT1 [3] asking “if we can use an existing access category for the access attempt to retrieve the latest available clock quality information” to which CT1 has replied as follows [4]:

Question to CT1: RAN2 would like to know if we can use an existing access category for the access attempt to retrieve the latest available clock quality information, e.g., the one for MO signalling, or if it would require a new one since it should not block access triggered by other MO signalling. Answer from CT1: Unless there is a service requirement to introduce a new access category for the access attempt to retrieve the latest available clock quality information, CT1 assumes that reusing Access Category 3 (MO_sig) is acceptable. 2. Actions: To RAN2 group. ACTION: CT1 kindly requests RAN2 to take the above answer into account. To SA1 group. ACTION: CT1 respectfully asks SA1 to inform if there is a service requirement to use a new access category for the access attempt described above.

SA1 has further replied in [4] as follows:

On RAN2’s question to CT1: RAN2 would like to know if we can use an existing access category for the access attempt to retrieve the latest available clock quality information, e.g., the one for MO signalling, or if it would require a new one since it should not block access triggered by other MO signalling. CT1 answered: Unless there is a service requirement to introduce a new access category for the access attempt to retrieve the latest available clock quality information, CT1 assumes that reusing Access Category 3 (MO_sig) is acceptable. And further raised the following question to SA1: CT1 respectfully asks SA1 to inform if there is a service requirement to use a new access category for the access attempt described above. [SA1 answer]: SA1 would like to inform that there are no stage 1 requirements to have a separate access category for TRS.

Given that SA1 has answered that there is no stage 1 requirement to have a separate access category, RAN2 can assume that Access Category 3 can be reused to attempt access for clock quality report retrieval. In our view, the current UAC would then be sufficient to avoid a RACH storm for the UEs attempting to retrieve clock information; Given that this design was done with a subscription model in mind, we do not see motivation to go beyond the existing UAC framework.

Proposal 1: No further work is needed on RAN2 to enhance UAC framework to avoid RACH collisions.

# DL Information Transfer

SA2 concluded which clock quality information can be provided to the UE based on feedback from RAN3. TS 23.501 (Rel-18) clause 5.7.1.12 now reads:

If the clock quality detail level equals "clock quality metrics", the NG-RAN provides clock quality metrics to the UE that reflect its current timing synchronization status. i.e. one or more of the following information elements: clock accuracy, traceability to UTC and to GNSS, frequency stability, parent time source, synchronization state as defined in Table 5.27.1.12-1. NG-RAN is locally configured which of the clock quality metrics supported by NG-RAN are provided to UE(s).

Table 5.27.1.12-1 has further details:

| Information Name | Description |
| --- | --- |
| Synchronization state | Indicates the state of the node synchronization, represented by the values "Locked", "Holdover", or "Freerun" (NOTE 1). |
| Clock quality |  |
| >> Traceable to GNSS | Indicates whether the current time source is traceable to the GNSS and represented by values “Yes” or “No”. |
| >> Traceable to UTC | Indicates whether the current time source is traceable to the UTC and represented by values “Yes” or “No”. |
| >> Frequency stability | Describes the estimate of the variation of the local clock when it is not synchronized to another source (NOTE 2). |
| >> Clock Accuracy | Describes the mean in ns over an ensemble of measurements of the time between the clock under test and a reference clock (NOTE 3). |
| Parent time source | Describes the primary source the node is currently using, represented by the values "SyncE", "PTP", "GNSS", "atomic clock", "terrestrial radio", "serial time code", "NTP", "hand_set", "other". |
| NOTE 1: Clock is in the "Locked", "Holdover", or "Freerun" mode, as defined in ITU-T G.810 [164]. NOTE 2: Frequency stability is estimated in a similar manner as for offsetScaledLogVariance attribute defined in clause 7.6.3.5 of IEEE Std 1588 [126]. NOTE 3: Clock accuracy measurement considers accuracy up to gNB antenna and RAN internal process. |  |

Given that SA2 has determined which information needs to be signalled to the UE, RAN2 can encode this information in DLInformationTransfer in the Stage 3 design phase.

Proposal 2: RAN2 to take Table 5.27.1.12-1 in 23.501 as a baseline for the clock quality report transmitted RRC_CONNECTED to UEs via DLInformationTransfer.

# Conclusion

Proposal 1: No further work is needed on RAN2 to enhance UAC framework to avoid RACH collisions.

Proposal 2: RAN2 to take Table 5.27.1.12-1 in 23.501 as a baseline for the clock quality report transmitted RRC_CONNECTED to UEs via DLInformationTransfer.

# References

[1]   RP-230754 New WID on NR Timing Resiliency and URLLC enhancements

[2] RAN2 121 bis Chair notes

[3] RAN2 122 Chair notes

[4] C1-232942 Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s)

[5] S1-231285 Response to Reply LS on Proposed method for Time Synchronization status reporting to UE(s)