| 3GPP TSG RAN WG2 Meeting #123 |  | R2-2307655 |
| --- | --- | --- |
| Toulouse, France, August 21st – August 25th, 2023 |  |  |
| Agenda item: | 7.9.2 |  |
| Source: | Fraunhofer IIS, Fraunhofer HHI |  |
| Title: | Discussion on using short ID in U2U relaying |  |
| Document for: | Discussion and Decision |  |

# Introduction

In RAN2#122 meeting, the following agreements were achieved regarding SRAP header design for U2U Relay:

Agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

This document discusses the pros and cons of two main ID formats (L2 ID and short ID) in U2U relaying and the impacts of short ID on possible multi-hop use cases.

# L2 ID and Short ID for U2U Relaying

Based on [1], one of the functionalities of SRAP sublayer is determination of UE ID field and BEARER ID field for data packets in case of U2N relay. This will be the case for U2U as well. Hence, the identity information of remote UE and the end-to-end Radio Bearer should be included in the adaptation layer in first and second PC5 hop [2].

According to the report of RAN2#122 meeting, the decision between two ID formats (L2 ID and short ID) in the SRAP header is still up for debate:

[ToDis] Proposal 14: RAN2 to discuss which ID (24-bit L2 ID or short ID) can be used in SRAP header.

The following aspects are to be considered regarding the U2U remote UE ID assignment in the adaptation layer:

- Specification impact

- Header overhead

- Issues of ID collision

- Future applicability to Multi-hop (Forward compatible)

In the following table, the pros and cons of L2 ID and short ID are listed also based on the summary of company contributions:

|  | Pros | Cons |
| --- | --- | --- |
| Layer 2 ID | - No spec impact for L2 ID allocation - Reuse the NAS mechanism for L2 ID collision resolution | - SRAP header overhead - Handle L2 ID update impacts - The requirement to update L2 ID frequently would be disruptive - Long ID will create intractable problems like the scalability over multiple hops, and the issues with the short ID may be more manageable() - Using L2 ID in multihop, one broken link can cause reconfiguration of the whole path() - L2 ID is the link ID not UE ID, and could be collision after multi-hop relay () - The remote UE does not know the Layer-2 ID of the peer remote UE ID () - If privacy is required, L2 ID will be updated every 5 minutes, then PC5-S/RRC signalling is needed every time L2 ID is updated from each Remote UE |
| Short ID | - align with U2N design - <24 bit short ID with lower overhead - Local ID of U2N remote UE length is 8 bits | - Not sure whether short ID will be updated similar to U2N relay - Design short ID allocation mechanism - Design AS mechanism for short ID collision resolution - Local IDs are not really “end-to-end” and need to be removed and replaced by each relay UE before each next-hop forwarding. |

The main issue with L2 ID is the large overhead (48 bits for both Source and Destination UEs) in the SRAP header. Howerver, one can view this issue as a benefit in terms of uniqueness of the remote UE ID which is important to avoid collision, especially when there are more than one U2U Relay UEs between Source and Destination UEs. The use of L2 ID instead of short ID is preferable since there is no need to cope with collision and multi-hop compatibility issue.

By using L2 ID instead of short ID, there is no need to cope with collision and multi-hop compatibility issue.

In addition, according to [3], Source L2 IDs are always self-assigned by the UE originating the corresponding L2 frames. The selection of the source and destination L2 ID(s) by a UE depends on the communication mode of V2X communication over PC5 reference point for this L2 link. Therefore, another advantage of using L2 ID is to minimize gNB involvement and spare the open question, which UE should assign UEs ID. Self-assigned L2 IDsalso avoids Relay UE(s) complexity to update IDs.

Source L2 IDs are always self-assigned by the UE originating the corresponding L2 frames.

Assuming large overhead is not a significant limiting factor in adaptation layer design for U2U relay, based on the above discussions we poropose:

It is preferable to include L2 ID as the remote UE ID in SRAP header.

On the other hand, to remain consistant with U2N Relay format it is suggested to rely on local ID. Different options were proposed during the RAN2#122 meeting:

[ToDis] Proposal 16a: If short ID is agreed, RAN2 to discuss which option should be supported:

Option 2: Target remote UE ID (local ID) in first hop and source remote UE ID (local ID) in second hop.

Option 4: Both source remote UE ID (local ID) and target remote UE ID (local ID) included in each hop, the local ID is same on each hop and relay UE does not replace the local ID on each hop.

Option 5: A local pair ID for a pair between source remote UE and target remote UE included in each hop, the local ID is unique within one PC5 hop and relay UE needs to replace the local ID on each hop.

Option 6: A local pair ID for a pair between source remote UE and target remote UE included in each hop, the local ID is same on each hop and relay UE does not replace the local ID on each hop.

In case short ID is agreed, it is preferable to consider signaling issues for both single-hop and multi-hop and alignment with U2N scenarios. From signaling overhead point of view, Option 2 is preferred with 8 bits of length for each Source and Destination local ID, but this is not feasible in multi-hop scenario.

Option 2 has the minimum signaling overhead but it is not applicable to multi-hop scenario.

Therefore, we suggest:

Exclude Option 2, as it is not applicable to multi-hop scenario.

In order to support multi-hop in the future release it is better to consider a single ID, which is common on each hop. This way, U2U relay UE(s) need to store just one local pair ID. As the local ID would be the same on each hop, there is no need to do additional calculations on every received SRAP PDU in U2U Relay(s). Thus, delivery could be done by mapping the local ID on corresponding E2E connection based on L2 IDs of Source and Destination UE. For this reason, we suggest to go for Option 6, which has less signaling overhead compared to Option 4 and Option 5.

If RAN2 agrees on short ID, include one common local pair ID in SRAP layer for each hop (Option 6).

RAN2 to consider a local pair ID for a pair of source/target remote UEto be included in each hop(Option 6) to avoid collision.

# Conclusions

The following observations and proposals have been made in this document:

Observation 1:	By using L2 ID instead of short ID, there is no need to cope with collision and multi-hop compatibility issue.

Observation 2:	Source L2 IDs are always self-assigned by the UE originating the corresponding L2 frames.

Observation 3:	Option 2 has the minimum signaling overhead but it is not applicable to multi-hop scenario.

Proposal 1:	It is preferable to include L2 ID as the remote UE ID in SRAP header.

Proposal 2:	Exclude Option 2, as it is not applicable to multi-hop scenario.

Proposal 3:	If RAN2 agrees on short ID, include one common local pair ID in SRAP layer for each hop (Option 6).

Proposal 4:	RAN2 to consider a local pair ID for a pair of source/target remote UEto be included in each hop(Option 6) to avoid collision.

# References

[1] TS 38.351 V17.5.0 “Sidelink Relay Adaptation Protocol (SRAP) Specification”, 2023-06

[2] TS 38.836 v17.0.0 “Study on NR sidelink relay”, 2021-03

[3] TS 23.287 V17.4.0 “Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services”, 2022-09