3GPP TSG RAN WG2 Meeting #123                                                           R2-2307659

# Toulouse, France, August 21-25, 2023

Agenda item:	7.19.3

Source:	Intel Corporation

Title:	UE Capability Discussion for Rel-18 eRedCap WI

Document for:	Discussion and decision

# Introduction

This document discusses the impacts on UE capabilities to support Rel-18 UEs with complexity/cost reduction in FR1 [1], characterized by a reduction of UE’s peak data rate and optionally a reduction of UE’s BB bandwidth, considering related agreements from RAN1/2.

# Discussion

## RAN1 feature list on eRedCap

RAN1 sent an LS [1] with the updated RAN1 feature list [2] and the following information was provided for Rel-18 eRedCap. It is highlighted in green key points for RAN2 consideration, in pink point flagged by RAN1 for RAN2 discussion and in yellow those that are still under discussion by RAN1. The TP associated to any capability included in RAN1 feature list [2] will be included in the Mega-Capability CR, however it is important that the working groups discuss any unclear point to easy the CR process. To easy the online discussion, an example of the corresponding TP is included in Annex.

| # | 48-1 | 48-2 |
| --- | --- | --- |
| Group | RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1 | RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1 |
| Components | The following components are the same as for supportOfRedCap-r17 (28-1): 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH 4. Separate initial UL BWP for RedCap UEs - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources 5. Separate initial DL BWP for RedCap UEs - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included 6. 1 UE-specific RRC configured DL BWP per carrier 7. 1 UE-specific RRC configured UL BWP per carrier 8. RRC reconfiguration of any parameters related to BWP 9. UE-specific RRC configured DL BWP with CD-SSB or NCD-SSB 10. NCD-SSB based measurements in RRC-configured DL BWP The following components are new compared to supportOfRedCap-r17 (28-1): [11. DL/UL peak data rate target of 10 Mbps] 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS 13. Relaxed RAR-PDSCH processing timeline FFS whether to add additional components | The capabilities of FG 48-2 are the same as for FG 48-1 except that the following restriction does not apply:: 12. Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS [13. Relaxed RAR-PDSCH processing timeline] |
| Pre-req. | - | 48-1 |
| Need for gNB to know if feature is supported | Yes | Yes |
| Consequence if the feature is not supported by the UE | Network assumes the UE is not a RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1. | Network assumes the UE is not a RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1. |
| Type | [Per UE] | [Per UE] |
| FDD/ TDD diff | [No] | [No] |
| FR1/ FR2 diff | [No] (FR1 only) | [No] (FR1 only) |
| Note | A UE supporting this FG is not required to support FG 6-1. A UE supporting this FG is not allowed to support FG 28-1. The specifications for a UE supporting FG 28-1 (‘RedCap UE’) also apply for a UE supporting this FG (FG 48-1) unless stated otherwise. It is up to RAN2 whether/how to capture the capabilities for early indication of RedCap UE in Msg 3 and Msg A. |  |
| Mandatory/ Optional | Optional with capability signaling UEs supporting Rel-18 eRedCap UE complexity reduction feature(s) indicate support of this FG instead of FG 28-1 (supportOfRedCap-r17). | Optional with capability signaling |

The following observations could be highlighted from RAN1 feature list:

RAN1 feature list have already included two new capabilities associated to Rel-18 eRedCap UE: (FG 48-1) RedCap UE with reduced peak data rate with reduced baseband bandwidth in FR1 and (FG 48-2) RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1.

RAN1 feature list clearly shows the differences and relation between Rel-17 RedCap (FG 28-1) and Rel-18 eRedCap (FG 48-1 and 48-2) while only uses the term RedCap for both (i.e., there seems no need from RAN1 PoV to introduce the term eRedCap).

RAN1 pointed how Rel-18 eRedCap shares majority of the features as Rel-17 RedCap capability except for the one highlighted in blue below:

supportOfRedCap-r17 Indicates that the UE is a RedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Maximum FR2 RedCap UE bandwidth is 100 MHz; - Support of RedCap early indication based on Msg1, MsgA (if UE indicated support of twoStepRACH-r16) and Msg3 for random access; - Separate initial UL BWP for RedCap UEs; - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP for RedCap UEs; - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. A RedCap UE shall set the field to supported.

RAN2 would need to discuss how to capture the corresponding feature information in TS 38.306 and 38.331 considering RAN1/2 agreements on Rel-18 eRedCap and how Rel-17 RedCap was also captured.

- RAN1 feature list seems to mainly use RedCap term even for Rel-18 adding the reference to the new key features with “reduced peak data rate” and with/without “reduced baseband bandwidth in FR1”.

- RAN2 endorsed running CRs uses the term (e)RedCap, eRedCap and RedCap considering the benefit identified to simplify the specification impact. This would also seems aligned to RAN1 point that the specifications for a UE supporting Rel-17 RedCap (FG 28-1) also apply for a UE supporting Rel-18 eRedCap (FG 48-1) unless stated otherwise.

Considering RAN2 decision taken on the running CRs, it seems simpler if the support of Rel-18 eRedCap is defined independently to Rel-17 RedCap support. RAN1 feature list also pointed that a UE supporting Rel-18 eRedCap UE complexity reduction feature(s) indicate support of this FG 48-1 instead of FG 28-1 (supportOfRedCap-r17).

Details on RAN1 feature list for FG 48-1 and 48-2 seems not consistent when using the term RedCap and eRedCap in reference to Rel-18 eRedCap. Our suggestion is to continue using RAN2 terminology agreed on RAN2 running CRs (i.e., eRedCap for Rel-18 eRedCap (which would include both FG 48-1 or 48-2), RedCap  for Rel-17 RedCan and (e)RedCap for both releases).

The following proposals could be discussed from RAN1 feature list and UE capability draftCRs to 38.306 and 38.331 are also updated accordingly to include RAN1 inputs in the latest feature list R1-2306223 [2]:

- The support of Rel-18 eRedCap (FG 48-1 and 48-2) is defined as independently of Rel-17 RedCap (FG 28-1) understanding that RAN1 also agreed that UE supporting Rel-18 eRedCap feature(s) indicate support of this FG 48-1 instead of FG 28-1 (supportOfRedCap-r17).

- New UE capability (referred e.g., as supportOfEnhancedRedCap-r18) is defined to capture FG 48-1 (i.e., RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1) with the corresponding details explained in RAN1 feature list (R1-2306223).

- New UE capability (referred e.g., supportOfNotReducedBB-BW-r18) is defined to capture FG 48-2 (i.e., RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1) with the corresponding details explained in RAN1 feature list (R1-2306223).

- To remove from RAN2 running Capability CRs any reference to supportOfEnhancedRedCap-r18 as it is part of RAN1 feature list and its corresponding TP should be captured as part of Mega-Capability CRs. If so, to agree to the update done on UE capabilities running CR to 38.3006 and 38.331 in R2-2307657 and R2-2307659.

## Msg3/MsgA PUSCH early indication

RAN1 pointed that it is up to RAN2 decision how to capture the Msg3/MsgA PUSCH early indication. 38.321 running CR already defined two new LCIDs for eRedCap UEs to indicate when sending CCCH/CCCH1 as agreed by RAN2.

- All R18 eRedCap UEs uses the two new LCIDs for Msg3/MsgA PUSCH for CCCH/CCCH1 during Random Access, i.e., both those with peak rate reduction + BB BW reduction, and those with only peak rate reduction.

As this feature seems to be required by any Rel-18 eRedCap UE seems sufficient to include it as part of the list of components required by the new UE capability defined for supportOfEnhancedRedCap-r18.

- To add in the list of functional components for the supportOfEnhancedRedCap-r18 the support of eRedCap early indication based on Msg3 and MsgA PUSCH.

## Applicability of RAN2 Rel-17 RedCap capabilities to Rel-18 eRedCap

Rel-18 WID [3] captures that existing UE capability framework is used, and changes to capability signalling are specified only if necessary (i.e., by default, all UE capabilities applicable to a Rel-17 RedCap UE are applicable unless otherwise specified). This is aligned to RAN1 agreement captured in above table, i.e., “the specifications for a UE supporting FG 28-1 (‘RedCap UE’) also apply for a UE supporting this FG (FG 48-1) unless stated otherwise”.

Rel-18 eRedCap WID and RAN1 agreement explicitly state that by default, any specification applicable to a Rel-17 RedCap UE is also applicable to Rel-18 eRedCap UEs unless otherwise captured.

This section discusses whether there is any RAN2-centric Rel-17 RedCap feature that should not be applicable to Rel-18 RedCap UEs considering the characteristics already explained.

RAN2 feature list defined for Rel-17 RedCap is summarized below:

| Feature group | Components | Field name in TS 38.331 | Parent IE in TS 38.331 |
| --- | --- | --- | --- |
| Support 16 DRBs | Defines whether the RedCap UE supports 16 DRBs as specified in TS 38.331. | supportOf16DRB-RedCap-r17 | RedCapParameters-r17 |
| Support 18 bit length of PDCP sequence number | Defines whether the RedCap UE supports 18 bit length of PDCP sequence number as specified in TS 38.331. | longSN-RedCap-r17 | PDCP-Parameters |
| Support AM DRB with 18 bit length of RLC sequence number | Defines whether the RedCap UE supports AM DRB with 18 bit length of RLC sequence number as specified in TS 38.331. | am-WithLongSN-RedCap-r17 | RLC-Parameters |
| Support of RRM relaxation in RRC_CONNECTED | Defines whether the RedCap UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331. | rrm-RelaxationRRC-ConnectedRedCap-r17 | MeasAndMobParametersCommon |

There is no reason why Rel-18 eRedCap UE cannot share the same RAN2 centric features defined for Rel-17 RedCap.

It seems reasonable to extend the support of RAN2-centric features defined for Rel-17 RedCap UE capabilities to Rel-18 eRedCap UEs.

Possible options on how to capture in TS 38.306 that Rel-17 RedCap UE capabilities of RAN2-centric features are all applicable to Rel-18 eRedCap UEs:

- Add in the field description of R18 eRedCap capability (i.e. supportOfEnhancedRedCap-r18) a general statement that explains that all supportOfRedCap-r17 related capabilities specified in this specification remain applicable for Rel-18 RedCap UEs, unless indicated otherwise.

- Update RedCap reference to (e)RedCap as it was done in other running CRs and add the definition of (e)RedCap. This can be a simple change as it would only update the field description of the RAN2-centric Rel-17 RedCap UE capabilities (i.e., supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17, rrm-RelaxationRRC-ConnectedRedCap-r17). On summary, the usage of (e)RedCap UEs (instead than RedCap) would make the feature an applicable capabilitu to any UE indicating supportOfEnhancedRedCap-r17 or supportOfEnhancedRedCap-r18. For example, as shown below:

| supportOf16DRB-RedCap-r17 Indicates whether the (e)RedCap UE supports 16 DRBs. This capability is only applicable for (e)RedCap UEs. | UE | No | No |
| --- | --- | --- | --- |

| longSN-RedCap-r17 Indicates whether the (e)RedCap UE supports 18 bit length of PDCP sequence number. This capability is only applicable for (e)RedCap UEs. | UE | No | No |
| --- | --- | --- | --- |

| am-WithLongSN-RedCap-r17 Indicates whether the (e)RedCap UE supports AM DRB with 18 bit length of RLC sequence number. This capability is only applicable for (e)RedCap UEs. | UE | No | No |
| --- | --- | --- | --- |

| rrm-RelaxationRRC-ConnectedRedCap-r17 Indicates whether (e)RedCap UE supports Rel-17 relaxed RRM measurements in RRC_CONNECTED as specified in TS 38.331 [9]. | UE | No | No | No |
| --- | --- | --- | --- | --- |

- A Rel-18 eRedCap UE (both FG 48-1 and FG 48-2) can also support all RAN2-centric Rel-17 RedCap UE capabilities in the same manner (i.e., supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17, rrm-RelaxationRRC-ConnectedRedCap-r17).

  - Discuss how to capture this in TS 38.306: option 1) add in the field description of R18 eRedCap capability (i.e. supportOfEnhancedRedCap-r18) the following statement “all supportOfRedCap-r17 related capabilities specified in this specification remain applicable for Rel-18 RedCap UEs, unless indicated otherwise” or option 2) update the field description of the RAN2-centric Rel-17 RedCap UE capabilities to be applicable to (e)RedCap UEs.

  - Definition of Rel-18 eRedCap in TS 38.306

The following definition of Rel-17 RedCap is included in TS 38.306:

4.2.21.1	Definition of RedCap UE

RedCap UE is the UE with reduced capability:

-	The maximum bandwidth is 20 MHz for FR1, and is 100 MHz for FR2. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 or wider than 100 MHz in FR2 are not supported by RedCap UEs;

-	The maximum mandatory supported DRB number is 8;

-	The mandatory supported PDCP SN length is 12 bits while 18 bits being optional;

-	The mandatory supported RLC AM SN length is 12 bits while 18 bits being optional;

-	For FR1, 1 DL MIMO layer if 1 Rx branch is supported, and 2 DL MIMO layers if 2 Rx branches are supported; for FR2, either 1 or 2 DL MIMO layers can be supported, while 2 Rx branches are always supported. For FR1 and FR2, UE features and §corresponding capabilities related to more than 2 UE Rx branches or more than 2 DL MIMO layers, as well as UE features and capabilities related to more than 1 UE Tx branch or more than 1 UL MIMO layer are not supported by RedCap UEs;

-	CA, MR-DC, DAPS, CPAC and IAB (i.e., the RedCap UE is not expected to act as IAB node) related UE features and corresponding capabilities are not supported by RedCap UEs. All other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise.

In running CR to 38.306, it was agreed to create similarly a new definition/section for Rel-18 eRedCap. Taken into consideration the points discussed in this document, we suggest considering the following points:

4.2.x.1	Definition of eRedCap UE

eRedCap UE is the UE with reduced peak data rate and, with or without reduced baseband bandwidth in FR1:

-	The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE in bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2.

-	The specifications and capabilities of a RedCap UE are also applicable to eRedCap UEs unless stated otherwise.

- To include the following in “section 4.2.x.1	Definition of eRedCap UE” of TS 38.306:

  - eRedCap UE is the UE with reduced peak data rate and, with or without reduced baseband bandwidth in FR1:

    - The maximum bandwidth is 20 MHz for FR1. UE features and corresponding capabilities related to UE bandwidths wider than 20 MHz in FR1 are not supported by eRedCap UEs. eRedCap UEs do not support operation in FR2.

    - The specifications and capabilities of a RedCap UE are also applicable to eRedCap UEs unless stated otherwise.

## Maximum peak data rate reduction – foreseen impact

RAN1 agreed to the following on UE peak data rate reduction [4]:

RAN1#110bis-e:

Agreement: - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, (replaced by later agreement) - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, [38.306] - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary.

RAN1#111:

Agreement: (replaced by later agreement) - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL

RAN1#112:

Agreement: Revise the earlier agreement by removing the square brackets like this: [38.306] - The minimum DL peak rate target (for FD-FDD) is [10] Mbps based on peak data rate calculation according to 38.306. - The same value for X is used for DL and UL Agreement: [38.306] For the relaxed constraint X in the following earlier RAN1 agreement, down-select between X = 3 and X = 3.2. - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X.

RAN1#113:

Agreement: [38.306] - For UE peak data rate reduction with UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to a vLayers·Qm·f of 3.2 - For UE peak data rate reduction without UE BB bandwidth reduction, - The 10-Mbps peak rate target corresponds to a vLayers·Qm·f of 0.75 - This is assuming 20 MHz bandwidth in the 38.306 peak rate expression. - Note: This does not imply that downlink MIMO and 256 QAM are not supported

Considering above agreements, RAN2 could agree on the specification impact to TS 38.306. For further reference on RAN1 discussion, the intention was to find a proper product X equal to (vLayers·Qm·f) that allows us to get 10Mbps peak data rate. I.e. a Rel-18 RedCap UE would need a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps. For example, the proper product of (layer, modulation, scaling factor) will be 3.2. One possible example can be (one layer, 16qam, scaling factor 0.75) to get product of 3.

Considering RAN1 feature list, different values of ‘x’ are used depending on whether UE BB BW is or not reduced which is differentiated via capabilities (i.e., FG 48-1, 48-2).

Rel-18 RedCap UE would indicate a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps and no smaller than X (for vLayers·Qm·f ≥ X) by using existing capability signaling (i.e., there is no impact to UE capability signaling associated with layer, modulation order and scaling factor).

To enable this reduction of the data rate, RAN1 is discussing to relax the value of X in vLayers·Qm·f ≥ X. From RAN2 point of view, this would require an update to section 4 on “Supported max data rate for DL/UL” in TS 38.306. For example, as shown below:

For single carrier NR SA operation, the UE (except a UE indicating supportOfERedCap-r18) shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4. For a UE indicating supportOfEnhancedRedCap-r18 in single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 0.75 if UE does not indicate supportOfNotReducedBB-BW-r18 or 3.2 if UE also indicates supportOfNotReducedBB-BW-r18.

NOTE 3: As an example, the value 4 in the component above can correspond to vLayers(j)=1, Qmj= 4 and fj=1.

Even though RAN1 has not sent an LS on this topic, RAN2 can happen as RAN1 related agreements are already available. Moreover RAN1 Rapporteur marked them with 38.306 foreseen input in its summary document that listed all RAN1 agreements [4].

- Section 4 on “Supported max data rate for DL/UL” in TS 38.306 needs to be updated to include RAN1 agreement on the new value(s) of X for which the legacy constraint “vLayers·Qm·f ≥ 4” is relaxed by capturing the following TP: “For single carrier NR SA operation, the UE (except a UE indicating supportOfERedCap-r18) shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4. For UE indicating supportOfEnhancedRedCap-r18 in single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 0.75 if UE does not indicate supportOfNotReducedBB-BW-r18 or 3.2 if UE also indicates supportOfNotReducedBB-BW-r18.”).

# Conclusion

The observations captured are the following:

Observation 1.	RAN1 feature list have already included two new capabilities associated to Rel-18 eRedCap UE: (FG 48-1) RedCap UE with reduced peak data rate with reduced baseband bandwidth in FR1 and (FG 48-2) RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1.

Observation 2.	RAN1 feature list clearly shows the differences and relation between Rel-17 RedCap (FG 28-1) and Rel-18 eRedCap (FG 48-1 and 48-2) while only uses the term RedCap for both (i.e., there seems no need from RAN1 PoV to introduce the term eRedCap).

Observation 3.	Details on RAN1 feature list for FG 48-1 and 48-2 seems not consistent when using the term RedCap and eRedCap in reference to Rel-18 eRedCap. Our suggestion is to continue using RAN2 terminology agreed on RAN2 running CRs (i.e., eRedCap for Rel-18 eRedCap (which would include both FG 48-1 or 48-2), RedCap  for Rel-17 RedCan and (e)RedCap for both releases).

Observation 4.	Rel-18 eRedCap WID and RAN1 agreement explicitly state that by default, any specification applicable to a Rel-17 RedCap UE is also applicable to Rel-18 eRedCap UEs unless otherwise captured.

Observation 5.	It seems reasonable to extend the support of RAN2-centric features defined for Rel-17 RedCap UE capabilities to Rel-18 eRedCap UEs.

Observation 6.	Rel-18 RedCap UE would indicate a combination of (layer, modulation, scaling factor) that allows obtaining the target 10Mbps and no smaller than X (for vLayers·Qm·f ≥ X) by using existing capability signaling (i.e., there is no impact to UE capability signaling associated with layer, modulation order and scaling factor).

The proposals captured are the following:

Proposal 1.	The support of Rel-18 eRedCap (FG 48-1 and 48-2) is defined as independently of Rel-17 RedCap (FG 28-1) understanding that RAN1 also agreed that UE supporting Rel-18 eRedCap feature(s) indicate support of this FG 48-1 instead of FG 28-1 (supportOfRedCap-r17).

Proposal 2.	New UE capability (referred e.g., as supportOfEnhancedRedCap-r18) is defined to capture FG 48-1 (i.e., RedCap UE with reduced peak data rate and reduced baseband bandwidth in FR1) with the corresponding details explained in RAN1 feature list (R1-2306223).

Proposal 3.	New UE capability (referred e.g., supportOfNotReducedBB-BW-r18) is defined to capture FG 48-2 (i.e., RedCap UE with reduced peak data rate without reduced baseband bandwidth in FR1) with the corresponding details explained in RAN1 feature list (R1-2306223).

Proposal 4.	To remove from RAN2 running Capability CRs any reference to supportOfEnhancedRedCap-r18 as it is part of RAN1 feature list and its corresponding TP should be captured as part of Mega-Capability CRs. If so, to agree to the update done on UE capabilities running CR to 38.3006 and 38.331 in R2-2307657 and R2-2307659.

Proposal 5.	To add in the list of functional components for the supportOfEnhancedRedCap-r18 the support of eRedCap early indication based on Msg3 and MsgA PUSCH.

Proposal 6.	A Rel-18 eRedCap UE (both FG 48-1 and FG 48-2) can also support all RAN2-centric Rel-17 RedCap UE capabilities in the same manner (i.e., supportOf16DRB-RedCap-r17, longSN-RedCap-r17, am-WithLongSN-RedCap-r17, rrm-RelaxationRRC-ConnectedRedCap-r17).

Proposal 6.1.	Discuss how to capture this in TS 38.306: option 1) add in the field description of R18 eRedCap capability (i.e. supportOfEnhancedRedCap-r18) the following statement “all supportOfRedCap-r17 related capabilities specified in this specification remain applicable for Rel-18 RedCap UEs, unless indicated otherwise” or option 2) update the field description of the RAN2-centric Rel-17 RedCap UE capabilities to be applicable to (e)RedCap UEs.

Proposal 8.	Section 4 on “Supported max data rate for DL/UL” in TS 38.306 needs to be updated to include RAN1 agreement on the new value(s) of X for which the legacy constraint “vLayers·Qm·f ≥ 4” is relaxed by capturing the following TP: “For single carrier NR SA operation, the UE (except a UE indicating supportOfERedCap-r18) shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 4. For UE indicating supportOfEnhancedRedCap-r18 in single carrier NR SA operation, the UE shall support a data rate for the carrier that is no smaller than the data rate computed using the above formula, with J=1 CC and component vLayers(j)⋅Qmj⋅fj is no smaller than 0.75 if UE does not indicate supportOfNotReducedBB-BW-r18 or 3.2 if UE also indicates supportOfNotReducedBB-BW-r18.”).

# Reference

- R1-2306225, LS on Rel-18 RAN1 UE features list for NR after RAN1#113, From RAN1, To RAN2, Ccin RAN4, May 2023.

- R1-2306223, Updated RAN1 UE features list for Rel-18 NR after RAN1#113, May 2023.

- RP-223544, Enhanced support of reduced capability NR devices, Rel-18 WID, December 2022.

- R1-2306261, RAN1 agreements for Rel-18 NR RedCap (sorted by topic), May 2023.

# Annex

Example of how the RAN1-centric capabilities may look in the Mega-Capability CR shown below.

#### 4.2.x.2	General parameters

| Definitions for parameters | Per | M | FDD-TDD DIFF |
| --- | --- | --- | --- |
| supportOfEnhancedRedCap-r18 Indicates that the UE is an eRedCap UE with comprised of at least the following functional components: - Maximum FR1 RedCap UE bandwidth is 20 MHz; - Support of RedCap early indication based on Msg1 for 4-step RACH; - Separate initial UL BWP for RedCap UEs; - It includes the configuration(s) needed for RedCap UE to perform random access - Enabling/disabling of frequency hopping for common PUCCH resources - Separate initial DL BWP for RedCap UEs; - It includes CSS/CORESET for random access - For separate initial DL BWP used for paging, CD-SSB is included - For separate initial DL BWP only used for RACH, SSB may or may not be included - For separate initial DL BWP used in connected mode as BWP#0 configuration option 1, CD-SSB is included - 1 UE-specific RRC configured DL BWP per carrier; - 1 UE-specific RRC configured UL BWP per carrier; - UE-specific RRC-configured DL BWP with CD-SSB or NCD-SSB; - NCD-SSB based measurements in RRC-configured DL BWP. - [DL/UL peak data rate target of 10 Mbps] - Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - Relaxed RAR-PDSCH processing timeline FFS whether to add additional components A eRedCap UE shall set the field to supported and shall not indicate support of supportOfRedCap-r17. | [UE] | CY | [No] |
| supportOfNotReducedBB-BW-r18 Indicates that the UE is an eRedCap UE without reduced baseband bandwidth in FR1. UE supporting this feature shall indicate support of supportOfEnhancedRedCap-r18 except the support of the following functional components: - Maximum number of PDSCH/PUSCH PRBs that can be scheduled for unicast per slot of 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - [Relaxed RAR-PDSCH processing timeline] | [UE] | CY | [No] |