3GPP TSG-RAN WG2 Meeting #123	R2-2307662

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:			7.2.1

Source:			Intel Corporation

Title:		Further Considerations on new SLPP specification

Document for:	 	Discussion and decision

# Introduction

At RAN2#121, based on [5] [6], RAN2 discussed the structure of the SLPP specification and concluded:

Agreement:

Use “SLPP” (without hyphen) as the name of the new protocol.

Regarding the structure of SLPP, e.g. general part, procedure part , Information Element Abstract Syntax Definition,  the structure of LPP (TS 37.355) can be used as baseline for further discussion. The content of each section will be added in accordance with future agreements, not based on LPP legacy directly. FFS on procedure description in the field description as LPP.

Regarding the ASN.1 part of SLPP, follow NR RRC approach, e.g.

-	FFS on Need code (e.g. how to support no UL/DL), support of delta signalling

-	Define ASN.1 elements for common UE capabilities in a dedicated section (i.e. “UE capability information elements”); FFS whether any positioning method specific capability IEs should be grouped by positioning method.

-	Common section for constraints

-	“nonCriticalExtension” at message level

-	Fields in the field description are sorted based on alphabetical order

-	FFS on whether setup release structure should be introduced in SLPP

At RAN#99, RAN plenary approved the updated WID on expanded and improved NR positioning [7] and allocated the specification number TS38.355 for new specification on Sidelink Positioning Protocol as:

| New specifications |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Remarks |
| TS | 38.355 | NR; Sidelink Positioning Protocol (SLPP); Protocol Specification | 101 | 102 | Core part, RAN2-led Editor: Yi Guo (yi.guo@intel.com) |

At RAN2#121bis-e, RAN2 further discussed the issues related to TS38.355 [8-10], and agreed:

TS 38.355 v0.0.2  Endorsed as a baseline for future discussion RAN2 agree that, for session-based SLPP, SLPP transactions are indicated at the SLPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response)” RAN2 agree that for session-based SLPP, messages within a transaction are linked by a common transaction identifier.

At RAN2#122, based on [13], RAN2 further agreed:

Agreement:

WA: The SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.

In this contribution, we continue the discussion on the details of TS38.355.

# Discussion

### 2.1 Changes for TS 38.355 v0.0.4

Based on comments received at RAN2#122 on TS 38.355 v0.0.3 [12], following changes are captured in the TS 38.355 v0.0.4 [14].

- The ASN.1 text in the spec follows the 38.331 approach with landscape/spaces instead of portrait/tabs.

- Add term “Target UE, Anchor UE, Ranging” in the clause 3,  refer to RAN1 agreements and TS 23.586:

  - Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE via PC5 interface.

  - Anchor UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. over the Sidelink interface.

  - Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple Anchor UEs using Sidelink in the Ranging based service and Sidelink positioning.

  - SL	Sidelink

- Clause 6.1, replace the last sentence by a description similar to what has been specified in LPP. Reason is that the description in clause 8.1, 8.3, 8.4, 8.5 in 38.331 is RRC specific, e.g. clause 8.5 describes padding in RRC (for PCCH and BCCH, if required due to the granularity of the TF signalling) that does not apply for SLPP

  - Replace “SLPP messages and IEs defined in this clause use the same transfer syntax rule as described in clause 8.1, 8.3, 8.4 and 8.5 of TS 38.331 [2].”

  - By the following similar to what has been specified for LPP:

“Transfer syntax for SLPP messages is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [x]. The encoded SLPP message always contains a multiple of 8 bits.”

- Clause 6.1, also add

  - “Transfer syntax for SLPP IEs is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [4]. The encoded SLPP IE always contains a multiple of 8 bits. This applies when a single SLPP IE is encoded as the basic production, i.e. for other purposes than encoding the SLPP IE within an SLPP message.SLPP messages and IEs defined in this clause use the same transfer syntax rule as described in clause 8.1, 8.3, 8.4 and 8.5 of TS 38.331 [2].

  - When specifying information elements which are to be represented by BIT STRINGs, if not otherwise specifically stated in the field description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	the last bit (rightmost bit) contains the least significant bit (LSB).”

- Annex A: The change history table should be filled with the Tdoc# for each version.

- Corrections:

  - Clause 6.2.1: in ASN.1 of SLPP-Message the field name “Message” should start with lowercase letter. Furthermore, there is a redundant dot in the description of SLPP-Message.

  - Clause 6.4: there is a typo in “End of SLPP-PDU-Definitioqns”.

The updated TS 38.355 v0.0.4 based on above changes is provided in [14].

Proposal 1: use TS38.355 v0.0.4 as baseline to capture agreements made in the meeting.

### 2.2 Open issues for TS 38.355

So far, following open issues are captured in the TS 38.355:

Editor’s note FFS on the definition of SLPP Session. Editor’s note FFS on the supported positioning methods. Editor's note FFS on whether SLPP message Segmentation is needed. Editor’s note FFS on the need of reliable transport. Editor's note The content of each section will be added in accordance with future agreements, not based on LPP legacy directly. Editor's note FFS on whether to add procedure description in the field description as LPP. Editor's note FFS on Need code (e.g. how to support no UL/DL), support of delta signalling, full configuration, import IE from LPP, setup/release. Editor's note FFS on whether message header, e.g. transaction ID should be put in the SLPP-message level or same as RRC in corresponding message. Editor's note FFS on whether any positioning method specific capability IEs should be grouped by positioning method. Editor's note FFS on SLPP message header, e.g. cast type, session ID, UE ID Editor's note FFS on The SLPP ASN.1 design should allow for "selective ASN.1 compilation". The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module. A "group" may correspond to a positioning method, but other grouping may also be possible. An implementation needs to compile only the SLPP modules which contain a supported "group" (functionality, positioning method, etc.)

1 Regarding the “Editor’s note	FFS on the need of reliable transport.”, RAN2 has agreed:

- SLPP over PC5-U/Uu will support reliable transport for at least unicast. FFS groupcast.

LPP also supports reliable/in sequence delivery of LPP messages, and include duplicate detection, acknowledgement and retransmission. As discussed in [16], it is reasonable to simply apply the same principles for the transport layer requirements for reliability as LPP. Therefore,

Proposal 2: SLPP reliable transport includes duplicate detection, acknowledgement and retransmission functions, LPP section 4.3.1-4.3.4 can be used as baseline.

Two alternatives of the TP on reliable transport are shown in the annex 5 (Alt 1) and annex 6.

Proposal 3: RAN2 to decide which alternative (alt 1 or alt 2) should be the baseline for SLPP reliable tranport.

2 Regarding open issue “Editor's note	FFS on whether message header, e.g. transaction ID should be put in the SLPP-message level or same as RRC in corresponding message.”, for LPP, message header IDs are put outside of message body IE because for acknowledgement, an acknowledgement response may contain no SLPP message body (in which case only the sequence number being acknowledged is significant). Therefore at least sequence number and acknowledgement should be put outside of message body.

Proposal 4: Same as LPP, transactionID, 	endTransaction, sequenceNumber, acknowledgement shall be put outside of message body, use text proposal in annex 7 as baseline.

3 Regarding open issue “Editor's note	FFS on The SLPP ASN.1 design should allow for "selective ASN.1 compilation". The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module. A "group" may correspond to a positioning method, but other grouping may also be possible. An implementation needs to compile only the SLPP modules which contain a supported "group" (functionality, positioning method, etc.)”, RAN2 has agreed:

- Agreement:

- WA: The SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.

The intention of “selective ASN.1 compilation” is to allow the low cost devices only need to compile the ASN.1 for supported functionality. Therefore, the SLPP message is organized/consist of individual IEs for different functionality group, e.g. an IE common for all positioning methods, and separate IEs for different positioning methods, and IEs are defined as an OCTET STRING. The UE only need to encode/decode the container of supported method, and does not need to understand the ASN.1 of non-supported positioning which can reduce efforts/complexity of UE. During the discussion, companies would like to see how it works.

The ASN.1 example on “selective ASN.1 compilation” is shown in annex of [15]. We also copied it based on TS 38.355 v 0.0.4 in annex 8.

Proposal 5: Confirm the WA “the SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.” and use text proposal in annex 8 as baseline.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: use TS38.355 v0.0.4 as baseline to capture agreements made in the meeting.

Proposal 2: SLPP reliable transport includes duplicate detection, acknowledgement and retransmission functions, LPP section 4.3.1-4.3.4 can be used as baseline.

Proposal 3: RAN2 to decide which alternative (alt 1 or alt 2) should be the baseline for SLPP reliable tranport.

Proposal 4: Same as LPP, transactionID, 	endTransaction, sequenceNumber, acknowledgement shall be put outside of message body, use text proposal in annex 7 as baseline.

Proposal 5: Confirm the WA “the SLPP ASN.1 design should allow "selective ASN.1 compilation", i.e. The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module.” and use text proposal in annex 8 as baseline.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] TS 36.331

[3] TS 37.355

[4] TS 38.331

[5] R2-2300409	Considerations on new SLPP specification	Intel Corporation

[6] R2-2302147	[AT121][411][POS] SLPP specification (Intel)	Intel Corporation

[7] RP-230328	Revised WID: Expanded and Improved NR Positioning	Intel Corporation

[8] R2-2304296	[AT121bis-e][422][POS] SLPP specification baseline (Intel)	Intel Corporation

[9] R2-2304306	TS 38.355 skeleton	Intel Corporation	draft TS	Rel-18	38.355	0.0.2

[10] R2-2304302	Report of [AT121bis-e][429][POS] Session-based SLPP (Samsung)	Samsung

[11] R2-2303591	Sidelink Positioning Protocol (SLPP) Signaling and Procedures	Qualcomm Incorporated

[12] R2-2305439	TS 38.355 v 0.0.3 	Intel Corporation

[13] R2-2305438	Further considerations on SLPP specification	Intel Corporation

[14] R2-2307663	TS 38.355 v 0.0.4 	Intel Corporation

[15] R2-2306422	Sidelink Positioning Protocol (SLPP) Signaling and Procedures	Qualcomm Incorporated

[16] R2-2307661Further Considerations on Sidelink Positioning Intel Corporation

# Text proposal of reliable transport (Alt 1)

## 4.3	SLPP Transport

### 4.3.1	Transport Layer Requirements

SLPP requires reliable, in-sequence delivery of SLPP messages from the underlying transport layers. This clause describes the transport capabilities that are available within SLPP. A UE implementing SLPP shall support SLPP reliable transport (including all three of duplicate detection, acknowledgement, and retransmission).

### 4.3.2	SLPP Duplicate Detection

A sender shall include a sequence number in all SLPP messages sent for a particular location session. The sequence number shall be distinct for different SLPP messages sent in the same direction in the same location session (e.g., may start at zero in the first SLPP message and increase monotonically in each succeeding SLPP message). Sequence numbers used in the uplink and downlink are independent (e.g., can be the same).

A receiver shall record the most recent received sequence number for each location session. If a message is received carrying the same sequence number as that last received for the associated location session, it shall be discarded. Otherwise (i.e., if the sequence number is different or if no sequence number was previously received or if no sequence number is included), the message shall be processed.

Sending and receiving sequence numbers shall be deleted in a server when the associated location session is terminated and shall be deleted in a target device when there has been no activity for a particular location session for 10 minutes.

Editor's note	May be updated based on the discussion on session management.

### 4.3.3	SLPP Acknowledgement

#### 4.3.3.1	General

Each SLPP message may carry an acknowledgement request and/or an acknowledgement indicator. A SLPP message including an acknowledgement request (i.e., that include the IE ackRequested set to TRUE) shall also include a sequence number. Upon reception of an SLPP message which includes the IE ackRequested set to TRUE, a receiver returns an SLPP message with an acknowledgement response (i.e., that includes the ackIndicator IE set to the same sequence number of the message being acknowledged). An acknowledgement response may contain no SLPP message body (in which case only the sequence number being acknowledged is significant); alternatively, the acknowledgement may be sent in an SLPP message along with an SLPP message body. An acknowledgement is returned for each received SLPP message that requested an acknowledgement including any duplicate(s). Once a sender receives an acknowledgement for an SLPP message, and provided any included sequence number is matching, it is permitted to send the next SLPP message. No message reordering is needed at the receiver since this stop-and-wait method of sending ensures that messages normally arrive in the correct order.

When an SLPP message is transported via a NAS MO-LR request, the message does not request an acknowledgement.

#### 4.3.3.2	Procedure related to Acknowledgement

Figure 4.3.3.2-1 shows the procedure related to acknowledgement.

Figure 4.3.3.2-1: SLPP Acknowledgement procedure

1.	Endpoint A sends an SLPP message N to Endpoint B which includes the IE ackRequested set to TRUE and a sequence number.

2.	If SLPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number, Endpoint B shall return an acknowledgement for message N. The acknowledgement shall contain the IE ackIndicator set to the same sequence number as that in message N.

3.	When the acknowledgement for SLPP message N is received and provided the included ackIndicator IE matches the sequence number sent in message N, Endpoint A sends the next SLPP message N+1 to Endpoint B when this message is available.

### 4.3.4	SLPP Retransmission

#### 4.3.4.1	General

This capability builds on the acknowledgement and duplicate detection capabilities. When an SLPP message which requires acknowledgement is sent and not acknowledged, it is resent by the sender following a timeout period up to three times. If still unacknowledged after that, the sender aborts all SLPP activity for the associated session. The timeout period is determined by the sender implementation but shall not be less than a minimum value of 250 ms.

#### 4.3.4.2	Procedure related to Retransmission

Figure 4.3.4.2-1 shows the procedure related to retransmission when combined with acknowledgement and duplicate detection.

Figure 4.3.4.2-1: SLPP Retransmission procedure

1.	Endpoint A sends an SLPP message N to Endpoint B for a particular location session and includes a request for acknowledgement along with a sequence number.

2.	If SLPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded), Endpoint B shall return an acknowledgement for message N. If the acknowledgement is received by Endpoint A (such that the acknowledged message can be identified and sequence numbers are matching), Endpoint A skips steps 3 and 4.

3.	If the acknowledgement in step 2 is not received after a timeout period, Endpoint A shall retransmit SLPP message N and shall include the same sequence number as in step 1.

4.	If SLPP message N in step 3 is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded and whether or not the message is considered a duplicate), Endpoint B shall return an acknowledgement. Steps 3 may be repeated one or more times if the acknowledgement in step 4 is not received after a timeout period by Endpoint A. If the acknowledgement in step 4 is still not received after sending three retransmissions, Endpoint A shall abort all procedures and activity associated with SLPP support for the particular location session.

5.	Once an acknowledgement in step 2 or step 4 is received, Endpoint A sends the next SLPP message N+1 for the location session to Endpoint B when this message is available.

# Text proposal of reliable transport (Alt 2)

## 4.3	SLPP Transport

### 4.3.1	Transport Layer Requirements

SLPP requires reliable, in-sequence delivery of SLPP messages from the underlying transport layers. This clause describes the transport capabilities that are available within SLPP. A UE implementing SLPP shall support SLPP reliable transport (including all three of duplicate detection, acknowledgement, and retransmission).

The SLPP reliable transport uses the same requirements specified in clauses 4.3.2, 4.3.3, and 4.3.4  of TS 37.355 [x].

# Text proposal of message header and message body

#### –	SLPP-Message

The SLPP-Message provides the complete set of information for an invocation or response pertaining to an LPP transaction.

-- ASN1START

-- TAG-SLPP-MESSAGE-START

SLPP-Message ::=            SEQUENCE {

transactionID           SLPP-TransactionID,

endTransaction          BOOLEAN,

sequenceNumber          SequenceNumber      OPTIONAL,

acknowledgement         Acknowledgement     OPTIONAL,

slpp-MessageBody        SLPP-MessageBody    OPTIONAL

}

SequenceNumber ::= INTEGER (0..255)

Acknowledgement ::= SEQUENCE {

ackRequested        BOOLEAN,

ackIndicator        SequenceNumber        OPTIONAL

}

-- TAG-SLPP-MESSAGE-STOP

-- ASN1STOP

Editor's note	FFS on whether message header, e.g. transaction ID should be put in the SLPP-message level or same as RRC in corresponding message.

Editor's note	FFS on whether any positioning method specific capability IEs should be grouped by positioning method.

Editor's note	FFS on SLPP message header, e.g. cast type, session ID, UE ID

Editor's note	FFS on The SLPP ASN.1 design should allow for "selective ASN.1 compilation". The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module. A "group" may correspond to a positioning method, but other grouping may also be possible. An implementation needs to compile only the SLPP modules which contain a supported "group" (functionality, positioning method, etc.)

#### –	SLPP-MessageBody

The SLPP-MessageBody identifies the type of an SLPP message and contains all SLPP information specifically associated with that type.

-- ASN1START

-- TAG-SLPP-MESSAGEBODY-START

SLPP-MessageBody ::= CHOICE {

c1                          CHOICE {

requestCapabilities         RequestCapabilities,

provideCapabilities         ProvideCapabilities,

requestAssistanceData       RequestAssistanceData,

provideAssistanceData       ProvideAssistanceData,

requestLocationInformation  RequestLocationInformation,

provideLocationInformation  ProvideLocationInformation,

abort                       Abort,

error                       Error,

spare8 NULL, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension    SEQUENCE {}

}

-- TAG-SLPP-MESSAGEBODY-STOP

-- ASN1STOP

# Text proposal of selective ASN.1 compilation (refer to [15] and TS 38.355 v0.0.4)

# SLPP PDU Structure

#### –	SLPP-PDU-Definitions

This ASN.1 segment is the start of the SLPP PDU definitions.

-- ASN1START

-- TAG-SLPP-PDU-DEFINITIONS-START

SLPP-PDU-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

CommonIEsRequestCapabilities,

CommonIEsProvideCapabilities,

CommonIEsRequestAssistanceData,

CommonIEsProvideAssistanceData,

CommonIEsRequestLocationInformation,

CommonIEsProvideLocationInformation

FROM

SLPP-PDU-Common-Contents

Method-A-RequestCapabilities,

Method-A-ProvideCapabilities,

Method-A-RequestAssistanceData,

Method-A-ProvideAssistanceData,

Method-A-RequestLocationInformation,

Method-A-ProvideLocationInformation

FROM

SLPP-PDU-Method-A-Contents

Method-B-RequestCapabilities,

Method-B-ProvideCapabilities,

Method-B-RequestAssistanceData,

Method-B-ProvideAssistanceData,

Method-B-RequestLocationInformation,

Method-B-ProvideLocationInformation

FROM

SLPP-PDU-Method-B-Contents

Method-C-RequestCapabilities,

Method-C-ProvideCapabilities,

Method-C-RequestAssistanceData,

Method-C-ProvideAssistanceData,

Method-C-RequestLocationInformation,

Method-C-ProvideLocationInformation

FROM

SLPP-PDU-Method-C-Contents;

-- TAG-SLPP-PDU-DEFINITIONS-STOP

-- ASN1STOP

NOTE: 	An implementation needs to include only the supported "Method" PDUs. Not supported methods do not need to be included, and therefore, do not contribute to the protocol size. For example, if "Method-A" is not supported by an implementation, the SLPP-PDU-Method-A-Contents PDU does not need to be included in the protocol.

#### –	SLPP-Message

The SLPP-Message provides the complete set of information for an invocation or response pertaining to an SLPP transaction.

-- ASN1START

-- TAG-SLPP-MESSAGE-START

SLPP-Message ::=            SEQUENCE {

message                     SLPP-MessageType

}

SLPP-MessageType ::=        CHOICE {

c1                          CHOICE {

requestCapabilities         RequestCapabilities,

provideCapabilities         ProvideCapabilities,

requestAssistanceData       RequestAssistanceData,

provideAssistanceData       ProvideAssistanceData,

requestLocationInformation  RequestLocationInformation,

provideLocationInformation  ProvideLocationInformation,

abort                       Abort,

error                       Error,

spare8 NULL, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension    SEQUENCE {}

}

-- TAG-SLPP-MESSAGE-STOP

-- ASN1STOP

Editor's note	FFS on whether message header, e.g. transaction ID should be put in the SLPP-message level or same as RRC in corresponding message.

Editor's note	FFS on whether any positioning method specific capability IEs should be grouped by positioning method.

Editor's note	FFS on SLPP message header, e.g. cast type, session ID, UE ID

#### –	RequestCapabilities

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

c1                      CHOICE {

requestCapabilities    RequestCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

commonIEsRequestCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestCapabilities

method-A-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-A-RequestCapabilities

method-B-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-B-RequestCapabilities

method-C-RequestCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-C-RequestCapabilities

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	ProvideCapabilities

-- ASN1START

-- TAG-PROVIDECAPABILITIES-START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions      CHOICE {

c1                      CHOICE {

provideCapabilities     ProvideCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideCapabilities-IEs ::= SEQUENCE {

commonIEsProvideCapabilities         OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideCapabilities

method-A-ProvideCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-A-ProvideCapabilities

method-B-ProvideCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-B-ProvideCapabilities

method-C-ProvideCapabilities         OCTET STRING    OPTIONAL, -- Containing Method-C-ProvideCapabilities

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDECAPABILITIES-STOP

-- ASN1STOP

#### –	RequestAssistanceData

-- ASN1START

-- TAG-REQUESTASSISTANCEDATA-START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

c1                        CHOICE {

requestAssistanceData     RequestAssistanceData-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture  SEQUENCE {}

}

}

RequestAssistanceData-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData       OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestAssistanceData

method-A-RequestAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-A-RequestAssistanceData

method-B-RequestAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-B-RequestAssistanceData

method-C-RequestAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-C-RequestAssistanceData

nonCriticalExtension          SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	ProvideAssistanceData

-- ASN1START

-- TAG-PROVIDEASSISTANCEDATA-START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions        CHOICE {

c1                        CHOICE {

provideAssistanceData     ProvideAssistanceData-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideAssistanceData-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData       OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideAssistanceData

method-A-ProvideAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-A-ProvideAssistanceData

method-B-ProvideAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-B-ProvideAssistanceData

method-C-ProvideAssistanceData       OCTET STRING    OPTIONAL, -- Containing Method-C-ProvideAssistanceData

nonCriticalExtension          SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	RequestLocationInformation

-- ASN1START

-- TAG-REQUESTLOCATIONINFORMATION-START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

c1                             CHOICE {

requestLocationInformation     RequestLocationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

RequestLocationInformation-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation  OCTET STRING    OPTIONAL, -- Containing CommonIEsRequestLocationInformation

method-A-RequestLocationInformation  OCTET STRING    OPTIONAL, -- Containing Method-A-RequestLocationInformation

method-B-RequestLocationInformation  OCTET STRING    OPTIONAL, -- Containing Method-B-RequestLocationInformation

method-C-RequestLocationInformation  OCTET STRING    OPTIONAL, -- Containing Method-C-RequestLocationInformation

nonCriticalExtension               SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	ProvideLocationInformation

-- ASN1START

-- TAG-PROVIDELOCATIONINFORMATION-START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions             CHOICE {

c1                             CHOICE {

provideLocationInformation    ProvideLocationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

ProvideLocationInformation-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation    OCTET STRING    OPTIONAL, -- Containing CommonIEsProvideLocationInformation

method-A-ProvideLocationInformation    OCTET STRING    OPTIONAL, -- Containing Method-A-ProvideLocationInformation

method-B-ProvideLocationInformation    OCTET STRING    OPTIONAL, -- Containing Method-B-ProvideLocationInformation

method-C-ProvideLocationInformation    OCTET STRING    OPTIONAL, -- Containing Method-C-ProvideLocationInformation

nonCriticalExtension               SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	Abort

-- ASN1START

-- TAG-ABORT-START

Abort ::= SEQUENCE {

criticalExtensions    CHOICE {

c1                    CHOICE {

abort                 Abort-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

Abort-IEs ::= SEQUENCE {

nonCriticalExtension    SEQUENCE {}     OPTIONAL

}

-- TAG-ABORT-STOP

-- ASN1STOP

#### –	Error

-- ASN1START

-- TAG-ERROR-START

Error ::= CHOICE {

criticalExtensions    CHOICE {

c1                    CHOICE {

error                 Error-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    SEQUENCE {}

}

}

Error-IEs ::= SEQUENCE {

nonCriticalExtension    SEQUENCE {}     OPTIONAL

}

-- TAG-ERROR-STOP

-- ASN1STOP

#### –	End of SLPP-PDU-Definitions

-- ASN1START

END

-- ASN1STOP

# SLPP PDU Common Contents

#### –	SLPP-PDU-Common-Contents

-- ASN1START

SLPP-PDU-Common-Contents

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	CommonIEsRequestCapabilities

-- ASN1START

CommonIEsRequestCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	CommonIEsProvideCapabilities

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	CommonIEsRequestAssistanceData

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	CommonIEsProvideAssistanceData

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	CommonIEsRequestLocationInformation

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	CommonIEsProvideLocationInformation

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	End of SLPP-PDU-Common-Contents

-- ASN1START

END

-- ASN1STOP

# SLPP PDU Method-A Contents

#### –	SLPP-PDU-Method-A-Contents

-- ASN1START

SLPP-PDU-Method-A-Contents

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	Method-A-RequestCapabilities

-- ASN1START

Method-A-RequestCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-A-ProvideCapabilities

-- ASN1START

Method-A-ProvideCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-A-RequestAssistanceData

-- ASN1START

Method-A-RequestAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-A-ProvideAssistanceData

-- ASN1START

Method-A-ProvideAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-A-RequestLocationInformation

-- ASN1START

Method-A-RequestLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-A-ProvideLocationInformation

-- ASN1START

Method-A-ProvideLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	End of SLPP-PDU-Method-A-Contents

-- ASN1START

END

-- ASN1STOP

# SLPP PDU Method-B Contents

#### –	SLPP-PDU-Method-B-Contents

-- ASN1START

SLPP-PDU-Method-B-Contents

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	Method-B-RequestCapabilities

-- ASN1START

Method-B-RequestCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-B-ProvideCapabilities

-- ASN1START

Method-B-ProvideCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-B-RequestAssistanceData

-- ASN1START

Method-B-RequestAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-B-ProvideAssistanceData

-- ASN1START

Method-B-ProvideAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-B-RequestLocationInformation

-- ASN1START

Method-B-RequestLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-B-ProvideLocationInformation

-- ASN1START

Method-B-ProvideLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	End of SLPP-PDU-Method-B-Contents

-- ASN1START

END

-- ASN1STOP

# SLPP PDU Method-C Contents

#### –	SLPP-PDU-Method-C-Contents

-- ASN1START

SLPP-PDU-Method-C-Contents

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	Method-C-RequestCapabilities

-- ASN1START

Method-C-RequestCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-C-ProvideCapabilities

-- ASN1START

Method-C-ProvideCapabilities ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-C-RequestAssistanceData

-- ASN1START

Method-C-RequestAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-C-ProvideAssistanceData

-- ASN1START

Method-C-ProvideAssistanceData ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-C-RequestLocationInformation

-- ASN1START

Method-C-RequestLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	Method-C-ProvideLocationInformation

-- ASN1START

Method-C-ProvideLocationInformation ::= SEQUENCE {

}

-- ASN1STOP

#### –	End of SLPP-PDU-Method-C-Contents

-- ASN1START

END

-- ASN1STOP