| 3GPP TS 38.355 V0.0.4 (2023-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Sidelink Positioning Protocol (SLPP); Protocol specification (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Abbreviations	6

4	Functionality of Protocol	6

4.1	General	6

4.1.1	SLPP Configuration	6

4.1.2	SLPP Sessions and Transactions	6

4.1.3	SLPP Position Methods	6

4.1.4	SLPP Messages	6

4.2	Common SLPP Session Procedure	6

5	SLPP Procedures	7

5.1	Procedures related to capability transfer	7

5.2	Procedures related to Assistance Data Transfer	7

5.3	Procedures related to Location Information Transfer	7

5.4	Error Handling Procedures	7

5.5	Abort Procedure	7

6	Protocol data units, formats and parameters (ASN.1)	9

6.1	General	9

6.2	SLPP messages	9

6.2.1	General message structure	9

–	SLPP-PDU-Definitions	9

–	SLPP-Message	10

6.2.2	Message definitions	11

–	RequestCapabilities	11

–	ProvideCapabilities	11

–	RequestAssistanceData	12

–	ProvideAssistanceData	12

–	RequestLocationInformation	13

–	ProvideLocationInformation	13

–	Abort	14

–	Error	15

6.3	SLPP information elements	15

6.4	Multiplicity and type constraint values	16

–	End of SLPP-PDU-Definitions	16

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the Sidelink Positioning Protocol (SLPP) for the interface between UEs and between UE and LMF.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[3]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[4]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Field: The individual contents of an information element are referred to as fields.

Ranging: Refers to the determination of the distance between two UEs or more UEs and/or the direction of one UE (i.e. Target UE) from another UE via PC5 interface.

Anchor UE: A UE, supporting positioning of target UE, e.g. by transmitting and/or receiving reference signals for positioning, providing positioning-related information, etc. over the Sidelink interface.

Target UE: A UE whose distance, direction and/or position is measured with the support from one or multiple Anchor UEs using Sidelink in the Ranging based service and Sidelink positioning.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

LMF	Location Management Function

SL	Sidelink

SLPP	Sidelink Positioning Protocol

UE	User Equipment

# 4	Functionality of Protocol

## 4.1	General

### 4.1.1	SLPP Configuration

### 4.1.2	SLPP Sessions and Transactions

Editor’s note	FFS on the definition of SLPP Session.

### 4.1.3	SLPP Position Methods

This version of the specification defines SL-TDOA, SL-AoA and SL-RTT positioning methods.

Editor’s note	FFS on the supported positioning methods.

### 4.1.4	SLPP Messages

The following message types are defined:

-	Request Capabilities;

-	Provide Capabilities;

-	Request Assistance Data;

-	Provide Assistance Data;

-	Request Location Information;

-	Provide Location Information;

-	Abort;

-	Error.

## 4.2	Common SLPP Session Procedure

Editor's note	FFS on whether SLPP message Segmentation is needed.

Editor’s note	FFS on the need of reliable transport.

# 5	SLPP Procedures

Editor's note	The content of each section will be added in accordance with future agreements, not based on LPP legacy directly.

Editor's note	FFS on whether to add procedure description in the field description as LPP.

## 5.1	Procedures related to capability transfer

## 5.2	Procedures related to Assistance Data Transfer

## 5.3	Procedures related to Location Information Transfer

## 5.4	Error Handling Procedures

## 5.5	Abort Procedure

# 6	Protocol data units, formats and parameters (ASN.1)

## 6.1	General

The contents of each SLPP message is specified in clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax. The syntax of the information elements that are defined as stand-alone abstract types is further specified in a similar manner in clause 6.3.

Editor's note	FFS on Need code (e.g. how to support no UL/DL), support of delta signalling, full configuration, import IE from LPP, setup/release.

The ASN.1 in this clause uses the same format and coding conventions as described in Annex A of TS 38.331 [2].

Transfer syntax for SLPP messages is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [4]. The encoded SLPP message always contains a multiple of 8 bits.

Transfer syntax for SLPP IEs is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [4]. The encoded SLPP IE always contains a multiple of 8 bits. This applies when a single SLPP IE is encoded as the basic production, i.e. for other purposes than encoding the SLPP IE within an SLPP message.

When specifying information elements which are to be represented by BIT STRINGs, if not otherwise specifically stated in the field description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	the last bit (rightmost bit) contains the least significant bit (LSB).

## 6.2	SLPP messages

### 6.2.1	General message structure

#### –	SLPP-PDU-Definitions

This ASN.1 segment is the start of the SLPP PDU definitions.

-- ASN1START

-- TAG-SLPP-PDU-DEFINITIONS-START

SLPP-PDU-Definitions DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- TAG-SLPP-PDU-DEFINITIONS-STOP

-- ASN1STOP

#### –	SLPP-Message

The SLPP-Message provides the complete set of information for an invocation or response pertaining to an SLPP transaction.

-- ASN1START

-- TAG-SLPP-MESSAGE-START

SLPP-Message ::=            			SEQUENCE {

message						                     SLPP-MessageType

}

SLPP-MessageType ::=        		CHOICE {

c1                          							CHOICE {

requestCapabilities         			RequestCapabilities,

provideCapabilities         			ProvideCapabilities,

requestAssistanceData       		RequestAssistanceData,

provideAssistanceData       		ProvideAssistanceData,

requestLocationInformation  	RequestLocationInformation,

provideLocationInformation  	ProvideLocationInformation,

abort                       						Abort,

error                       						Error,

spare8 NULL, spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL, spare3 NULL, spare2 NULL, spare1 NULL

},

messageClassExtension    SEQUENCE {}

}

-- TAG-SLPP-MESSAGE-STOP

-- ASN1STOP

Editor's note	FFS on whether message header, e.g. transaction ID should be put in the SLPP-message level or same as RRC in corresponding message.

Editor's note	FFS on whether any positioning method specific capability IEs should be grouped by positioning method.

Editor's note	FFS on SLPP message header, e.g. cast type, session ID, UE ID

Editor's note	FFS on The SLPP ASN.1 design should allow for "selective ASN.1 compilation". The overall SLPP functionality is divided into "groups", where each group is defined as a separate ASN.1 module. A "group" may correspond to a positioning method, but other grouping may also be possible. An implementation needs to compile only the SLPP modules which contain a supported "group" (functionality, positioning method, etc.)

### 6.2.2	Message definitions

#### –	RequestCapabilities

-- ASN1START

-- TAG-REQUESTCAPABILITIES-START

RequestCapabilities ::= SEQUENCE {

criticalExtensions      		CHOICE {

c1                      						CHOICE {

requestCapabilities    		RequestCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

RequestCapabilities-IEs ::= SEQUENCE {

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTCAPABILITIES-STOP

-- ASN1STOP

#### –	ProvideCapabilities

-- ASN1START

-- TAG-PROVIDECAPABILITIES-START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions      		CHOICE {

c1                      						CHOICE {

provideCapabilities     		ProvideCapabilities-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

ProvideCapabilities-IEs ::= SEQUENCE {

nonCriticalExtension        SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDECAPABILITIES-STOP

-- ASN1STOP

#### –	RequestAssistanceData

-- ASN1START

-- TAG-REQUESTASSISTANCEDATA-START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions        		CHOICE {

c1                        						CHOICE {

requestAssistanceData     	RequestAssistanceData-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture  	SEQUENCE {}

}

}

RequestAssistanceData-IEs ::= SEQUENCE {

nonCriticalExtension          SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTASSISTANCEDATA-STOP

-- ASN1STOP

#### –	ProvideAssistanceData

-- ASN1START

-- TAG-PROVIDEASSISTANCEDATA-START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions        		CHOICE {

c1                        						CHOICE {

provideAssistanceData     	ProvideAssistanceData-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

ProvideAssistanceData-IEs ::= SEQUENCE {

nonCriticalExtension          SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDEASSISTANCEDATA-STOP

-- ASN1STOP

#### –	RequestLocationInformation

-- ASN1START

-- TAG-REQUESTLOCATIONINFORMATION-START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions             		CHOICE {

c1                             						CHOICE {

requestLocationInformation     	RequestLocationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

RequestLocationInformation-IEs ::= SEQUENCE {

nonCriticalExtension               SEQUENCE {}     OPTIONAL

}

-- TAG-REQUESTLOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	ProvideLocationInformation

-- ASN1START

-- TAG-PROVIDELOCATIONINFORMATION-START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions             		CHOICE {

c1                             						CHOICE {

provideLocationInformation 	   ProvideLocationInformation-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

ProvideLocationInformation-IEs ::= SEQUENCE {

nonCriticalExtension               SEQUENCE {}     OPTIONAL

}

-- TAG-PROVIDELOCATIONINFORMATION-STOP

-- ASN1STOP

#### –	Abort

-- ASN1START

-- TAG-ABORT-START

Abort ::= SEQUENCE {

criticalExtensions    		CHOICE {

c1                    						CHOICE {

abort                 					Abort-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

Abort-IEs ::= SEQUENCE {

nonCriticalExtension    SEQUENCE {}     OPTIONAL

}

-- TAG-ABORT-STOP

-- ASN1STOP

#### –	Error

-- ASN1START

-- TAG-ERROR-START

Error ::= CHOICE {

criticalExtensions    CHOICE {

c1                    CHOICE {

error                 					Error-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture    	SEQUENCE {}

}

}

Error-IEs ::= SEQUENCE {

nonCriticalExtension    SEQUENCE {}     OPTIONAL

}

-- TAG-ERROR-STOP

-- ASN1STOP

## 6.3	SLPP information elements

### 6.3.1	Common information elements

### 6.3.2	UE capability information elements

### 6.3.3	Positioning Method information elements

## 6.4	Multiplicity and type constraint values

#### –	End of SLPP-PDU-Definitions

-- ASN1START

END

-- ASN1STOP

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04/2023 | RAN2#121bis-e | R2-2302739 |  |  |  |  | 0.0.1 |
| 04/2023 | RAN2#121bis-e | R2-2304306 |  |  |  |  | 0.0.2 |
| 05/2023 | RAN2#122 | R2-2305439 |  |  |  |  | 0.0.3 |