3GPP TSG-RAN WG2 Meeting #123	R2-2307666

Toulouse, FR, 21st – 25th Aug, 2023

Agenda item:			7.2.5

Source:			Intel Corporation

Title:		Considerations on RAN1 led items

Document for:	 	Discussion and decision

# Introduction

At RAN#98, RAN plenary agreed the new WI on expanded and improved NR positioning [1] with following objectives led by RAN1:

- Specify support of positioning for UEs with Reduced Capabilities (RedCap UEs) - Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2]. - NOTE: The complexity of the corresponding capabilities for RedCap UEs should be addressed for the introduction of appropriate capabilities for RedCap UEs. - Specify RRM requirements for positioning including RRM measurements and procedures for RedCap UEs for both with and without frequency hopping [RAN4]. - Specify bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers [RAN1, RAN2, RAN4]. - Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference). - Specify RRM requirements with measurement gaps in connected mode, and in inactive mode, including PRS measurement period/reporting [RAN4]. - Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4]. - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL. - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

In addition, RAN1 will discuss PRU under carrier phase positioning agenda.

In this contribution, we continue the discussion on how to specify these objectives.

# Discussion

### 2.1 Support of positioning for UEs with Reduced Capabilities (RedCap UEs)

The objective is

- Specify support of Frequency Hopping (FH) beyond maximum RedCap UE bandwidth for reception of DL PRS and transmission of UL SRS for positioning [RAN1, RAN2].

During SI phase, RAN1 identified following potential impact:

From RAN1's perspective, the following have been identified for potential specification impact to support NR positioning for RedCap UEs: - Maximum tolerable phase error, timing gap, and timing error between hops. - Considerations for IIoT, commercial, Public Safety and V2X scenarios, and UE capabilities. - Details on the Tx or Rx hopping pattern(s), including frequency overlapping between hops, if supported.

From RAN2 perspective, the main work should be to capture RAN1 parameters and capabilities which can be done once there is reasonable progress in RAN1. RAN1 made lots of agreements in RAN1#112bis-e and RAN1#113. However it is still not stable enough for RAN2 to start the work since the details of parameters, capabilities, procedures are all FFS. Therefore RAN2 should postpone the discussion on positioning for RedCap UE until there is reasonable progress in RAN1.

Proposal 1: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for RedCap UE until there is reasonable progress in RAN1.

### 2.2 Bandwidth aggregation for positioning measurements across up to three intra-band contiguous carriers

The objective is

- Specify signalling and procedures to support aggregation of PRS/SRS (respectively) resources across PFLs/carriers (respectively) for positioning measurements under the assumption that the signals over aggregated resources are transmitted and received (respectively) using a single RF chain (same antenna) [RAN1, RAN2]. - NOTE: The support of bandwidth aggregation for positioning measurements applies only to timing related measurements (e.g., RSTD, RTOA, and UE/gNB Rx-Tx time difference).

As captured in the TR [2],

From the perspective of Radio Resource Management (RRM), the following are assumed for PRS bandwidth aggregation: - A common numerology is required across all intra-band contiguous PFLs to be aggregated. - PRS resources to be aggregated from different PFLs can have different bandwidths (i.e., different number of PRS RBs). - PRS resources to be aggregated from different PFLs are transmitted in the same slot and in the same symbols. - PRS resources to be aggregated from different PFLs are transmitted by the same TRP and associated with a common Antenna Reference Point (ARP). From RRM perspective, the following are assumed for SRS bandwidth aggregation: - A common numerology is required across all intra-band contiguous carriers to be aggregated. - SRS resources to be aggregated from different carriers can have different bandwidths (i.e., different number of SRS RBs). - SRS resources to be aggregated from different carriers are transmitted in the same slot and in the same symbols.

It is unclear whether the bandwidth aggregation is similar to CA, i.e. based 3 intra-band contiguous carriers (3 cells). That is:

- For PRS, 3 cells (intra-band contiguous carriers ) send the PRS in the aggregated bandwidth;

- For SRS, UE sends the SRS in 3 cells  (intra-band contiguous carriers ) at the same time.

Based on RAN1 agreements below, looks like the answer is yes.

Agreement For PRS bandwidth aggregation across PFLs, support enhancement of PRS configuration to inform UE by LMF (or inform LMF by NG-RAN) PRS resources from which two or three PFLs are linked. Agreement For SRS bandwidth aggregation across two or three carriers, support enhancement of SRS configuration to indicate the SRS resources from which two or three carriers are linked

If yes, it will have significant impact on RAN2 work, e.g. RAN2 needs to consider:

- Do we need to introduce new assistance data structure to support PRS/SRS in aggregated bandwidth?

- The relationship with CA, should CA be the pre-condition for aggregated bandwidth? If yes, how to trigger the UE in intra-band contiguous carriers CA case? Or the LMF only use this feature when UE has been configured with intra-band contiguous carriers CA?

Based on RAN1 agreements, they are also discussing the relationship with CA, see

Agreement Study the relationship between UL communication CA and SRS bandwidth aggregation, including - Whether to support the decoupling of the SRS bandwidth aggregation and the communication carrier aggregation for UE capabilities - Whether to support the configuration of SRS BW aggregation not limited by the allowed configuration of communication CA, i.e., SRS outside BWP and across carriers

At RAN1#112bis-e. RAN1 further agreed:

Agreement At least from UE capability perspective, the UE support of positioning SRS bandwidth aggregation in RRC_CONNECTED state is decoupled from the UE support of communication CA.

To our understanding, it means the UE may support SRS bandwidth aggregation even if CA is not configured or not supported.

In addition, RAN1 agreed to leave the details of on-demand PRS on PRS bandwidth aggregation to RAN2 and RAN3. RAN2 should work on the details of it.

Conclusion The details for on-demand PRS on PRS bandwidth aggregation are up to RAN2 and RAN3.

From RAN2 perspective, the main work should be to capture RAN1 parameters, capabilities which can be done once there is reasonable progress in RAN1. RAN1 made lots of agreements in RAN1#112bis-e/RAN1#113. However, it is still not stable enough for RAN2 to start the work since the details of parameters, capabilities, procedures are all FFS. Therefore, RAN2 should postpone the discussion on positioning for Bandwidth aggregation until there is reasonable progress in RAN1.

Proposal 2: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for Bandwidth aggregation until there is reasonable progress in RAN1.

However, RAN1 sent LS [5], and asked whether their working assumption is feasible from RAN2 perspective

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources. Send an LS to RAN2 to confirm the feasibility.

Considering we already supported MAC CE to activate PPW and MG, therefore it should be feasible to use MAC CE to activate/deactivate SRS resources set in one or two or three aggregated carriers

Proposal 3: Reply RAN1 that it is feasible to use MAC CE to activate/deactivate SRS resources set in one or two or three aggregated carriers.

### 2.3 Support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning

The objective is:

- Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements. - Specify measurements that are limited to a single carrier/PFL.

Based on the objective, if existing DL PRS and UL SRS are reused, RAN2 does not need to introduce new assistance information. However, RAN1 has agreed to introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. RAN1 has so far agreed to only support reporting of CP measurements along with existing positioning methods, therefore RAN2 only need to add corresponding parameters/measurements to existing positioning methods. RAN1 also agreed to support the reuse of existing physical layer procedures for UL/DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR UL/DL carrier phase positioning. In addition, RAN1 agreed to enable simultaneous measurements on same DL PRS by a target UE and a PRU, corresponding procedure should be introduced.

RAN1 made lots of agreements in RAN1#112bis-e and RAN1#113. However, it is still not stable enough for RAN2 to start the work since the procedure details, measurement details are all FFS. Therefore, it is difficult for RAN2 to start the discussion before knowing the details and it would be good to avoid duplicated discussion. Therefore, we propose:

Proposal 4: Potential RAN2 impact is to capture RAN1 parameters, feature lists, new configuration and corresponding measurements, procedure based on existing positioning method. Postpone the discussion on Carrier phase positioning until there is reasonable progress in RAN1.

# Conclusion

Based on the discussion, we have following proposals:

Proposal 1: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for RedCap UE until there is reasonable progress in RAN1.

Proposal 2: Potential RAN2 impact is to capture RAN1 parameters and feature lists. Postpone the discussion on positioning for Bandwidth aggregation until there is reasonable progress in RAN1.

Proposal 3: Reply RAN1 that it is feasible to use MAC CE to activate/deactivate SRS resources set in one or two or three aggregated carriers.

Proposal 4: Potential RAN2 impact is to capture RAN1 parameters, feature lists, new configuration and corresponding measurements, procedure based on existing positioning method. Postpone the discussion on Carrier phase positioning until there is reasonable progress in RAN1.

# Reference

[1] RP-223549 New WID on Expanded and Improved NR Positioning

[2] TR 38.859

[3] R2-2301939	LS on PRU procedures (S2-2303861; contact: Qualcomm)	SA2

[4] R2-2302048/R1- 2302146 LS Reply on PRU Procedures, RAN1

[5] R1-2306214 LS to RAN2 on SRS bandwidth aggregation for positioning