3GPP TSG-RAN WG2 Meeting #123											   R2-2307669

Toulouse, France, 21-25 Aug, 2023 				    				revision of R2-2305574

Source: 	Xiaomi

Title:  	Remaining issues of RRC configured Layer-2 reset

Agenda Item:	7.4.2.2

Document for:	Discussion and Decision

# Introduction

According to the RAN2#121 meeting discussion, RAN2 made the following agreements on the L2 reset configuration:

- To determine if to reset L2 or not is based on RRC configuration (e.g. set of cells. FFS if separate for RLC, MAC, PDCP).

In this contribution, we provide our analysis on the remaining issues related to the L2 reset configured by RRC, via set of cells.

# Discussion

## L2 reset configuration

Although the Rel-18 mobility item is only to support intra-CU case, we consider that it is possible that the ASN.1 framework for the L2 reset configuration of LTM can be applicable for all mobility cases as illustrated below, including:

- intra-DU

- inter-DU intra-CU

- inter-CU

Figure 1: L2 reset procedure controlled by RRC

Firstly, we think that the UE does not need to decode the cell information (e.g. PCI or CGI) from the candidate configuration when determining whether to reset L2 or not, as configuring a set of candidate configurations would be sufficient to cover all mobility scenarios as provided above. The following are extracted from the running 38.331 CR [2] for the L2 reset configuration:

ltm-ServingCellNoResetID This field is used by the UE to determine on whether L2 reset should be performed for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in an LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during the LTM cell switch procedure.

ltm-NoResetID This field indicate whether the UE should perform no L2 reset for an LTM candidate cell upon an LTM cell switch procedure. If the value of ltm-NoResetID in the LTM candidate cell is the same as the value of ltm-ServingCellNoResetID in the serving cell of a cell group, then the UE shall not perform any L2 reset during an LTM cell switch procedure.

Editor’s Note: FFS on whether the LTM-CandidateNoResetL2-List field should include separate reset flags for RLC, and PDCP recovery.

Observation 1: As captured in the running 38.331 CR, a set ID per candidate configuration is configured to determine whether to reset L2.

The following are extracted from the running 38.331 CR [2] for the UE behaviours while comparing the L2 reset IDs between the source cell and the target cell:

1> reset the MAC entity of this cell group; 1> if the value of field ltm-NoResetID contained within the LTM-Candidate IE related to the LTM candidate cell configuration identity as received by lower layers is equal to the value of ltm-ServingCellNoResetID in current serving cell for this cell group: 2> continue using the current RLC entity in the LTM candidate cell configuration indicated by lower layers; 2> replace the value of ltm-ServingCellNoResetID for this cell group with the value received within ltm-NoResetID; 1> else: 2> for each RLC-BearerConfig within rlc-BearerToAddModList that is part of current UE configuration: 3> re-establish the RLC entity as specified in TS 38.322 [4]; 2> for each drb-Identity value included in the drb-ToAddModList that is part of the current UE configuration: 3> trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5]; Editor’s Note: The handling of the MAC and RLC entity is still FFS as it depends on how the L2 reset is indicated by the network. Editor’s Note: FFS on how to capture UE actions when the L2 reset is needed.

Observation 2: As captured in the running 38.331 CR, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to the same candidate configuration set, it is unclear whether the UE continues using the PDCP entity.

Observation 3: As captured in the running 38.331 CR, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to different candidate configuration sets, the UE always triggers RLC reestablishment and PDCP recovery.

For the intra-DU mobility, we think that the UE can keep RLC and PDCP at cell switch. The RLC reestablishment and PDCP recovery is only required when the bearer type is changed from CA split bearer (for PDCP duplication) to non-split bearer. However, we consider that the bearer type should not change frequently as the QoS requirements of different services would not change.

For the inter-DU intra-CU mobility and the inter-CU mobility, the MAC/RLC reset is any required. Whether to perform RLC reestablishment or PDCP recovery/reestablishment can be based on the legacy RRC configuration (i.e. reestablishRLC for RLC and reestablishPDCP/recoverPDCP for PDCP) when the candidate configuration of the target cell and the candidate configuration of the source cell belongs to different candidate configuration sets. Then the legacy L2 UE behaviours for mobility can be reused.

Proposal 1: Separate configuration for RLC/PDCP reset is not needed.

Thus, at cells switch, if the candidate configuration of the target cell and the candidate configuration of the source cell belongs to the same candidate configuration set, the UE applies MAC reset and keeps RLC and PDCP. If the candidate configuration of the target cell and the candidate configuration of the source cell belongs to different candidate configuration sets, the L2 behaviours of the UE follows the legacy handover procedure.

Proposal 2: At cell switch, if the candidate configuration of the target cell and the candidate configuration of the source cell belongs to the same candidate configuration set, the UE keeps RLC and PDCP.

Proposal 3: At cell switch, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to different candidate configuration sets, the L2 behaviours of the UE follows the legacy handover procedure, i.e. RLC reestablishment / PDCP recovery is configured by legacy RRC fields of reestablishRLC for RLC and recoverPDCP for PDCP.

# Conclusions

According to the analysis given above, we have the following observations and proposals:

Observation 1: As captured in the running 38.331 CR, a set ID per candidate configuration is configured to determine whether to reset L2.

Observation 2: As captured in the running 38.331 CR, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to the same candidate configuration set, it is unclear whether the UE continues using the PDCP entity.

Observation 3: As captured in the running 38.331 CR, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to different candidate configuration sets, the UE always triggers RLC reestablishment and PDCP recovery.

Proposal 1: Separate configuration for RLC/PDCP reset is not needed.

Proposal 2: At cell switch, if the candidate configuration of the target cell and the candidate configuration of the source cell belongs to the same candidate configuration set, the UE keeps RLC and PDCP.

Proposal 3: At cell switch, if the candidate configuration of the target cell and the candidate configuration of the source cell belong to different candidate configuration sets, the L2 behaviours of the UE follows the legacy handover procedure, i.e. RLC reestablishment / PDCP recovery is configured by legacy RRC fields of reestablishRLC for RLC and recoverPDCP for PDCP.

# Reference

[1] RAN2#121 meeting minutes.

[2] [Post122][055][Mob18] 38.331 Running CR and Open issues (Ericsson)