3GPP TSG-RAN WG2 Meeting #123											    R2-2307671

Toulouse, France, 21-25 Aug, 2023

Source: 	Xiaomi

Title:  	Clarification on the TCI state indicated in the LTM MAC CE

Agenda Item:	7.4.2.3

Document for:	Discussion and Decision

# Introduction

According to the RAN1/RAN2 discussion on the content in the cell switch MAC CE, the TCI-state of the target cell can be indicated in the cell switch MAC CE. In this contribution, we provide some clarifications on how to use the TCI-state indicated in the MAC CE for the RACH-based LTM procedure.

# Discussion

## RO selection for RACH-based LTM

RAN2#122 meeting agreements: - Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

According to the RAN2#122 meeting agreements quoted above, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM Cell Switch MAC CE. According to the RAN1 agreements, the beam indication is always included in the LTM Cell Switch MAC CE for Scenario 2. However, it is possible that for Scenario 2 or other scenarios, the gNB could use the RACH-based LTM procedure, while indicating the beam in the LTM Cell Switch MAC CE. It is also possible that the UE not supporting the RACH-less LTM can be indicated with the beam in the LTM Cell Switch MAC CE.

Observation 1: The RACH-based LTM can have the LTM Cell Switch MAC CE indicating the beam for the target cell.

38.321: ### 5.1.2 Random Access Resource selection … 1> else if the contention-free Random Access Resources associated with SSBs have been explicitly provided in rach-ConfigDedicated and at least one SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs is available: 2> select an SSB with SS-RSRP above rsrp-ThresholdSSB amongst the associated SSBs; 2> set the PREAMBLE_INDEX to a ra-PreambleIndex corresponding to the selected SSB. … 1> else (i.e. for the contention-based Random Access preamble selection): 2> if at least one of the SSBs with SS-RSRP above rsrp-ThresholdSSB is available: 3> select an SSB with SS-RSRP above rsrp-ThresholdSSB. 2> else: 3> select any SSB.

However, according to the current MAC specification as quoted above, if the RACH procedure is triggered, the UE would select any SSB with RSRP above the configured threshold. Then the beam indicated by the LTM Cell Switch MAC CE would not be used by the UE for the RACH-based LTM procedure. As such, we consider that the UE behaviour for the RACH-base LTM procedure can be aligned with the UE behaviour for the RACH-less LTM procedure. This means that the RO selected for the RACH-based LTM should be associated with beam indicated by the LTM Cell Switch MAC CE.

Observation 2: The current RACH procedure ignores the beam indicated by the LTM Cell Switch MAC CE.

Proposal: For the RACH-based LTM procedure, the UE selects the RO associated with the SSB indicated in the “LTM Cell Switch Command MAC CE”.

# Conclusion

According to the above analysis, we have the following observations and proposals.

Observation 1: The RACH-based LTM can have the LTM Cell Switch MAC CE indicating the beam for the target cell.

Observation 2: The current RACH procedure ignores the beam indicated by the LTM Cell Switch MAC CE.

Proposal: For the RACH-based LTM procedure, the UE selects the RO associated with the SSB indicated in the “LTM Cell Switch Command MAC CE”.

# Reference

[1] 3GPP TS 38.331, “NR; Radio Resource Control (RRC) protocol specification”

# Annex

The followings are RAN1 agreements related to the TCI-state indication for LTM procedure:

RAN1#112 meeting agreements:

Agreement - At least for Rel-17 unified TCI framework based beam indication included in cell switch command (i.e. scenario 2), beam indication applies to signals/channels that follow or are configured to follow Rel-17 unified TCI at the target cell(s) - FFS: beam indication for mTRP case Agreement · Adopt Alt.2 for beam indication of target cell(s) and TCI state activation for candidate cell(s) (if supported) , o Alt. 1: By indicating RS identifier, i.e. mapping between RS identifier and Rel-17 unified TCI state is done by a UE o Alt. 2: By indicating Rel-17 TCI state index Agreement From RAN1 point of view, at least the following information can be included in the cell switch command, which is conveyed by MAC CE - Information to identify the target cell(s) - The details including bit number are designed by RAN2 - TA related information (details up to the discussion in A.I. 9.10.2) - 1 joint or 1 pair of UL and DL unified TCI State index for the target Cell - Note: discussion on target SpCell is not precluded - Active DL and UL BWPs for the target cell - FFS: Triggering of aperiodic TRS transmitted from the target cell - FFS: Triggering the CSI acquisition of the target cell and reporting to the target cell - FFS: Triggering of aperiodic SRS transmission to the target cell · FFS: C-RNTI FFS: the presence of each field (i.e. always present or configurable) Agreement For the Rel-17 unified TCI based beam indication in Rel-18 LTM, at least Alt 1 is supported: - Alt 1: TCI state activation of a candidate cell is received before the reception of beam indication of the candidate cell, - Alt 2: TCI state activation of a candidate cell is received together with the reception of beam indication of the candidate cell - FFS: signalling details for TCI state indication, if both activation and indication are done in the same MAC CE message carrying switch command - Alt 3: Alt 1 and/or Alt 2 can be supported based on the UE capability FFS: signalling details for TCI state activation FFS: For Alt 1, whether/how TCI state activation for candidate cell(s) is allowed Note: If scenarios 1 and 3 are to be supported other beam indication/TCI activation timing relationships are not precluded.

RAN1#112bis-e meeting agreements:

Working Assumption On the presence of beam indication within cell switch command, at least for scenario 2, following is supported: · A field to indicate 1 joint or 1 pair of UL and DL unified TCI State index for the target cell field is always present in the cell switch command. Note: If scenarios 1 and 3 are agreed to be supported in R18 LTM other solutions may be considered.

RAN1#113 meeting agreements:

Conclusion For R18 LTM, in order to activate multiple joint TCI state or/and pair of (DL/UL) TCI states for candidate cell case, do not support TCI state activation together with beam indication of the candidate cell in the same MAC-CE message. · FFS: UE assumption on the active TCI states other than the indicated TCI state after the reception of the cell switch command. Agreement A UE can be indicated and activated a single joint TCI state or a pair of UL/DL TCI state in the cell switch command.