3GPP TSG-RAN WG2 Meeting #123	 R2-2307687

Toulouse, France, August 21-25, 2023

Source:	KDDI Corporation

Title:	Discussions on remaining issues for RACH-less LTM

Tdoc Type:	Discussion

Agenda Item:	7.4.2.3 - L2 centric parts

Document for:  Discussion and decision

Release:       Rel-18

1	Introduction

In this contribution, we will discuss and share our views on the remaining issues for RACH-less LTM.

2	Discussion

RAN2#121b-e (April 2023) agreement

R2 assumes RRCReconfigurationComplete message is always sent at each LTM execution. In RACH-based LTM, the target cell is aware of the UE’s arrival based on the reception of preamble in CFRA and on the reception of Msg3/MsgA in CBRA, like the legacy HO. In RACH-less LTM, the target cell is aware of the UE’s arrival based on reception of the first UL transmission from this UE In RACH-less LTM, RRCReconfigurationComplete can be the content of the first UL MAC PDU/transmission to indicate UE arrival, i.e. no need to introduce any new signaling to indicate UE arrival (for the MCG-switch case) For RACH-based LTM, the UE considers that LTM execution procedure is successfully completed when the RACH is successfully completed. For RACH-less LTM, the UE considers that LTM execution procedure is successfully complete when the UE determines the NW has successfully received its first UL data.

RAN2#122 (Incheon, Korea, May 2023), agreement

Dynamic grant can be used for RACH-less LTM, for the first UL data transmission to the target cell: - - the UE monitors PDCCH for dynamic scheduling from the target cell, upon LTM cell switch. - - upon cell switch decision, R2 assumes that the source DU informs the target DU about the selected beam, so that the target DU can start scheduling dynamic UL grant. Configured grant can be used for RACH-less LTM, for the first UL data transmission to the target cell, the UE selects the configured grant occasion, which is associated with the beam indicated in the LTM MAC CE (as set by source cell). FFS further optimization

In RAN2#121b-e (April 2023), RAN2 agreed that RRCReconfigurationComplete can be the first transmission to indicate UE arrival to the target cell. And in RAN2#122 (Incheon, Korea, May 2023), RAN2 also agreed that UL grant mechanism for the first transmission for the cases both dynamic grant and configured grant. Based on those agreements, we propose the followings

Proposal1: RAN2 agree that UE is supposed to transmit RRCReconfigurationComplete (potentially BSR can be transmitted as well) using UL grant provided by DCI on PDCCH for dynamic grant case.

Proposal2: RAN2 agree that UE is supposed to transmit RRCReconfigurationComplete (potentially BSR can be transmitted as well) using configured grant occasion included in candidate cell configuration provided by source cell for configured grant case.

Figure: RACH-less LTM (Dynamic grant)

Regarding how UE recognizes successful completion of LTM cell switch, we prefer the same mechanism as LTE RACH-less HO, gNB send UE Contention Resolution Identity MAC CE as an explicit acknowledge of the UL first transmission rather than just the reception of target cell PDCCH addressed to the UE’s C-RNTI. This is also discussed in [Post122][058][Mob18] Contents of Cell Switch MAC CE (Huawei) 2.8 LTM completion determination, so what makes UEs stop the LTM supervisor timer is up to the email discussion, but if RAN2 agree option 3 (UE Contention Resolution identify MAC CE) as LTM completion determination, then RAN2 can agree UE stops the timer upon successful reception of UE Contention Resolution identify MAC CE.

Proposal3: RAN2 agree that UE stops the LTM supervisor timer, upon successful reception of UE Contention Resolution Identity MAC CE (same mechanism as LTE RACH-less HO)

LTE RACH-less HO TS36.321

- if the MAC entity is configured with rach-Skip or rach-SkipSCG and a UE Contention Resolution Identity MAC control element for this TTI has been received on the PDSCH indicated by the PDCCH of the SpCell addressed to the C-RNTI: - indicate to upper layer the successful reception of a PDCCH transmission addressed to the C-RNTI.

LTE RACH-less HO TS36.331

1> if MAC successfully completes the random access procedure; or 1> if MAC indicates the successful reception of a PDCCH transmission addressed to C-RNTI and if rach-Skip is configured: 2> stop timer T304;

As for LTM supervisor timer value we think that it should be configurable considering T304 timer value for handover is configurable, and the value is included in the RRCReconfiguration of the target cell.

Proposal4: Timer value for LTM supervisor timer is included in the RRCReconfiguration generated by the target cell.

3	Conclusion

We make the following observations and proposals:

Proposal1: RAN2 agree that UE is supposed to transmit RRCReconfigurationComplete (potentially BSR can be transmitted as well) using UL grant provided by DCI on PDCCH for dynamic grant case.

Proposal2: RAN2 agree that UE is supposed to transmit RRCReconfigurationComplete (potentially BSR can be transmitted as well) using configured grant occasion included in candidate cell configuration provided by source cell for configured grant case.

Proposal3: RAN2 agree that UE stops the LTM supervisor timer, upon successful reception of UE Contention Resolution Identity MAC CE (same mechanism as LTE RACH-less HO)

Proposal4: Timer value for LTM supervisor timer is included in the RRCReconfiguration generated by the target cell.

3	Reference

[1] RP-231311	Status Report: Further NR mobility enhancements	Rapporteur (MediaTek Inc., Apple)