3GPP TSG-RAN WG2 Meeting #123                 	R2-2307690

Toulouse, France, 21th – 25th August, 2023

Agenda Item:	7.17.2

Source: 	vivo

Title:         	Early indication for MUSIM temporary capability restriction

Document for: 	Discussion and Decision

# Introduction

In Rel-18 MUSIM WI, the below objective was agreed in [1].

1. Enhancements for MUSIM procedures to operate in RRC_CONNECTED state simultaneously in NW A and NW B. [RAN2, RAN3, RAN4]. - Specify mechanism to indicate preference on temporary UE capability restriction and removal of restriction (e.g. capability update, release of cells, (de)activation of configured resources) with NW A when UE needs transmission or reception (e.g., start/stop connection to NW B) for MUSIM purpose - RAT Concurrency: Network A is NR SA (with CA) or NR DC. Network B can either be LTE or NR. - Applicable UE architecture: Dual-RX/Dual-TX UE The work item shall identify whether the WI will have RAN3 or RAN4 impacts by RAN#99 [RAN2].

In RAN2#121bis meeting, early indication was agreed to support temporary UE capability restriction indication during RRC setup/resume procedure with the below FFSs:

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

This paper will discuss the above FFSs.

# Discussion

Early indication during RRC resume procedure.

Figure 1: Early indication for MUSIM temporary capability restriction during RRC resume procedure.

When the UE is already in RRC_CONNECTED in NETWORK B, the UE in NETWORK A in RRC_INACTIVE may start RRC resume with partial UE capabilities. As RRC resume message may configure DC/CA configuration which may exceed the UE’s current capabilities, it would be better for the UE to indicate its capabilities are in constrained state before RRC resume message. There are two options for early temporary capability restriction indication:

- Option 1: Use the spare bit in RRC resume request/RRC resume request1.

- Option 2: Introduce two LCIDs for CCCH and CCCH1 for temporary capability restriction indication.

For option 1, the main concern is using the only remaining 1 spare bit for supporting this feature. Moreover, the network cannot know the true cause of RRC connection resume, which may be unexpected at the network side for access control. For option 2, the UE can use two dedicated LCIDs for CCCH / CCCH1 to indicate to the network A its capabilities are in constrained state. Compared to option 1, option 2 can cover RRC setup case by dedicated LCID for CCCH. Similar mechanism has been used for early identification for a RedCap UE. Given above, we have the following proposal:

- The UE can indicate capability restriction during RRC resume procedure via the following options:

- Option 1: Use the spare bit in RRC resume request/RRC resume request1.

- Option 2: Introduce two LCIDs for CCCH and CCCH1 for temporary capability restriction indication.

When the network A receives the early temporary capability restriction indication, the network A will not configure DC/CA configuration and can configure the UE to report the detailed capability restriction information via UE Assistance Information in RRC Resume message.

- The network can configure the UE to report capability restriction information via UAI in RRC Resume message.

Early indication during RRC setup procedure.

Figure 2: Early indication for MUSIM temporary capability restriction during RRC setup procedure.

For RRC connection setup, if the option 2 of proposal 1 is not agreed, the UE can report UE’s capability restriction status in RRC setup complete message. Because the RRC setup message is only used for the establishment of SRB1 and the UE will receive the first RRC configuration after AS security is activated or ongoing. When the network A receives the early temporary capability restriction indication, the temporary capability restriction reporting for UE in RRC_CONNECTED can be reused.

- If the option2 of proposal 1 is not agreed, the UE can indicate capability restriction in RRC setup complete.

Because the network may not support early indication for MUSIM capability restriction indication during RRC resume/RRC setup procedure, the network may not be able to compile RRC resume request or obtain the early indication successfully. Therefore, the UE can be configured by the network via system information whether the UE is allowed to report capability restriction during RRC resume /RRC setup procedure. If not allowed, the UE will perform RRC resume/RRC setup procedure as legacy. And it is left to UE implementation on how to ensure the RRC resume/RRC setup procedure, e.g., switch the capability back to the network A.

- The UE can be configured by the network via system information whether the UE is allowed to report capability restriction during RRC resume /RRC setup procedure.

# Conclusion

In this paper, the following proposals are given:

- The UE can indicate capability restriction during RRC resume procedure via the following options:

- Option 1: Use the spare bit in RRC resume request/RRC resume request1.

- Option 2: Introduce two LCIDs for CCCH and CCCH1 for temporary capability restriction indication.

- The network can configure the UE to report capability restriction information via UAI in RRC Resume message.

- If the option2 of proposal 1 is not agreed, the UE can indicate capability restriction in RRC setup complete.

- The UE can be configured by the network via system information whether the UE is allowed to report capability restriction during RRC resume /RRC setup procedure.

# References

[1] RP-223492, Revised WID: Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo.

# Appendix

## RAN2#121bis Agreements

- Consider “proactive” approach (wherein the UE can request capability restrictions which can be independent of current RRC configuration if allowed by the NW) to MUSIM capability restrictions in addition to the reactive approach (which has been agreed previously). Such a mechanism shall still be under NW control, i.e. it is up to network whether to allow such signalling. FFS on the details – should aim for a common framework for the reactive and proactive approach. FFS on UE capabilities

- Support “early indication” from UE to network during RRC connection setup/resume procedure.

- FFS how to indicate this and in which message. The indication will tell network that UE capabilities are temporarily restricted.

- FFS on details (i.e. when UE can indicate this, what does it indicate, how does it relate to UAI, etc.)

- No consensus to support UE-initiated SCell deactivation for MUSIM in Rel-18.

- 1: For Rel-18 MUSIM dual active operation, the maximum MIMO layer may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 3: For Rel-18 MUSIM dual active operation, the measurement gap requirement may be changed and the change can be indicated to the NW. FFS if this is only for NW A or also NW B.

- 4: For Rel-18 MUSIM dual active operation, the measurement gap requirement change is reported for each serving cells, and for target bands or all supported NR bands depending on whether target bands are configured by the NW. FFS on whether the reporting can reuse the current needForGapInfoNR in RRC reconfiguration complete or extend the similar function in UAI.  FFS if this is only for NW A or also NW B.

- 8: The maximum UL power may be changed due to Rel-18 MUSIM dual active operation, but there is no need to introduce any new UE behavior for reporting this change.

- 6: UE can explicitly request specific serving cells or serving cell group to be released for Rel-18 MUSIM purpose. FFS how/whether this works for the proactive case.

- 9: RAN2 should avoid duplicating all the capabilities that UE reports via the UECapabilityInformation in the UAI for R18 MUSIM purpose.

- RAN2 can discuss P2, P5 and P7 from R2-2304397 during RAN2#123.

- RAN2 will aim to address the RAN4 LS in Rel-18 signalling. Should discuss how to handle Rel-17 gaps without priority (e.g. lowest, highest, network-decided somehow, etc.). Handled in email [231]

- 1: Introduce 1 optional per-UE capability bit (with

- out xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17.

- 2: Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI.

- 4: The existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap.

- 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

## RAN2#121 Agreements

- A2a: When the UE is in Connected mode in two NR networks, it is up to the UE implementation to select which NW to perform signaling for temporary UE capability restrictions.

- A2b: When the UE is in Connected mode in NR NW A and moving from Idle/Inactive to connected mode in NR NW B, the signaling for temporary UE capability restrictions can happen on NW A. FFS how to handle if UE is moving from IDLE/INACTIVE in NW A and is in CONNECTED with NW B.

- A2c: When the UE is in Connected mode in both networks and one is E-UTRAN, the signaling for temporary UE capability restrictions happens on the NR network.

- A3: The UE will request a temporary capability restrictions (e.g. via UAI) only after the NW signals via RRC that this is allowed. FFS whether the UE can indicate if it is already connecting with reduced capabilities during connection set-up/resume.

- A4: RAN2 to discuss whether prohibit timer is needed for the signaling of temporary UE capability restrictions This can wait until after progress is made on the signaling framework.

- A1: UAI can be used for the signaling of temporary UE capability changes for dual-active MUSIM. FFS if we have additional signalling (depends on e.g. SCell/SCG deactivation usability for MUSIM)

- A8: For dual-active MUSIM, at least the following type of UE capabilities can be expected to be impacted:

- •	Transmission and reception capabilities (e.g. MIMO layers)

- •	Measurement capabilities (e.g. gaps)

- •	Supported bandwidth

- •	Supported band-combinations (FFS whether this is CA or DC or both)

- FFS what is the granularity of reported temporary UE capability restrictions (also pending the band conflict discussion).

- FFS whether UE reports some or all of the above or whether we can do something simpler

- A6: For dual-active MUSIM, UE signaling will support the request for release (and reversal) of SCells and SCG. The signaling details (e.g. implicit or explicit request of each SCell or SCG) is FFS. FFS if we support deactivation (based on discussion in which case it can be used). It is up to network how to react to UE request.

- RAN2 does not intend to create new procedures for e.g. SCell/SCG deactivation for MUSIM purposes in Rel-18. Existing procedures can be used based on NW choice.

- B4: RAN2 considers the only RAN3 impact may be to support the UE request of SCG/SCell release via SRB3 (if supported) for MUSIM purpose (e.g. cause value).  If this can be done via inter-node messages, RAN2 expects no RAN3 impacts.

- 1: The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

- Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

- RAN2 confirms that the band conflict scenarios will be covered by the temporary UE capability restrictions. FFS on signalling details.

## 5.2 RAN2#119bis Agreements

The R18 MUSIM solution should work in DC/CA and RAN sharing scenarios (but need not be optimized for RAN sharing). RAN2 aims to address at least the Scenario 1: the UE in network A in RRC_CONNECTED indicates (i.e. adds/removes) its preference on temporary UE capability due start/stop connection in NW B. This can be e.g. CA/DC capability restriction. 2 The following is assumed when defining the solution: - The two networks are independent (i.e. no inter-network communication); - The Core Network is not aware of the temporary restrictions of the UE capability; 1: RAN2 can discuss NW A MN-SN coordination of Rel-18 MUSIM temporary capability restrictions due to UE being configured with NR-DC in NW A. RAN2 thinks MN-SN coordination for Rel-17 MUSIM gaps requires WI clarification in RAN RAN2 needs to discuss which UE capabilities can be impacted by sharing of resources between the MUSIM links. RAN2 aims to prioritize only few solutions and avoid multiple solutions for the same problem (FFS pending on solution details). A7: The UE can initiate signalling for UE capability restrictions on NW A if NW A allows it. The specification will not capture NW B events which can cause such need. A4: RAN2 to discuss whether the following UE capabilities (not a complete list) are impacted for dual-active MUSIM: MIMO layers, BC capabilities, Measurement capabilities, Bandwidth, srs-TxSwitch, UL tx power, Power Class. For proposals A1-A2, the solution details need more discussion. Other solutions are not precluded (requires company input with details). Will discuss further over email on the solutions (after this meeting) and which capabilities can be affected. For B1-B3, B5, the solution details need more discussion. May prioritize B1, B2 and B5. FFS on signalling details. Other solutions are not precluded (requires company input with details) and none of B1-B5 are agreed as solutions for this WI. Do not consider solution B4 in Rel-18 (since it may have CN impacts which are precluded in this WI) - B1: For UAI based solution, the following steps can be used as a baseline: - The UE is in Connected Mode on NW A . - The UE is configured for UE capability update via UAI. - The UE intends to start or stop connection with NW B or is already in Connected mode in NW B. - The UE requests a change (restriction or removal of restriction) of the UE capabilities at NW A via UAI. - NW A reconfigures the UE, if needed, according to its new capabilities (FFS if NW response is mandatory) - The UE operates in NW A with the updated configuration. - B2: For delta-signaling of UE capability, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE signals the changed UE capabilities to NW A via delta-signaling. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. - B3: The solution for the repetition of UE capability enquiry, the following steps can be used as a baseline: - The UE is in Connected Mode in NW A. - The UE is configured for UE capability update. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests a UE capabilty update request. - NW A sends UECapabilityEnquiry to the UE - UE sends UECapabilityInformation to the NW A gNB. - NW A reconfigures, if needed, the UE according to its new capabilities (FFS if NW response is mandatory. - The UE operates in NW A with the updated configuration. - B4: The solution based on using UE-profiles for capability restriction, the following steps can be used as a baseline: - The UE signals different temporary UE capability sets during registration (FFS if these profiles can be updated later) - The UE is in Connected Mode in NW A . - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE requests to switch to a different UE capability profile, e.g. by signaling an index of the profile. - NW A reconfigures the UE according to its new capabilities. - The UE operates in NW A with the updated configuration. - B5 (11/15): A baseline procedure for MAC-CE based SCell (de)-activation can be considered as follows: - The UE is in Connected Mode in NW A . - The UE is configured for MAC-CE based SCell (de)-activation operation. - The UE starts or stops connection with NW B or is already in Connected mode in NW B. - The UE sends a request to deactivate SCells via MAC-CE. - NW A deactivates, if needed, the requested SCells (FFS if NW response is mandatory). - The UE operates in NW A with the updated configuration. CX: RAN2 to continue evaluation of any Xn-AP, F1-AP or RAN4 impact due to dual-active MUSIM operation. 1: RAN2 can consider such Band conflict scenarios for MUSIM in CONNECTED to arrive at a graceful specification-based solution intended to mitigate such conflicts. Wait for RAN4 feedback on MUSIM gap priority.