3GPP TSG-RAN WG2 Meeting #123		R2-2307693

Toulouse, France，21st – 25th Aug, 2023

Agenda Item:	7.17.4

Source: 	vivo

Title:         	Discussion on MUSIM gap priorities

Document for: 	Discussion and Decision

# Introduction

Regarding Rel-17 MUSIM gap priorities in Rel-18 MUSIM WI [1], we have achieved below agreements in RAN2#121bis-e meeting:

RAN2 will aim to address the RAN4 LS in Rel-18 signalling. Should discuss how to handle Rel-17 gaps without priority (e.g. lowest, highest, network-decided somehow, etc.). Handled in email [231] - Bulk agreements 1: Introduce 1 optional per-UE capability bit (without xDD/FRx differentiation) to indicate MUSIM gap priority configuration and preference. A UE supporting this feature shall also support musim-GapPreference-r17. 2: Introduce a new indication in the OtherConfig to indicate whether UE is allowed to report MUSIM gap priority preference via UAI. 4: The existing IE GapPriority-r17 is re-used to configure the priority for periodic MUSIM gap. 9: RAN2 assumes no RAN4 impact is expected on maximum UL power change due to R18 MUSIM. Can re-discuss if critical issues are found in RAN2.

In this contribution, we will discuss how to support MUSIM gap priority.

# Discussion

  - MUSIM gap priority preference reporting

In Rel-17, if allowed, UE can provide MUSIM gap preference in UAI message to assist the NW A to configure MUSIM gaps. In Rel-18, UE should naturally report MUSIM gap(s) priority preference together with MUSIM gap preference.

- MUSIM gap priority preference is transmitted together with MUSIM gap preference.

There is an open issue in the TS 38.331 running CR[2]:

Editor note:	FFS whether prohibit timer is needed for the signaling of MUSIM gap(s) priority preference.

The musim-GapProhibitTimer was introduced for Rel-17 MUSIM gap preference reporting. If MUSIM gap priority preference is transmitted together with MUSIM gap preference, we can reuse musim-GapProhibitTimer for the MUSIM gap(s) priority preference reporting. That means, no additional prohibit timer is needed for the signaling of MUSIM gap(s) priority preference.

- The prohibit timer musim-GapProhibitTimer is reused for the signaling of MUSIM gap priority preference.

  - MN-SN MUSIM gap coordination

In RAN2#121bis-e meeting, the below agreements were achieved for MN-SN MUSIM gap coordination:

1: The UE is only allowed to provide MUSIM assistance information for Rel-17 MUSIM gap preference to NR MN and NR MN configures the UE with Re-17 MUSIM gap(s). This requires no specification impacts.

Use inter-node messages to convey Rel-17 MUSIM gap configuration from MN to SN in NW A when UE is in NR-DC.

In Rel-18, MUSIM gap priority is supported in MUSIM gap configuration. MN should send MUSIM gap configuration to SN, including MUSIM gap priority configuration.

- MN signals the MUSIM gap configuration to SN, including MUSIM gap pattern and gap priority.

  - Other open issues

Constraints on MUSIM gap priority configuration

In the previous RAN4 LS [3], it is up to NW A on how to use the gap priority assistance information for gap priority configuration. Now RAN4 is discussing the constraints on MUSIM gap priority configuration from NW A. One potential solution is that NW A maintains the same relative priorities requested by the UE, the configured priority level may or may not be the same as that requested by UE. We suggest waiting for RAN4 progress.

MUSIM gap collision solution

RAN4 is also discussing the solutions for collision between different MUSIM gaps, and it was agreed to define two solutions for collision handling between different MUSIM gaps:

- Priority based solution: Network controls the MUSIM gaps priority. The lower priority will be dropped in case of gap collision.

- “Keep” solution: keep all collided MUSIM gaps.

Since RAN4 is still discussing the details for the solutions, we can wait for RAN4 progress, then trigger the discussion in RAN2 for potential signalling support, if needed.

And, it is pending to RAN4 discussion on whether aperiodic gap can have a gap priority.

Therefore, we propose that:

- Wait for RAN4 feedback on the below issues:

- constraints on MUSIM gap priority configuration

- solutions for collision between different MUSIM gaps

- whether introduce priority for aperiodic gap

# Conclusion

In this paper, the following proposals are given:

- MUSIM gap priority preference is transmitted together with MUSIM gap preference.

- The prohibit timer musim-GapProhibitTimer is reused for the signaling of MUSIM gap priority preference.

- MN signals the MUSIM gap configuration to SN, including MUSIM gap priority.

- Wait for RAN4 feedback on the below issues:

- -	constraints on MUSIM gap priority configuration

- -	solutions for collision between different MUSIM gaps

- -	whether introduce priority for aperiodic gap

# References

[1] RP-220955, Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR, vivo.

[2] R2-2306953- Running RRC CR for NR MUSIM enhancements

[3] R2-2302430 LS on priority levels for MUSIM gaps