3GPP TSG-RAN WG2 Meeting #123					R2-2307695

Toulouse, France, August 21-25, 2023

Agenda item:	7.20.3

Source:	Samsung

Title:	Single-DCI based unified TCI extension to multi-TRP operation

Document for:	Discussion & Decision

Introduction

In RAN2#122 meeting, among many issues in [1], RAN2 made following agreements sDCI based Unified TCI extension to mTRP operation [2].

For sDCI based mTRP operation using unified TCI state framework, introduce the new MAC CE, with the following high level design principles: - If the signaling type of the unified TCI state configuration is configured by RRC (i.e. either joint DL/UL TCI state or separate DL/UL TCI state), it applies to both TRP (i.e., as configured by RRC for both TRPs). - The following information can be indicated by the MAC CE (for joint DL/UL TCI mode): - if the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs, - if the indicated TCI codepoint consists of one TCI state, whether the indicated TCI state(s) is for the first or second TRP(s) FFS for the separate DL/UL TCI mode.

Above RAN2 agreements are good high-level guideline to design the new MAC CE for sDCI based mTRP operation using unified TCI state. In this contribution, we provide more detail functionalities on Rel-18 enhanced unified TCI framework for sDCI based mTRP operation.

Multi-TRP operation using unified TCI framework

In Rel-17, unified TCI framework is introduced and is supported for single TRP operation. Given a list of joint/DL TCI states and a list of UL TCI states configured by RRC, unified TCI state activation/deactivation MAC CE is used to active a set of TCI states and DCI indicates one of the activated TCI state to be applied for PDCCH/PDSCH/AP CSI-RS/PUCCH/PUSCH/SRS.

In Rel-18, unified TCI framework is extended to support multi-TRP operation. For multi-DCI multi-TRP, the activated TCI states for each TRP is indicated by a unified TCI state activation/deactivation MAC CE including a CORESET pool index corresponding to the TRP. For single-DCI multi-TRP, a new unified TCI state activation/deactivation MAC CE is introduced to indicate the mapping between the activated TCI states of two TRPs and DCI codepoints. Different from single-TRP operation where only one joint TCI state or one pair of DL/UL TCI states is mapped to a codepoint in DCI, two joint TCI states (each for a TRP) or two pairs of DL/UL TCI states (each pair for a TRP) can be mapped to a codepoint in DCI. Therefore, how to indicate one (either the 1st or the 2nd) or both of the two TCI states to be applied for PDCCH/PDSCH/AP CSI-RS/PUCCH/PUSCH/SRS is an issue. Based on RAN1 agreements (see Appendix), we summarize the solutions of unified TCI state indication for different channel and RS and the relevant RRC parameters, as illustrated in the table below.

- Multi-DCI based multi-TRP operation

Figure 1. TCI selection scheme for mDCI mTRP operation

- Single-DCI based multi-TRP operation

Figure 2. TCI selection scheme for sDCI mTRP operation

Single-DCI based multi-TRP operation

According to the RAN1 agreements in Appendix, the expected sDCI based multi-TRP operation using unified TCI state framework would be depicted as below.

Based on unified TCI framework, scheduling DCI cannot indicate scheduled PDSCH’s TCI state. In Rel-15/16 TCI framework, dynamic switching between single-/multi-TRP based PDSCH scheduling is based on the indicated TCI states. However, TCI states for all channels are simultaneously updated by unified TCI state framework either by MAC CE or DCI after BAT (Beam Application Time). To enable dynamic switching in unified TCI framework, TCI selection field is introduced in Rel-18. Except for the PDSCH reception, other channels use the semi-static TCI selection by RRC configuration (see Figure 2).

One important rule for unified TCI state and considering two TCI states indication using MAC CE (or DCI) was determined by RAN1.

Agreement in RAN1#112b-e (subset/fullset update) On unified TCI framework extension for S-DCI based MTRP operation, support the followings: - For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE) - TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design) - The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

According to the above RAN1 agreements, at least TCI state activation command (MAC-CE) should indicate whether each TCI state mapped to a TCI codepoint is the first or second. In addition, if the UE receives a TCI codepoint mapped with a sub-set of all possible TCI states, the UE update it as indicated by MAC CE and keep other indicated TCI states as the previous TCI state. We provide the figure based on our understanding.

Figure 3. Rule for unified TCI state and two TCI state indication

For example in Figure 3, at the initial state, UE activates {TCI#1, TCI#2} for each TRP1 and TRP2.

If {TCI#3, none} is indicated by MAC CE, UE applies it after BAT but in this case, 2nd TCI state for TRP2 should be kept (i.e. maintain TCI#2). Later, UE receives the MAC CE with a TCI codepoint {none, TCI#4}, then UE applies it after BAT but in this case, 1st TCI state for TRP1 should be kept (i.e. maintain TCI#3).

To summarize, following aspects should be considered to design a new Rel-18 unified TCI state activation/deactivation MAC CE for sDCI based mTRP operation.

- Mode of unified TCI state (either joint DL/UL or separate DL/UL mode) is determined by RRC (no field to indicate it is required in MAC CE)

- For the joint DL/UL mode

  - At most two TCI states (i.e. using joint DL/UL TCI state) can be indicated for a TCI codepoint.

  - Introduce the field(s) to indicate the number of activated TCI state(s) and whether the indicated TCI state(s) is for the first or second TRP(s). I.e. If the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs

- For the separate DL/UL mode

  - At most two TCI states (i.e. separte DL/UL TCI states) can be indicated for a TRP.

  - So, at most four TCI states (i.e. two set of above TCI states) can be indicated for a TCI codepoint.

  - Introduce the field(s) to indicate the number of activated TCI state(s) and whether the indicated TCI state(s) is for the first or second TRP(s). I.e. If the unified TCI state is for one of the TRPs (i.e., 1st or 2nd) or for both TRPs

Example of MAC CE design

Based on our explanation above, we provide an example of Rel-18 unified TCI state activation/deactivation MAC CE for sDCI-based mTRP operation.

For easy understanding, we think bitmap to map the indicating field(s) with the indicated TCI state(s) is an option to support all functionality and requirements.

- Use other fields defined in Unified TCI States Activation/Deactivation MAC CE (see 6.1.3.47 in [3]).

- Pij: This field indicates the bitmap information as shown in the below table. Each bit indicates whether the TCI states for DL/UL per TRP is present or not. The maximum number of TCI codepoint (i) is 8.

- Codepoint: This field indicates the number of indicated TCI codepoint.

Figure 4. Example of MAC CE for Separate DL/UL TCI mode

Figure 5. Example of MAC CE for Joint DL/UL TCI mode

Proposal 1: RAN2 consider above example of Rel-18 unified TCI state activation/deactivation MAC CE for sDCI-based mTRP operation.

Conclusion

We propose following proposals:

Proposal 1: RAN2 consider above example of Rel-18 unified TCI state activation/deactivation MAC CE for sDCI-based mTRP operation.

References

RP-213598	WID: MIMO Evolution for Downlink and Uplink	Samsung.

R2-2306552	Report from NR MIMO evolution session	Session chair (CATT)	Report

TS 38.321	Medium Access Control (MAC) protocol specification (Release 17), v17.5.0.

Appendix

PDSCH

DCI format 1_1/1_2 [TCI selection field] in DCI, the presence is configured by RRC

RAN1#111 Agreement

On unified TCI framework extension for S-DCI based MTRP, a DCI field in DCI format 1_1/1_2 that schedules/activates PDSCH reception is used to determine which one or both of the indicated joint/DL TCI states shall be applied to the scheduled/activated PDSCH reception

- The presence of the DCI field is configurable by RRC; when the DCI field is not present in DCI format 1_1/1_2, the UE shall apply the default indicated joint/DL TCI state(s) to PDSCH reception

  - FFS: Details on the default indicated joint/DL TCI state(s) to PDSCH reception

- FFS: The DCI field is a new indicator field or an existing field (e.g., the existing TCI field)

- FFS: Regardless the DCI field is present or not present, how to apply the indicated joint/DL TCI state(s) to PDSCH reception if the offset between the reception of the DCI format 1_1/1_2 and the corresponding PDSCH reception is less than a threshold

FFS: How to apply the indicated joint/DL TCI state(s) to PDSCH reception scheduled/activated by DCI format 1_0.

Above applies for the case where PDSCHs scheduled by the same DCI.

Agreement in RAN1#112b-e (subset/fullset update)

On unified TCI framework extension for S-DCI based MTRP operation, support the followings:

- For a serving cell configured with joint DL/UL TCI mode, a full-set or any sub-set of {first joint TCI state, second joint TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE)

- For a serving cell configured with separate DL/UL TCI mode, a full-set or any sub-set of {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state} can be mapped to a TCI codepoint of the existing TCI field in a DCI format 1_1/1_2 by TCI state activation command (MAC-CE)

- TCI state activation command (MAC-CE) should indicate that each joint/DL/UL TCI state mapped to a TCI codepoint is the first or second joint/DL/UL TCI state (detail on how to indicate above is up to RAN2 design)

- The first/second indicated joint/DL/UL TCI state(s) is updated according to the corresponding first/second joint/DL/UL TCI state(s) mapped to the TCI codepoint received by the UE

  - If the UE receives a TCI codepoint mapped with a sub-set of {first joint TCI state, second joint TCI state} or {first DL TCI state, first UL TCI state, second DL TCI state, second UL TCI state}, the UE shall update the first/second indicated joint/DL/UL TCI state(s) according to the first/second joint/DL/UL TCI state(s) in the subset and keep other indicated first/second joint/DL/UL TCI state(s) that is not updated by the received TCI codepoint

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP, for PDSCH reception scheduled/activated by DCI format 1_1/1_2 configured w/o the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to the scheduled/activated PDSCH reception

- If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception.

- If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_1/1_2 and the scheduled/activated PDSCH reception is equal to or larger than a threshold.

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP, when a 2-bit [TCI selection field] is configured by RRC to be present in a DCI format 1_1/1_2 in a DL BWP:

- If the DCI format 1_1/1_2 indicates codepoint "10" for the [TCI selection field], the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by the DCI format 1_1/1_2 based on the Rel-16 rules for mapping legacy TCI states to PDSCH transmission occasions, CDM groups, or non-overlapping frequency domain resource allocations by replacing the first and the second indicated legacy TCI states with the first and the second indicated joint/DL TCI states, respectively.

- The codepoint "11" of the [TCI selection field] is reserved.

DCI format 1_0 RRC configures { first, second, or both } of the indicated joint/DL TCI states

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP, support the following:

- Using RRC configuration to indicate whether the first, second, or both of the indicated joint/DL TCI states is/are applied to PDSCH reception scheduled/activated by DCI format 1_0

  - If not configured, the first indicated joint/DL TCI state is applied

  - Only when the UE is configured with PDSCH-CJT and the UE supports two joint TCI states for PDSCH-CJT or the UE is configured with PDSCH-SFN, the RRC configuration can indicate both indicated joint/DL TCI states are applied.

    - For PDSCH-CJT and PDSCH-SFN, if the RRC configuration indicates both indicated joint/DL TCI states are applied, the UE shall apply both indicated joint/DL TCI states to PDSCH reception scheduled/activated by DCI format 1_0 on a search space other than Type0/0A/2 CSS on CORESET#0 (FFS: Other search space and/or CORESETs)

If the UE is in FR1, or the UE supports the capability of two default beams for S-DCI based MTRP in FR2, above applies regardless of the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception.

If the UE doesn’t support the capability of two default beams for S-DCI based MTRP in FR2, above applies when the offset between the reception of the scheduling DCI format 1_0 and the scheduled/activated PDSCH reception is equal to or larger than a threshold.

PDCCH

RRC indication per CORESET { first one, the second one, or both } of the indicated joint/DL TCI states

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP:

- If a CORESET other than a CORESET with index 0 is associated only with USS sets and/or Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or both of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- If a CORESET other than a CORESET with index 0 is associated at least with CSS sets other than Type3-PDCCH CSS sets, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

- For a CORESET with index 0:

  - If the CORESET is associated with SS#0 for Type 0/0A/2 CSS sets, the CORESET is configured by RRC to apply the first one, the second one, or none of the indicated joint/DL TCI state to PDCCH reception on the CORESET

  - Otherwise, the CORESET is configured by RRC to apply the first one, the second one, both, or none of the indicated joint/DL TCI states to PDCCH reception on the CORESET

Note: RAN1 already agrees to use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the indicated joint/DL TCI states to a CORESET in S-DCI based MTRP.

Note: There is no consensus in RAN1 on whether to reuse the Rel-17 RRC parameter followUnifiedTCIstate as a part of above RRC configuration, and whether to reuse followUnifiedTCIstate is up to RAN2 design.

RAN1#110b Agreement

On unified TCI framework extension for S-DCI based MTRP, to inform the association with the joint/DL TCI state(s) indicated by DCI/MAC-CE for PDCCH repetition, PDCCH-SFN, and PDCCH w/o repetition/SFN, support the following:

- Use RRC configuration to inform that the UE shall apply the first one, the second one, both, or none of the joint/DL TCI states indicated by DCI/MAC-CE to a CORESET or a group of CORESETs (if CORESET group configuration is supported)

PUSCH

DCI indicates to apply the {first, second, both} indicated joint/UL TCI state, DCI format 0_1/0_2 (including DG and Type2 CG)

Agreement in RAN1 #112

On unified TCI framework extension for S-DCI based MTRP, when two SRS resource sets for CB/NCB are configured, support the followings for PUSCH transmission scheduled/activated by a DCI format 0_1/0_2 (including DG and Type2 CG):

- If the DCI format 0_1/0_2 indicates codepoint "00" for the existing SRS resource set indicator, the UE shall apply the first indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "01" for the existing SRS resource set indicator, the UE shall apply the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If the DCI format 0_1/0_2 indicates codepoint "10" or “11” for the existing SRS resource set indicator:

  - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB (note: the association between an SRS resource set for CB/NCB and PUSCH transmission occasions(s) is defined according to TS 38.214)

  - FFS: SDM and SFN based PUSCH Tx schemes

FFS: The case that the spatial Tx filter(s) determined from the indicated joint/UL TCI state(s) applied to a PUSCH transmission is different from the spatial Tx filter(s) used for the SRS transmission corresponding to the SRS resource(s) indicated to the PUSCH transmission

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUSCH transmission

- For SDM and SFN based PUSCH Tx schemes, the UE shall apply the first indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the first SRS resource set, and the second indicated joint/UL TCI state to the PUSCH antenna port(s) associated with the second SRS resource set, respectively.

- Note: The association between PUSCH antenna port(s) and an SRS resource set is discussed and defined in STxMP AI.

RAN1#112b Agreement

On unified TCI framework extension for S-DCI based MTRP, the UE shall apply the first indicated joint/UL TCI state to PUSCH transmission(s) scheduled/activated by DCI format 0_0 (including DG and Type2 CG).

RRC configs 1st or 2nd or both indicated joint/UL TCI states is used for PUSCH using type-1 CG

RAN1#112b Agreement

On unified TCI framework extension for S-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first, the second, or both indicated joint/UL TCI states to the corresponding CG-PUSCH transmission

- If the first or the second indicated joint/UL TCI state is applied, the UE shall apply the first or the second indicated joint/UL TCI state to all PUSCH antenna port(s) of corresponding PUSCH transmission occasions(s)

- If both indicated joint/UL TCI states are applied:

  - For TDM based PUSCH Tx scheme, the UE shall apply the first indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the first SRS resource set for CB/NCB, and the second indicated joint/UL TCI state to the PUSCH transmission occasions(s) associated with the second SRS resource set for CB/NCB

  - FFS: SDM and SFN based PUSCH Tx schemes

RAN1#112b Agreement

On unified TCI framework extension for M-DCI based MTRP, an RRC configuration is provided to a Type1 CG configuration to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding CG-PUSCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

PUCCH

RRC per PUCCH resource/resource group indicates to apply the first or the second indicated joint/UL TCI state

RAN1#112 Agreement

On unified TCI framework extension for M-DCI based MTRP, for PUCCH transmission:

- An RRC configuration can be provided per PUCCH resource/resource group to inform that the UE shall apply the first or the second indicated joint/UL TCI state to the corresponding PUCCH transmission, where the first and the second indicated joint/DL TCI states correspond to the indicated joint/UL TCI states specific to coresetPoolIndex value 0 and value 1, respectively.

RAN1#111 Agreement

On unified TCI framework extension for S-DCI based MTRP, use RRC configuration to inform that the UE shall apply the first one, the second one, or both of the indicated joint/UL TCI states to a PUCCH resource/group

- Note: Detail of the RRC configuration is left to RAN2 design

RAN1#113 Agreement

On unified TCI framework extension for S-DCI based MTRP, when two indicated joint/UL TCI states are applied to a PUCCH resource/resource group:

- For TDM based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to repetitions of the PUCCH transmission corresponding to the PUCCH resource/resource group based on the Rel-17 rules for mapping spatial settings to the repetitions by replacing the first and second spatial settings with the first and second indicated joint/UL TCI states, respectively.

- For SFN based PUCCH Tx scheme, the UE shall apply two indicated joint/UL TCI states to the PUCCH transmission corresponding to the PUCCH resource/resource group.