3GPP TSG-RAN WG2 Meeting #123	R2-2307698

# Toulouse, France, August 21-25, 2023				   Revision of R2-2306227

Agenda item:	7.4.3

Source:	Samsung

Title:	Considerations on Subsequent CPAC after SCG Change

Document for:	Discussion & Decision

Introduction

In this contribution, we handles further issues need to be concluded to support the subsequent CPAC after SCG change.

Discussion

Background

For the Rel-16/17 CPA/CPC (conditional PSCell addition/conditional PSCell change) operation, current specifications allow only one activated SN and UE is configured with the multiple candidate SNs for CPAC. Below is the structure of legacy Rel-16/17 CPA/CPC configurations.

- List of candidate CG configurations (maximum 8 entries).

- condReconfigId-r16 (from 1 to 8) represent the index of candidate configurations.

Fig 1: Structure of CHO/CPAC configurations

Even though the multiple candidate CG configurations can be configured, these CG configuration stored in the UE would be release after SCG change so the subsequent SCG change using CPAC configuration has not been supported.

In RAN2#119bis-e meeting, RAN2 made high level agreements to support subsequent secondary cell group change as captured above. In addition, RAN2 also agreed to handle the reference configuration as a separate configuration in RAN2#120 meeting. We think above baseline agreements are most essential procedure and details can be further discussed, i.e. FFS points above can be discussed in this contribution.

MCG reference cell configuration

In RAN2#121bis-e meeting, RAN2 made following agreements and supporting whether MCG configuration is included in the Rel-18 CPC configurations leaves FFS, so it is not captured in the running CR yet [8].

The following may be included in the initial RRC reconfiguration message containing the Rel-18 CPC configurations:

- Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty.

- FFS whether MCG configuration is included.

- FFS RRC model for the reference configuration.

Current specification for CPC allows to change the MCG configuration in CPC configuration as shown in Fig 1., so we think MCG configuration should be changed during the Rel-18 CPC operations (i.e. subsequent CPC) because it is required to change MCG configuration associated with the newly configured SCG configurations. We provide some options to support MCG configuration for the subsequent CPC.

- Option 1: Complete MCG configuration is configured with SCG configuration for Rel-18 CPC.

- Option 2: Introduce the separate MCG/SCG reference configurations for Rel-18 CPC and each candidate cell creates the condRRCReconfig for LTM which includes the MCG configuration and SCG configuration based on the reference configurations

We think option 2 has merits to reduce the signalling overhead and harmonized signalling structure for MCG/SCG i.e. delta configuration is applied to both MCG/SCG configurations for the candidate cells based on the reference MCG/SCG configurations.

Proposal 1: Introduce the separate MCG/SCG reference configurations for Rel-18 CPC and each candidate cell creates the condRRCReconfig for LTM which includes the MCG configuration and SCG configuration based on the reference configurations.

Security Aspects

Fig 2. Example scenario of subsequent CPC

Regarding the FFS point that how to handle the CPC configuration if PSCell change is conducted or indicated by the network, additionally how to handle the CPC and S-SN configurations after the subsequent cell group change is also needed. We think that handling for CPC configuration and S-SN configuration is highly related with the SA3 decision which RAN2 asked (in R2-2213337 [6]) in the RAN2#120 meeting. According to the answers regarding the same sk-counter/ S-KgNB is used every time it is connected to SN #1 (which UE previously connected) before and after being connected to SN #2. SA3 replied following answers in [7].

Q1: RAN2 would like to know whether SA3 considers the existing handling of sk-counter/ S-KgNB in the above scenarios acceptable, i.e. the same sk-counter/ S-KgNB is used while connected to SN #1 before and after being connected to SN #2. SA3 Answer: No, it is not acceptable from security point of view. Q2: If SA3 considers the existing handling of sk-counter/ S-KgNB in the above scenarios not acceptable, RAN2 kindly asks SA3 to provide requirements for a solution. SA3 Answer: Same S-KgNB shall not be used. SA3 will specify an additional sub-clause in TS 33.501 under 6.10 for the security of selective SCG activation.

According to the SA’s response, it is not allowed to use the same sk-counter/ S-KgNB while connected to SN #1 before and after being connected to SN #2 (see Fig 2).

In addition, SA3 further update the security aspects in [9] including the status of security analysis and to provide feedback on the solution details.

SA3 wants to update on the state of the security analysis on selective SCG. SA3 has considered different alternative solutions to address the issue and concluded to have a SN Counter based solution for fresh key derivation for selective SCG activation. Following is the solution details from security perspective: 1. The MN provides to the UE the SN Counter values for each SN, and the UE stores these values along with the CPC. 2. The UE derives the KSN using the KgNB together with an unused SN Counter value pre-provisioned by the MN. The UE changes the SN Counter upon every SN change. SA3 is offering a joint conference call that can be used to discuss and to enhance the above described solution details.

Therefore, RAN2 need to consider following problems and we provides some candidate solutions for each question based on SA3 response.

Q: How to handle the CPC condition/configuration for the executed PSCell

- Alt 1: Remove the executed CPC configurations in the stored candidate CPC configurations (or if S-KgNB is per SN not a cell, then all the SN originated PSCells are removed)

  - When sk-counter is configured to the candidate PSCells (or SN), it should be different between PSCells even they are in the same SN. This prevent the used sk-counter is reused in the subsequent PSCell changes, this also prevent UE from revisiting PSCells that once UE visited before.

- Alt 2: Keep the executed conditional PSCell configurations as the candidate CPC configuration

If we go to the Alt 1-1, the specification impacts are not huge i.e. add UE procedure to remove the stored CPC configuration whenever UE execute the CPC but it is not exactly what RAN2 intended in this WI, Rel-18 feMOB has to support following scenario based on WID:

- UE remains connected to the same PCell and is configured with several conditional reconfigurations (a conditional reconfiguration is an RRC reconfiguration message, possibly including sk-counter), each with a different candidate target PSCell, and, based on conditions on measurement results on candidate target PSCells, the UE selects and executes one of these conditional reconfigurations, thus changing PSCell.

- After executing a conditional reconfiguration, the UE may keep conditional reconfigurations to the other candidate target PSCells and continue switching between the candidate target PSCells multiple times, including to an earlier selected PSCell. All of this happens without any reconfiguration by the network (i.e. using the stored conditional reconfigurations).

So, RAN2 need solve this security problem to support the intended operation on subsequent CPC i.e. how to configure the sk-counter S-KgNB for the candidate PSCell for CPC to support subsequent CPC.

We think there are some candidate solution to solve how to configure the sk-counter/ S-KgNB for the candidate PSCell to support subsequent CPC. All candidate options should be consulted by SA3.

- Option 1: Introduce the multiple sk-counter/ S-KgNB for the subsequent CPC per SN (or per candidate PSCell)

- MN and SN decide the multiple security keys which will be used for subsequent CPC i.e. MN provides the multiple S-KgNB to SN (or a candidate PSCell).

- In CPC configuration, UE receives the list of sk-counter (e.g. sk-counterList = {sk-counter #1, sk-counter #2, … sk-counter #N}) which was made by above MN-SN procedure.

- UE and NW should know how many times this CPC is executed to the same PSCell or SN.

  - UE and NW store the information for the number of CPC execution to the same PSCell or SN. (e.g. association between UE and PSCell which CPC is performed)

- Option 2: Use a sk-counter/ S-KgNB for the subsequent CPC as legacy but introduce the new rule to increment the sk-counter to create the multiple S-KgNB which will be used in same SN after subsequent CPC.

- Whenever the CPC to a same PSCell (or SN) is conducted, apply sk-counter +1 (or new rule for the increment).

  - When sk-counter is configured to the candidate PSCells, it should be different between PSCells even they are in the same SN.

- UE and NW should know how many times this CPC is executed to the same PSCell and/or SN

From our understanding, above candidate options are valid and those can be easily support from RAN2 perspective (signaling and procedure). However, it requires the changes of the SA3 specification and all security related procedures are needed to be consulted by SA3.

Proposal 2: RAN2 ask to SA3 if the multiple sk-counter/ S-KgNB for the subsequent CPC per SN (or per candidate PSCell) is valid.

Additional Information between MN and SNs

As a RAN2/RAN3 impact, we think the additional information between MN and SNs are required to negotiate the support of subsequent CPAC after SCG change using inter-node RRC message and/or Xn message.

Proposal 3: Additional information between MN and SNs are required to negotiate the support of subsequent CPAC after SCG change using inter-node RRC message and/or Xn message.

There is another open issue for same target cell, whether to support simultaneous configuration of CPA and CPC and how to support both configuration and conditions are different for CPA and CPC. We think this issue can be applied for following scenario:

- Step 0: UE in Cell A with single connectivity

- Step 1: UE receives the CPA/CPC configurations (for subsequent change)

- Step 2: Cell B in SN1 is added to PSCell with CPA procedure

- Step 3: PScell is changed to Cell C in SN2 by CPC procedure

- Step 4: PSCell is changed to Cell B in SN1 again by CPC procedure

To support the simultaneous configuration of CPA and CPC in the single connectivity UE, we think following options can be considered.

- Option 1: one condReconfig ID includes CPA condition and CPC condition

- Only condition could be different (i.e. two condition and one configuration)

      - Type 1: CPA condition (e.g. CondEvent A4)

      - Type 2: CPC condition (e.g. CondEvent A3)

- Both condition and configuration could be different

      - Introduce the new field for an additional configuration of measID 2

- Option 2: Each condReconfig ID includes only one type of condition (i.e. both condition and configuration could be different)

- condReconfig ID 1:  Types for CPA (for same MO)

- condReconfig ID 2:  Types for CPC (for same MO)

Option 2 seems simple and no further specification change is required, only need to describe that the simultaneous configuration of CPA and CPC are supported using the current signaling. However, it requires redundant signaling for the same measurement object to configure different type of condition and configuration. Meanwhile, Option 1 has benefit of signaling reduction to configure the CPA and CPC simultaneously.

Proposal 4: One condReconfig ID includes CPA condition and CPC condition to support the simultaneous configuration of CPA and CPC.

When to configure UE on initial source PSCell can be used for the subsequent SCG change

In RAN2#121 meeting, RAN2 made one following agreement on the subsequent CPAC after SCG change [3].

It is assumed that if the UE need to be able to return to a current SCG by conditional procedure, then the network could explicitly configure a candidate configuration for that cell.

After looking through the whole subsequent SCG change procedure (e.g., MN initiated subsequent SCG change procedure in Fig.3), we can see that possible configuration time is before or after execution of CPC.

-Alternative#1: configuration time is before CPC execution

Even though UE received execution condition of source PSCell in advanced, UE shouldn’t evaluate the CPC execution condition of the source PSCell until the source PSCell becomes candidate PSCell after CPC execution.

-Alternative#2: configuration time is after CPC execution

If network want to configure it as one candidate PSCell for subsequent SCG change, the UE receives the configuration of the source PSCell after CPC execution. And then UE starts to evaluate the CPC execution condition of source PSCell.

Fig 3.MN initiated subsequent SCG change procedure

Compared with the above two schemes, we prefer the Alternative#1 because source PSCell can be configured using the same RRCConfiguration message simultaneously including configuration of other candidate PSCells. However, in Alternative#2, a separate RRCConfiguration will be needed for configuration of source PSCell. But we would like to say that when to configure source PSCell is decided by network, it is possible for network to configure it before or after CPC execution.

Based on above discussion in this section 2.6, we have a below proposal on when to configure UE on initial source PSCell be used for the subsequent SCG change.

Proposal 5: Whenever for before or after CPC execution, it is both acceptable for NW to explicitly configure the execution condition of initial source PSCell as one of the candidate PSCells for the subsequent SCG change.

Conclusion

Based on the above, RAN2 is requested to discuss and agree on the following proposals:

Proposal 1: Introduce the separate MCG/SCG reference configurations for Rel-18 CPC and each candidate cell creates the condRRCReconfig for LTM which includes the MCG configuration and SCG configuration based on the reference configurations.

Proposal 2: RAN2 ask to SA3 if the multiple sk-counter/ S-KgNB for the subsequent CPC per SN (or per candidate PSCell) is valid.

Proposal 3: Additional information between MN and SNs are required to negotiate the support of subsequent CPAC after SCG change using inter-node RRC message and/or Xn message.

Proposal 4: One condReconfig ID includes CPA condition and CPC condition to support the simultaneous configuration of CPA and CPC.

Proposal 5: Whenever for before or after CPC execution, it is both acceptable for NW to explicitly configure the execution condition of initial source PSCell as one of the candidate PSCells for the subsequent SCG change.

References

Chairman notes for RAN2#119bis-e meeting	RAN2.

Chairman notes for RAN2#120 meeting	RAN2.

Chairman notes for RAN2#121 meeting	RAN2.

Chairman notes for RAN2#121bis-e meeting	RAN2.

RP-213565	New WID on Further NR mobility enhancements	MediaTek.

R2-2213337	LS on security for selective SCG activation	RAN2.

S3-231397	Reply LS R2-2213337 LS on security for selective SCG activation	SA3.

R2-23xxxxx	[Post122][056][Mob18] 38.331 Running CR for selective activation of SCGs for NR-DC (OPPO).

S3-233200	draft LS on Security Solution for Selective SCG		SA3.