3GPP TSG-RAN WG2 Meeting #123		R2-2307708

Toulouse, France, 21st– 25th August, 2023

Source:	CATT

Title:	SON Enhancement for NR-U

Agenda Item:	7.13.5

Document for:	Discussion and Decision

Introduction

In RAN2#122 meeting, several agreements on SON for NR-U were achieved in [1]. In this contribution, we further analyse remaining NR-U issues in SON and our proposals are provided.

Discussion

SON enhancements for NR-U in RA report

In RAN#122 meeting, one remaining issue on NR-U in RA report is how to represent the preamble transmission attempts blocked by LBT. One way is to introduce a new field to count the number of preamble transmissions blocked by LBT per RA procedure, another way is to reuse the existing field to count the associated number. In legacy spec, the parameter numberOfPreamblesSentOnSSB and numberOfPreamblesSentOnCSI-RS are used to indicate the number of preamble transmitted successfully, and the maximum number is 200, it is not necessary to extend the value of this IE. Besides, RAN2 already agreed that only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam, if we extend these two parameters with the failed preamble transmissions by LTB failure indication, it seems conflict to the preamble transmission attempt agreement in last meeting. So, it is better to introduce a new filed to count the number of preamble transmissions blocked by LBT per RA procedure.

RAN2#122 meeting agreement: 1 Only the preamble transmission attempts for which LBT was successful are represented in the “per RA attempt info list” for a given beam. 2 On how to represent the preamble transmission attempts blocked by LBT, Introduce a field (or reusing the existing field) that counts the number of preamble transmissions blocked by LBT per RA procedure, and a flag indicating transmission failures experienced right before beam switching. Details can FFS.

Proposal 1: Introduce a new filed to count the number of preamble transmissions blocked by LBT per RA procedure.

SON enhancements for NR-U in SHR

In NR-U, it may happen that the handover is performed successfully while UL consistent LBT failures has been triggered and not cancelled. Based on current specification, this information will not be reported in SHR. Some companies propose some enhancements to SHR, e.g. report LBT failure information in SHR.

In current SHR, when SHR is triggered and reported, it indicates that the UE experiences poor channel condition in the source cell before assessing the target cell. From this perspective, the network knows that this is one late handover. Hence, the network can perform corresponding optimizations. For example, the network can update the threshold for measurement report or trigger one earlier handover for the UE.

In NR-U, the gNB and UE may both apply LBT before performing a transmission including handover procedure. Hence, the network may figure out whether the channel is busy or not by LBT detection for large time scale. So the benefits for reporting LBT failure in SHR should be studied carefully.

In current specification, the UE will trigger SHR in the following cases:

- if the ratio between value of the elapsed time of the timer T304 and the configured value of the timer T304,  is greater than thresholdPercentageT304; or

- if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310 is greater than thresholdPercentageT310 included in the successHO-Config; or

- if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312 is greater than thresholdPercentageT312; or

- if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running

Similarly, one condition for SHR for consistent LBT failure should be defined. Possible solutions for SHR can be the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful.

Proposal 2: Triggering condition for SHR reporting consistent LBT failure information can be: the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful.

SON enhancements for NR-U in RLF report

RAN2#122 meeting agreement: The UE logs RA-InformationCommon including LBT info in the RLF-Report, in case of HOF and when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure (as in legacy). FFS: The UE logs in the RLF-Report the BWP information (at least the locationAndBandwidth, and the subcarrierSpacing) of all the BWPs in which the UE detected the consistent UL LBT failures right before the RLF/HOF.

In RAN#122 meeting, RAN2 agreed to introduce RA-InformationCommon in RLF report when the RLF cause is randomAccessProblem or beamFailureRecoveryFailure. One remaining issue on NR-U in RLF report is as above yellow part. In order to give a whole picture on BWP(s) where UE detected the consistent UL LBT failure, then network will avoid make UE work on the associated BWP, we suggest to confirm the FFS and UE could logs all the BWP information with detected the consistent UL LBT failures in RLF-report.

Proposal 3: The UE logs in the RLF-Report the BWP information of all the BWPs in which the UE detected the consistent UL LBT failures right before the RLF/HOF.

Conclusion

According to the analysis in section 2, we make the following proposals.

SON enhancements for NR-U in RA report:

Proposal 1: Introduce a new filed to count the number of preamble transmissions blocked by LBT per RA procedure.

SON enhancements for NR-U in SHR:

Proposal 2: Triggering condition for SHR reporting consistent LBT failure information can be: the number of LBT failure in RA procedure is larger than a threshold during a duration when UE receives the HO command to the time of HO is successful.

SON enhancements for NR-U in RLF report:

Proposal 3: The UE logs in the RLF-Report the BWP information of all the BWPs in which the UE detected the consistent UL LBT failures right before the RLF/HOF.

Reference

- R2-2306546, Report from SON/MDT session, CMCC