3GPP TSG-RAN WG2 Meeting #123                                                                    R2-2307709

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT

Title:	RACH enhancement for SON

Agenda Item:	7.13.6

Document for:	Discussion and Decision

Introduction

For RACH enhancement, there is a summarized document [1] in last RAN2 meeting, and the following agreements are achieved:

Agreements:

RACH Partitioning

1	RAN2 confirms agreed “used feature combination” is all the features configured in the FeatureCombination applied for the RACH procedure.

2	Feature specific RACH information is included in RA-InformationCommon and is also included for RLF report and CEF report.

Msg3 repetition

3	Not include the number of Msg3 repetition applied in RACH procedure in RA report.

SN RACH Report

4	When reporting SN NR RA-report to LTE BS, the unique PSCell identities (i.e. if a PSCell occurs more than once in NR RA-ReportList, it is recorded only once in the list of PSCell identities) are included outside the NR RA report container.

5	Revert the agreement that UE does not support reporting NR RA report to LTE when it is in standalone LTE mode i.e., eNB may fetch the NR RA report irrespective to whether the UE is in single connectivity or dual connectivity.

6	No need to introduce availability bit to notify LTE BS there are available NR RA report for fetching.

7	Enhance the LTE UE information Request procedure with NR RA-Report request flag to fetch the NR RA-Report in LTE.

8	For NR RACH report, UE performs RPLMN checking before sending the NR RACH report to LTE BS.

9	A new UE capability is introduced to indicate whether UE supports NR RACH Report in LTE.

Based on the question of RAN3 [2], RAN2 has revisited the agreement that eNB may fetch the NR RA report irrespective to whether the UE is in SA or DC, and a reply LS has been sent to RAN3 [3]. In this contribution, we discuss the rest issues for SgNB RACH report, and also the issues on RACH report for RACH partitioning including how to indicate SDT failure in RA report, the RACH resource reporting for RA partitioning and the slicing information in RA report.

Discussion

SgNB RACH report for MR-DC scenarios

In RAN2#121 meeting, it is agreed that “To have “a list of SN RA report entries as a single NR container (i.e. NR RA-ReportList)”.”

The NR RA-ReportList IE structure defined in TS38.331 can be used to include a list of up to maxRAReport-r16 number of successful random access procedures, or failed or successful completion of on-demand system information request procedure. But the NR UE variable can only be used to record the MN/SN RACH information of NR SA or NR-NR DC, and be reported to the NR MN for DC case or be reported to the gNB in NR SA case. For SgNB RACH information recorded in (NG) EN-DC, even the same format of RA-ReportList (IE structure) is used, an UE variable in LTE specification for the specific scenario is necessary.

Since in LTE, the legacy LTE RA report (defined in R9) is almost an immediate report after UE has connected to a cell, there is no variable about RA report at all. Therefore to support the SgNB RA report mechanism, an additional UE Variable should be introduced, and the UE variable should include a NR format container with RA-ReportList IE structure, a PSCell list and a PLMN list for RPLMN checking.

Proposal 1: An additional UE Variable for SgNB RACH information should be introduced in TS36.331 to include a NR format container with RA-ReportList IE structure, a PSCell list and a PLMN list for RPLMN checking.

RACH report about RACH partitioning information

2.2.1 SDT failure in RA reprot

In RAN2#120 meeting [2], some preliminary agreements were made on RACH report about RACH portitioning information.

Agreements:

4	UE includes RA and SDT information in RA report when an SDT operation fails.

But there are some other issues which are needed to be further studied. In the following, we analyse SDT issues in RA report as well as some other issues related to RACH partitioning.

For SDT, it has been agreed to included RA and SDT information in RA report when an SDT operation fails. But it is it still unclear on how to associate SDT feature and information for failed SDT operation.

In Rel-17, onDemandSISuccess was introduced [3]. When on-demand SI request is successful, onDemandSISuccess is set to be true. When on-demand SI request fails, onDemandSISuccess is absent. And the network can figure out whether SI request procedure is successful or not by onDemandSISuccess together with raPurpose. For example, when raPurpose is set to requestForOtherSI or msg3RequestForOtherSI, and onDemandSISuccess is set to be true, it means this on-demand SI procedure is successful; when raPurpose is set to requestForOtherSI or msg3RequestForOtherSI, and onDemandSISuccess is absent, it means this on-demand SI procedure failed.

onDemandSISuccess This field is set to true when the RA report entry is included because of either msg1 based on demand SI request or msg3 based on demand SI request and if the on-demand SI request is successful. Otherwise, the field is absent.

Similarly, the same mechanism can be reused for SDT. I.e. defining a flag which indicates whether the SDT procedure is successful or not.

Proposal 2: Define a flag to indicate whether RA-SDT procedure is successful or not.

But there is no corresponding raPurpose associating with this flag.

In RAN2#119bis [4], it was agreed that as follows:

1	Agree to add the following parameters into RACH report for RACH partitioning:

-	Feature or the combination of features that triggered the RACH

-	Used feature combination

So, the feature or the feature combination that triggered the RACH can be used together with this new defined flag to indicate whether SDT is successful or not.

Or we define one new RA purpose for SDT which is similar to consistent LBT failures in the SpCell which was agreed in RAN2#119bis meeting. RAN2 can down select solutions in both options.

Proposal 3: RAN2 to down select one of the options to associate SDT and the new flag to indicate whether RA-SDT procedure is successful or not:

- Associating with feature or feature combination triggering the RACH and or used for RACH;

- Defining one new RA purpose for RA-SDT.

2.2.2 RACH resource configuration for RACH partitioning

In Rel-17 RACH partitioning, multiple RACH resources are configured for different RACH partitioning in order to enable early identification of the RACH feature or RACH feature combination on the network side. But it may be happened that the UE triggers one feature or feature combination, but there is no corresponding RACH resource. Based on current spec procedure, the UE will select another RACH resource to initiate RACH procedure based on the used feature priority. So, it is useful to report the RACH resources for the configured RACH feature/feature combination, i.e., the RACH resource configuration for RACH partitioning.

Proposal 4: The UE reports the RACH resource configuration for feature/feature combination for SON.

In RACH partitioning procedure, the UE will select the RACH resources with a feature which has the highest priority among the features applicable to the RACH procedure. For example, when the SDT (Small data transmission), Msg3 repetition and NSAG (Network Slice AS Group) 1 are applicable to the RACH procedure, the UE will compare the featurePriorities of each feature. And assuming the featurePriorities for NSAG1 is the feature with highest priority, the UE will select the RACH resource corresponding to NSAG1 feature. Hence, it is useful to report the featurePriorities to the network to figure out why the mismatch happens between the feature/feature combination that triggered the RACH and the feature/feature combinations that used in the RACH (i.e. intended feature/feature combination).

Hence, we propose that:

Proposal 5: The UE indicates the featurePriorities information for feature /feature combination for SON.

2.2.3 Slicing

RAN2#121bie-e meeting => 2 FFS: Include NSAG priority in RA report. 3b FFS: UE reports NSAG IDs which are associated with the S-NSSAI(s) that triggered the random access attempt or NSAG IDs which associated with the S-NSSAI(s) triggering the access attempt and that are included in SIB1. 3c FFS: Include S-NSSAI(s) in RA report.

In RAN2#121bis-e meeting, as above, RAN2 discussed whether/what NSAG associated information to be reported in RA report. According to below agreement, for slicing, UE could report the used NSAG#n1 to the network, meanwhile, the UE could also report the network the other NSAG, such as NSAG#n2, which trigger the same access attempt and has higher priority than NSAG#n1, but this NSAG#n2 is not used because of the restriction in SIB1, this could help network understanding the RACH resource requiring for different NSAG(s) and make some optimization on the resource configuration.

RAN2#119bis Agreements: 1 Agree to add the following parameters into RACH report for RACH partitioning: - Feature or the combination of features that triggered the RACH - Used feature combination

Proposal 6: It is suggested that NSAG-ID(s) in RA report at least includes the applied NSAG-ID(s) and not applied NSAG-ID(s) which triggered the same random access procedure and has higher priority than applied NSAG-ID(s).

Conclusion

According to the analysis in section 2, we propose:

Proposal 1: An additional UE Variable for SgNB RACH information should be introduced in TS36.331 to include a NR format container with RA-ReportList IE structure, a PSCell list and a PLMN list for RPLMN checking.

Proposal 2: Define a flag to indicate whether RA-SDT procedure is successful or not.

Proposal 3: RAN2 to down select one of the options to associate SDT and the new flag to indicate whether RA-SDT procedure is successful or not:

- Associating with feature or feature combination triggering the RACH and or used for RACH;

- Defining one new RA purpose for RA-SDT.

Proposal 4: The UE reports the RACH resource configuration for feature/feature combination for SON.

Proposal 5: The UE indicates the featurePriorities information for feature /feature combination for SON.

Proposal 6: It is suggested that NSAG-ID(s) in RA report at least includes the applied NSAG-ID(s) and not applied NSAG-ID(s) which triggered the same random access procedure and has higher priority than applied NSAG-ID(s).

Reference

- R2-2306760	Summary of 7.13.6 RACH enhancement (ZTE)

- R2-2304631 Reply LS on RACH enhancement for R18 SONMDT RAN3

- R2-2306848 Reply LS on RACH enhancement RAN2