3GPP TSG-RAN WG2 Meeting #123                                                                    	R2-2307710

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT

Title:	SON and MDT Enhancement for NPN

Agenda Item:	7.13.7

Document for:	Discussion and Decision

Introduction

In last meeting, we made a summary based on companies’ proposals, but only one agreement is achieved for SON and MDT enhancement for NPN:

=>	FFS: Include the SNPN ID/CAG ID(s) in the logged MDT report or cell type indication (e.g., NPN cell).

Agreements:

1	Include CAG ID(s) in the logged MDT area configuration.

So in this meeting, we first discuss whether the CAG ID(s) is enough for logged MDT area configuration and the NPN related information in logged MDT report, and then some other issues are also discussed.

Discussion

NPN enhancement per use case

For logged MDT

For area scope configuration

It is agreed in last RAN2 meeting to include CAG ID(s) in the logged MDT area configuration. But the details are still FFS. RAN3 has discussed the PNI-NPN area configuration in previous meetings, and a TP for MDT BL CR to TS 38.413 has been agreed [1]. We list it in the Annex. In the MDT Configuration-NR IE, the PNI-NPN area scope is added using the format of a list, and each entry of the list includes a PLMN ID and a CAG-ID.

From the agreement in RAN2 and the running CR [2], it is aligned with the PNI-NPN configuration with RAN3 BLCR: Include PLMN ID+CAG ID pair(s) in the logged MDT area configuration. But also in RAN3 discussion, the area configuration related with PNI-NPN can have 3 cases:

- Case A: NW configures PNI-NPN only;

- Case B: NW configures PN only;

- Case C: NW configures both PN and PNI-NPN.

To include all the three cases, the PNI-NPN area scope list is appeared twice in the MDT Configuration-NR IE, i.e. inside and outside the legacy Area Scope of MDT IE.

RAN2 should also consider these 3 cases in the area scope configuration in TS38.331. The legacy area scope in RAN2 has 3 levels of configuration limitation for logged MDT configuration which can be aligned with RAN3: PLMN wide, TAI/TAC wide or cell wide. In the current AreaConfiguration-r16 IE in TS38.331, the field of areaConfig-r16 is mandatory present. And the areaConfig-r16 only contains Cell based/ TAC based/TAI based information. The PLMN wide level is implicitly indicated by “the AreaConfiguration is not configured”. Since there is no NCE or extension marker in the AreaConfiguration IE, to make the spec clear, we suggest using a critical extension in R18 to indicate the 3 cases of area scope limitation agreed by RAN3.

Observation 1: There is no NCE or extension marker in current AreaConfiguration IE.

Proposal 1: RAN2 should include the 3 cases of PNI-NPN only/PN only/ both PN and PNI-NPN in the MDT area scope, and a critical extension (i.e. AreaConfiguration-r18) can be considered in R18.

The SNPN has the separated PLMN ID which is isolated from the normal PLMN, the PNI-NPN or other SNPN. Therefore it is reasonable to assume that only the UE registered in a SNPN can be configured with the SNPN related logged MDT. The entire configuration and the measurement result included in the UE variable should be removed if the UE changes outside the SNPN. Since SA2 has agreed to introduce support of equivalent SNPNs, and based on the MDT SNPN List agreed by RAN3 (see Annex), the MDT SNPN list can be configured in the area scope of logged MDT measurement configuration similar as the MDT PLMN list in air-interface for the ease of future evolution  of NPN. It is a subset of the equivalent SNPN list and registered SNPN at the time when MDT is initiated. In this release, we can limit the number of SNPN ID in the MDT SNPN list to 1 since RAN2 agreed to use R16 NPN functionality as baseline for R18 SONMDT. A NOTE should be added to clarify this matter.

Proposal 2: Introduce the definition of MDT SNPN list, and add a NOTE to clarify that “MDT SNPN list can only include one SNPN ID in this Release”.

Proposal 3: Only the UE registered in SNPN can be configured with this SNPN related logged MDT, and the UE can continue logging across the MDT SNPN list.

Proposal 4: The solution of adding the SNPN ID in the area scope configuration should align with the RAN3 BLCR.

For NPN ID/cell type reporting

For logged MDT reporting, it has been agreed that the CAG-ID checking is not needed, and the SNPN ID checking is needed before sending the availability indication. It is, only when the PLMN ID and the NID of the UE current registered network are equal to the IDs stored in the UE, the UE could indicate the logged MDT result available to the network.

Meanwhile, since the logged MDT is used to draw a picture of the channel power/quality or to monitor the coverage hole, even if a cell is shared by the normal/SNPN/PNI-NPN network, the measurement result is the result of the cell. The measurement result should be the same from UEs in the same geographic location even with different registered network types. The cell may be PLMN specific, SNPN specific, PNI-NPN specific or may be a cell with network sharing. The allowed network information should be broadcasted in the SIB1 and can be easily obtained by the operator. Therefore, the NPN related information, e.g. NID/CAG-ID, is not needed to be reported to the network. And based on the reason above, other information involving cell type (e.g., NPN cell) is also not needed in the logged MDT report.

Observation 2: The measurement result of logged MDT is the result of the cell which is used to draw a picture of the channel power/quality, and the operator can easily obtain the network/cell type information of the cell without UE reporting.

Proposal 5: Do not add NPN ID related information (e.g. NID/CAG-ID) or cell type (e.g. NPN cell indicator) into the logged MDT report.

For RLF report

For MRO, the RLF/HOF report needs to be enhanced to include the NPN related information. One typical scenario is that only one handover target cell A satisfies handover triggering condition, but the UE cannot access to the cell A because UE does not support to access NPN cell. When analyze the RLF Report retrieved from the UE, the good handover target cell A is not selected as handover target cell. From the legacy reported parameters in the HOF report, we can only deduce the handover triggering condition should be optimized and cannot consider the failure is caused by UE NPN capability limit because the UE context has already been removed when the network performing MRO analysis. In order to correctly analyze RLF and handover failure involving with the NPN cell, the RLF/HOF Report shall be enhanced to include the NPN information. To make the MRO enhancement clearer for the NPN cell and the PN cell, at least the NID/CAG-ID should be included.

Since it is agreed that SPNP ID (e.g. nid) should be introduced into RLF/HOF report at last meeting, here we propose to introduce PNI-NPN related ID into RLF/HOF report.

For detailed consideration, the network identifier (e.g. CAG-ID) at the time when the RLF/HOF occurred is a very important piece of information for MRO. It is, for the RLF report, the CAG-ID of the failedPCellId should be recorded, and for HOF report, the CAG-ID of the previousPCellId should be recorded. From the received network identifier recorded by UE and the network identifier supported by the re-established cell (for RLF) or the failed target cell (for HOF), the network could deduce whether the failure is due to the inability to connect to the authorized PN network, or due to weak of signal.

For example, the following figure shows four cells that all belong to the PNI-NPN network. Cell 0 and cell 1 support both CAG0 and CAG1, cell 2 supports CAG0, and cell 3 supports CAG1.

Figure 1 UE moves across PNI-NPN cells

During the UE0's handover from cell 0 to cell 1, UE0, which only supports CAG0, selected cell 1 as the target cell for handover, but the handover failed. UE0 then performed an RRC connection reestablishment with cell 2. UE0 recorded a RLF report and sent it to the network. After analyzing the report, the network found that the handover error was caused by selecting the wrong cell, and cell 2 should be selected as the target cell for handover.

Similarly, during the UE1's handover from cell 0 to cell 1, UE1, which only supports CAG1, selected cell 1 as the target cell for handover, but the handover failed. UE1 then performed an RRC connection reestablishment with cell 3. UE1 recorded a RLF report and sent it to the network side. After analyzing the report, the network found that the handover error was caused by selecting the wrong cell, and that cell 3 should be selected as the target cell for handover.

To record the network identifier in UE, the PLMN ID in the legacy RLF/HOF report in the fields of failedPCellId and previousPCell can be reused as part of the PNI-NPN ID. The PLMN IDs in the fields above can be the same for SNPN ID since before R18 equivalent SNPNs is not support. But for PNI-NPN IDs, different PNI-NPNs can have different PLMN IDs+CAG-IDs, therefore to report the PNI-NPN ID, both PLMN ID and CAG-ID are needed. Since the PLMN IDs are already set in the CGI-Info-Logging-r16 of failedPCellId and previousPCellId fields, the CAG-IDs of the failed cell and the previous Pcell should be included.

Observation 3: The PLMN ID parts of PNI-NPNs are already set in the IE structure (CGI-Info-Logging-r16) of failedPCellId and the previousPCellId fields.

Proposal 6: Included PNI-NPN IDs (e.g. cag-IDs) into the RLF/HOF Report of the failed cell and the previous Pcell, respectively.

A UE could store an allowed CAG list which includes all the subscribed CAG Identifiers. Only when at least one of the CAG-IDs supported by the candidate cell is in the UE's allowed CAG list, the UE can access the cell. Since the UE’s allowed CAG list will not be stored in the RLF/HOF occurred cell and the allowed CAG list may be changed as time goes by, when recording the RLF/HOF information, it is also needed to record the allowed CAG list at the time of  RLF/HOF occurring.

When analyzing the cause of failure, the network can determine whether different failure cases or RRC connection re-establishments to a cell are due to the NPN access limitation based on the cell supported PNI-NPN ID and UE's allowed CAG list. This can help to find suitable target cells for handovers between public and private networks, optimize handover parameters, and improve handover success rates.

Proposal 7: Included UE's allowed CAG list at the time of RLF/HOF occurring into the RLF/HOF Report.

For other SON/MDT measurements

Besides the typical use cases of RLF/HOF report and logged MDT, other use cases can also be considered for NPN enhancement.

We consider it is necessary to enhance the L2 measurement to include the NPN information. The NPN capable UE may have high priority than other normal UEs for access control or for flow control. Therefore some L2 measurement result recorded per UE may be impacted due to the different UE NPN capabilities. Therefore it is necessary to improve the L2 measurement record and report to provide more accurate result to the Trace Collection Entity (TCE). For example, the throughput or the data volume of the NPN capable UE could be better than the non-NPN capable UE, if the resource of the cell cannot satisfy all UE’s requirement. To let the operator know the reason of different L2 measurement result for different UEs, the NPN related information e.g. NPN capability or the NPN/PLMN the UE access to should be sent to the TCE together with the per UE L2 measurement result.

Proposal 8: Report the NPN related information to the TCE together with the L2 measurement, e.g. throughput or data volume measurement.

For other SON related use cases e.g. RACH report, SHR, and MHI, the NPN related enhancement can be introduced by requirements. This can be studied after the NPN related enhancements are identified for the RLF/HOF report and logged MDT.

Proposal 9: NPN enhancements for other SON related use cases (e.g. SHR, MHI) can be further studied after the enhancement to RLF/HOF report and logged MDT are identified.

Common or separate UE variable(s)

The issue of storage method was raised at last meeting, and 2 options can be selected:

- Option 1: Common UE variable is used for NPN and PN network;

- Option 2: Separate UE variables are used for NPN and PN network, respectively.

We think this issue can be discussed for SNPN and PNI-NPN respectively:

Since SNPN is a separate network and different from the PLMN, if a UE switches the network between the PLMN and the SNPN, network registration should be performed. But for the network switching between the PLMN and the PNI-NPN, the same PLMN network can be used and no new registration is needed.

PNI-NPN

The PNI-NPN is a non-public network deployed with the support of a PLMN, so a common PLMN ID as normal PLMN, e.g. HPLMN should be used. Therefore the UE can perform SON/MDT measurement across the normal PLMN and the PNI-NPN with the same PLMN ID. The current UE variable(s) for SON/MDT can be used for PLMN and PNI-NPN recording together, with some PNI-NPN specific enhancement, and no separate UE variables are needed.

Observation 4: PNI-NPN and normal PLMN can use the current UE variable(s) commonly, with some PNI-NPN specific enhancement.

SNPN

For SNPN, it is operated by an NPN operator and not relying on network functions provided by a PLMN [3]. And SNPN is isolate from other networks at least for R16 and R17 since it has no equivalent PLMNs or equivalent SNPNs:

TS 23.501[3]: For a UE registered in an SNPN, the AMF shall not provide a list of equivalent PLMNs to the UE and shall not provide a list of permitted PLMNs to NG-RAN.

And:

TS 24.501[4]: j) equivalent SNPN is not supported;

SA2 has finalised the Rel-18 work on further evolution of NPNs and has agreed to introduce support of equivalent SNPNs. Therefore, if R18 SON/MDT enhancements has the further consideration for subsequent R18 NPN, the “Support for enhanced mobility by enabling support for idle and connected mode mobility between SNPNs without new network selection” should be considered. But the equivalent between SNPN and normal PLMN is still not allowed.

Observation 5: SNPN can have equivalent SNPNs based on SA2 agreement in Rel-18 but has no equivalent PLMNs.

So for MDT, since the MDT PLMN List is a subset of the EPLMN list and RPLMN, the normal PLMN and the SNPN is assumed to be not in the same MDT PLMN List [3].

For logged MDT, the UE shall release the configuration and the measurement upon deregistration [5]:

TS 38.331: 5.5a.2 Release of Logged Measurement Configuration [Omitted part] 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in another RAT. The UE shall also initiate the procedure upon power off or upon deregistration. The UE shall: 1> stop timer T330, if running; 1> if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

The SNPN can be considered as a kind of special PLMN which cannot be equivalent with other PLMNs. Since in CN only one 3GPP access network can be allowed, UE should perform de-registration to the old network and then perform registration to the new network when exchange between PLMN and SNPN, the UE variable in the old network should be removed, and new variables can be built for the logged MDT configuration and for measurement result in the new network.

Observation 6: In current specification, only one 3GPP access network can be allowed, UE should perform de-registration to the old network and then perform registration to the new network when exchange between PLMN and SNPN, and the old UE variable should be removed upon de-registration.

And to save the UE memory, even if the UE does not perform deregistration or re-registration, e.g. change to a PLMN in the equivalent PLMN list but not in the MDT PLMN list, the UE only stores one UE variable for logged MDT [6]:

TS 37.320: 5.1.1.1.2 Configuration effectiveness [Omitted part] In case the new PLMN that does not belong to the MDT PLMN list provides a logged measurement configuration any previously configured logged measurement configuration and corresponding log are cleared and overwritten without being retrieved.

We think this principle is appropriate for all the SON/MDT use cases. If a UE performs SON/MDT measurement in the SNPN, the measurement and recording should not be continued when the UE registers to another PLMN since they have different operators.

For the SON use cases, in legacy specification [5], e.g. RACH report, the information included in the variable will be cleared if the RPLMN or the selected PLMN is not included in plmn-IdentityList stored in the variable.

Therefore in the legacy SON/MDT measurement, more than one UE variable for a specific use case is not allowed for the sake of UE memory saving, but the measurement overwritten or clearing can be occurred if the current PLMN cannot match the logging/recording requirement.

Observation 7: In current specification, more than one UE variable for a specific SON/MDT use case is not allowed for the sake of UE memory saving, and the measurement overwritten or clearing is allowed if the current PLMN cannot match the logging/recording requirement.

The SNPN can be considered as a kind of special PLMN which cannot be equivalent with other PLMNs, thus no more UE variables should be allowed. If the UE switches between the PLMN and the SNPN, the SON/MDT measurement result of the new network can overwrite the old ones. We believe that a smart network can retrieve the required measurement result before being overwritten or cleared. And to align with the checking and reporting of NPN ID which is agreed last meeting, the NPN specific information should also be included in the UE variable. Therefore, the current UE variable(s) for SON/MDT can be reused for SNPN with some SNPN specific enhancement, and no separate UE variables are needed.

To sum up the analyses above, separate UE variable(s) is not allowed or needed for SNPN or for PNI-NPN.

Proposal 10: Do not introduce separate SON/MDT related UE variable(s) for SNPN or for PNI-NPN.

Proposals 11: The current UE variable structure can be enhanced with some SNPN/PNI-NPN specific parameters.

Reply LS to RAN3

RAN3 has set an LS [7] to RAN2 about storing logged MDT results in PN (normal PLMN) and SNPN networks:

In this work, RAN3 noticed that logged MDT reports collected in one type of network, e.g. an SNPN, are lost when UE moves to a network of another type, e.g. a Public Network, due to the UE deregistering from the SNPN network. RAN3 believes that this would lead to a loss of the stored logged MDT reports if the reports are not retrieved before moving to the network of another type. RAN3 would like to check with RAN2 if there are any solutions to avoid the loss of stored logged MDT reports upon moving from a network of one type to another type, even upon deregistration.

For this issue, the logged MDT results of PN and SNPN may be lost when registering to a new network due to the UE should remove the recorded result upon deregistering from the old network. This is not a special case for PN/SNPN network. In the legacy principle of MDT, when a UE registers to a network other than the PLMN in the equivalent PLMN list, the UE should remove relevant MDT result either. Since at least in Release 18, the SNPN cannot have equivalent PLMNs, we do not think the result should be maintained when the UE move across PN/SNPN network. The logging and reporting of PN and SNPN can be performed by different UEs, which seem not necessary to enhance the continuous recording between PN and SNPN networks.

Proposal 12: RAN2 does not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3.

Conclusion

According to the analysis in section 2, we propose:

For logged MDT

Observation 1: There is no NCE or extension marker in current AreaConfiguration IE.

Proposal 1: RAN2 should include the 3 cases of PNI-NPN only/PN only/ both PN and PNI-NPN in the MDT area scope, and a critical extension (i.e. AreaConfiguration-r18) can be considered in R18.

Proposal 2: Introduce the definition of MDT SNPN list, and add a NOTE to clarify that “MDT SNPN list can only include one SNPN ID in this Release”.

Proposal 3: Only the UE registered in SNPN can be configured with this SNPN related logged MDT, and the UE can continue logging across the MDT SNPN list.

Proposal 4: The solution of adding the SNPN ID in the area scope configuration should align with the RAN3 BLCR.

Observation 2: The measurement result of logged MDT is the result of the cell which is used to draw a picture of the channel power/quality, and the operator can easily obtain the network/cell type information of the cell without UE reporting.

Proposal 5: Do not add NPN ID related information (e.g. NID/CAG-ID) or cell type (e.g. NPN cell indicator) into the logged MDT report.

For RLF report

Observation 3: The PLMN ID parts of PNI-NPNs are already set in the IE structure (CGI-Info-Logging-r16) of failedPCellId and the previousPCellId fields.

Proposal 6: Included PNI-NPN IDs (e.g. cag-IDs) into the RLF/HOF Report of the failed cell and the previous Pcell, respectively.

Proposal 7: Included UE's allowed CAG list at the time of RLF/HOF occurring into the RLF/HOF Report.

For other SON/MDT measurements

Proposal 8: Report the NPN related information to the TCE together with the L2 measurement, e.g. throughput or data volume measurement.

Proposal 9: NPN enhancements for other SON related use cases (e.g. SHR, MHI) can be further studied after the enhancement to RLF/HOF report and logged MDT are identified.

For common or separate UE variable(s)

Observation 4: PNI-NPN and normal PLMN can use the current UE variable(s) commonly, with some PNI-NPN specific enhancement.

Observation 5: SNPN can have equivalent SNPNs based on SA2 agreement in Rel-18 but has no equivalent PLMNs.

Observation 6: In current specification, only one 3GPP access network can be allowed, UE should perform de-registration to the old network and then perform registration to the new network when exchange between PLMN and SNPN, and the old UE variable should be removed upon de-registration.

Observation 7: In current specification, more than one UE variable for a specific SON/MDT use case is not allowed for the sake of UE memory saving, and the measurement overwritten or clearing is allowed if the current PLMN cannot match the logging/recording requirement.

Proposal 10: Do not introduce separate SON/MDT related UE variable(s) for SNPN or for PNI-NPN.

Proposals 11: The current UE variable structure can be enhanced with some SNPN/PNI-NPN specific parameters.

For the reply LS to RAN3

Proposal 12: RAN2 does not enhance the continuous logged MDT recording between PN and SNPN networks, and send RAN2 decision to RAN3.

Reference

- R3-233470 (TP for MDT BL CR to TS 38.413) Support of SNPN in MDT area scope Nokia, Nokia Shanghai Bell

- R2-2308623 Running 38.331 CR for logged MDT enhancements and NPN Huawei, HiSilicon

- TS23.501-h60

- TS24.501-h80

- TS38.331-h20

- TS37.320-h10

- R3-232118 LS on potential override of logged MDT reports upon moving from SNPN to PLMN RAN3

Annex - TP for MDT BL CR to TS 38.413 (R3-233470)

<<< start of changes >>>

#### 9.3.1.169	MDT Configuration-NR

This IE defines the MDT configuration parameters of NR.

| IE/Group Name | Presence | Range | IE type and reference | Semantics description | Criticality | Assigned Criticality |
| --- | --- | --- | --- | --- | --- | --- |
| MDT Activation | M |  | ENUMERATED (Immediate MDT only, Logged MDT only, Immediate MDT and Trace, …) |  | - |  |
| CHOICE Area Scope of MDT | M |  |  |  | - |  |
| >Cell based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - |  |
| >TA based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TA List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAC | M |  | 9.3.3.10 | The TAI is derived using the current serving PLMN. | - |  |
| >PLMN wide |  |  | NULL | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >TAI based |  |  |  | If PNI-NPN Area Scope of MDT IE is present, it covers non-CAG cells only, where non-CAG cells refer to cells that only provide public access. |  |  |
| >>TAI List for MDT |  | 1..<maxnoofTAforMDT> |  |  |  |  |
| >>>TAI | M |  |  |  | - |  |
| >PNI-NPN Based MDT |  |  |  |  | YES | Ignore |
| >>CAG List for MDT |  | 1..<maxnoofCAGforMDT> |  |  |  |  |
| >>>PLMN ID |  |  | 9.3.3.5 |  |  |  |
| >>>CAG ID | M |  | 9.3.3.43 |  |  |  |
| >SNPN Cell Based MDT |  |  |  |  | YES | ignore |
| >>SNPN Cell ID List for MDT |  | 1..<maxnoofCellIDforMDT> |  |  |  |  |
| >>>NR CGI | M |  | 9.3.1.7 |  | - | - |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the NR CGI IE. | - | - |
| > SNPN TAI Based MDT |  |  |  |  | YES | ignore |
| >>SNPN TAI List |  | 1..<maxnoofTAforMDT> |  |  | - | - |
| >>>TAI | M |  | 9.3.3.11 |  | - | - |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity in the TAI IE. | - | - |
| > SNPN Based MDT |  |  |  |  |  |  |
| >>MDT SNPN List |  | 1..<maxnoofMDTSNPNs> |  |  |  |  |
| >>>PLMN Identity | M |  | 9.3.3.5 |  |  |  |
| >>>NID | M |  | 9.3.3.42 | Identifies an SNPN together with the PLMN Identity IE. |  |  |
| CHOICE MDT Mode | M |  |  |  | - |  |
| >Immediate MDT |  |  |  |  |  |  |
| >>Measurements to Activate | M |  | BITSTRING (SIZE(8)) | Each position in the bitmap indicates a MDT measurement, as defined in TS 37.320 [41]. First Bit = M1, Second Bit= M2, Third Bit = M4, Fourth Bit = M5, Fifth Bit = M6, Sixth Bit = M7, Seventh Bit = logging of M1 from event triggered measurement reports according to existing RRM configuration, other bits reserved for future use. Value “1” indicates “activate” and value “0” indicates “do not activate”. | - |  |
| >>M1 Configuration | C-ifM1 |  | 9.3.1.171 |  | - |  |
| >>M4 Configuration | C-ifM4 |  | 9.3.1.172 |  | - |  |
| >>M5 Configuration | C-ifM5 |  | 9.3.1.173 |  | - |  |
| >>M6 Configuration | C-ifM6 |  | 9.3.1.174 |  | - |  |
| >>M7 Configuration | C-ifM7 |  | 9.3.1.175 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>MDT Location Information | O |  | 9.3.1.176 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >Logged MDT |  |  |  |  |  |  |
| >>Logging interval | M |  | ENUMERATED (320ms, 640ms, 1280ms, 2560ms, 5120ms, 10240ms, 20480ms, 30720ms, 40960ms, 61440ms, infinity, …) | This IE is defined in TS 38.331 [18]. | - |  |
| >>Logging duration | M |  | ENUMERATED (10, 20, 40, 60, 90,120, …) | This IE is defined in TS 38.331 [18]. Unit: [minute]. | - |  |
| >>CHOICE Report Type | M |  |  |  | - |  |
| >>>Periodical |  |  | NULL |  |  |  |
| >>>Event Triggered |  |  |  |  |  |  |
| >>>>Event Trigger Logged MDT Configuration | M |  | 9.3.1.180 |  | - |  |
| >>Bluetooth Measurement Configuration | O |  | 9.3.1.177 |  | - |  |
| >>WLAN Measurement Configuration | O |  | 9.3.1.178 |  | - |  |
| >>Sensor Measurement Configuration | O |  | 9.3.1.179 |  | - |  |
| >>Area Scope of Neighbour Cells | O |  | 9.3.1.182 |  | - |  |
| >>Early Measurement | O |  | ENUMERATED (true, ...) | This IE indicates whether the UE is allowed to log measurements on early measurement related frequencies in logged MDT as specified in TS 38.331 [18]. | YES | ignore |
| Signalling Based MDT PLMN List | O |  | MDT PLMN List 9.3.1.168 |  | - |  |
| PNI-NPN Area Scope of MDT | O |  | 9.3.3.x |  | YES | Ignore |

| Range bound | Explanation |
| --- | --- |
| maxnoofCellIDforMDT | Maximum no. of Cell ID subject for MDT scope. Value is 32. |
| maxnoofTAforMDT | Maximum no. of TA subject for MDT scope. Value is 8. |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT scope. Value is 256. |
| maxnoofMDTSNPNs | Maximum no. of SNPNs in the MDT SNPN list. Value is FFS. |

| Condition | Explanation |
| --- | --- |
| C-ifM1 | This IE shall be present if the Measurements to Activate IE has the first bit set to “1”. |
| C-ifM4 | This IE shall be present if the Measurements to Activate IE has the third bit set to “1”. |
| C-ifM5 | This IE shall be present if the Measurements to Activate IE has the fourth bit set to “1”. |
| C-ifM6 | This IE shall be present if the Measurements to Activate IE has the fitth bit set to “1”. |
| C-ifM7 | This IE shall be present if the Measurements to Activate IE has the sixth bit set to “1”. |

Next Change

#### 9.3.3.x	PNI-NPN Area Scope of MDT

| IE/Group Name | Presence | Range | IE type and reference | Semantics description |
| --- | --- | --- | --- | --- |
| CAG List for MDT |  | 1..< maxnoofCAGforMDT > |  |  |
| >PLMN Identity | M |  | 9.3.3.5 |  |
| >CAG ID | M |  | 9.3.3.43 |  |

| Range bound | Explanation |
| --- | --- |
| maxnoofCAGforMDT | Maximum no. of CAG IDs for MDT area scope. Value is 256. |

<<< end of changes >>>