3GPP TSG-RAN WG2 Meeting #123                                                                     	R2-2307711

Toulouse, France, 21st– 25th Aug, 2023

Source:	CATT

Title:	Discussion on Fast MCG Recovery MRO Enhancement

Agenda Item:	7.13.8

Document for:	Discussion and Decision

Introduction

The agreement about Fast MCG recovery MRO enhancement in last meeting is adding the scenario of near failure (case c) and case f1:

Agreements

1	RAN2 confirms scenario of near failure fast MCG recovery.

2	RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. FFS RAN2 impact.

From the RAN3 suggested UE report contents in the incoming LS and in the agreements of previous RAN3 meeting, we consider the corresponding scenarios could be clearly decided in RAN2. Therefore in this contribution, first we want to compare and make align about the scenarios/cases in RAN2/3 to make clear which scenarios are missed by RAN2, and then we intend to discuss the detailed enhanced content about Fast MCG recovery MRO.

Discussion

Scenarios clarification

Unaligned scenarios by RAN2 and RAN3

Based the respective discussion by RAN2 and RAN3 group meetings, RAN2 and RAN3 have some misalignment about the fast MCG recovery MRO enhancement scenarios. So we need to compare and make align about the scenarios/cases first.

The RAN3 cases are excerpted from the agreements/WF[1]/LS[2] from RAN3#117 meeting until now, and the RAN2 scenarios are excerpted from the agreements from RAN2#120 meeting until now.

Table 1 Comparison between RAN3 cases and RAN2 scenarios

|  | RAN3 cases description | RAN2 scenarios mapping |
| --- | --- | --- |
| Case a | Agreed in RAN3#117: SCG fails or is deactivated when the UE attempts MCG recovery (i.e. a SCG failure/deactivation while T316 is running after MCG failure) Redefined in RAN3#119bis: SCG fails when the UE is undergoing fast MCG recovery (i.e. SCG failure happens while T316 is running). | Agreed in RAN2#120: SCG failure/deactivation during fast MCG recovery (i.e., running of T316). Misaligned scenario: RAN3 has removed “SCG deactivation during fast MCG recovery” scenario since it does not exist. |
| Case b | Agreed in RAN3#117: the signalling delay is longer than the time the UE waits for the response (T316 expired); WF in RAN3#117bis: Sub-Case b1: T316 runs out on the UE side while the SN is trying to deliver the MN message, in this case the maximum number of retransmissions at the SN side has not been reached. Sub-Case b2: The SN reaches the maximum number of retransmissions while T316 has not expired on the UE side. In this case the SN can not make any further attempts to deliver the MN message but the UE will continue to wait for it for the remainder of the T316 time. Agreed in RAN3#119: Sub-Case b1/Sub-Case b2 would not be considered for MRO for fast MCG recovery failure. | Agreed in RAN2#120: T316 expiry Align with RAN3. |
| Case c | WF in RAN3#117bis: Fast recovery near failure case, i.e. UE receives the response message from MN via SN while T316 is running which almost expires but not yet. Agreed in RAN3#120: Agree to define case c in R18. Solution can be further discussed. | Agreed in RAN2#122: RAN2 confirms scenario of near failure fast MCG recovery. Align with RAN3. |
| Case d | WF in RAN3#117bis: Failure case for CHO based recovery failure after fast MCG recovery failure. This scenario is not confirmed by RAN3 yet. | Also not considered by RAN2 yet. |
| Case e | WF in RAN3#117bis: Subsequent failure after successful fast MCG recovery. This scenario is not confirmed by RAN3 yet. | Also not considered by RAN2 yet. |
| Case f | WF in RAN3#117bis: Dual failure case, i.e. MCG failure occur while at about the same time SCG is deactivated/suspended/de-configured. Only case f1 is agreed in RAN3#119bis: where the SCG fails or is deactivated yet before the UE sends the MCGFailureInformation is to be addressed. | Agreed in RAN2#122: RAN2 confirms scenario f1, i.e., SCG fails or is deactivated before the UE sends the MCGFailureInformation. Align with RAN3. |
| Other case | Agreed LS R3-230992 in RAN3#119: - the cause of the fast MCG recovery failure containing at least: - T316 expiry, - SCG failure, and - SCG was deactivated or other cases where SCG is not available | Misaligned scenario: “PSCell change /addition ongoing before fast MCG recovery” scenario mentioned below. |

From the comparison in the table above, we can see that there are two cases not align by RAN2 and RAN3:

- Unaligned scenario 1: RAN3 has removed “SCG deactivation during fast MCG recovery” scenario since it does not exist, but RAN2 still has it.

- Unaligned scenario 2: “PSCell change /addition ongoing before fast MCG recovery” which is suggested by RAN3 LS is not covered by RAN2.

We list our analysis about these 2 scenarios as below:

Unaligned scenario 1: The RAN2 already agreed SCG deactivation scenario

RAN3 already remove the scenario of “SCG deactivation during fast MCG recovery (i.e., running of T316)”. But here we still provide some analysis that why this scenario does not exist and should be removed:

From RAN2 technical point of view, for SCG deactivation case, it is the MN’s responsibility to send the SCG deactivation configuration, and the SCG deactivation cannot be configured to the UE if the MCG link is broken:

- If the MCG failure occurs, the UE shall suspend all RBs except SRB0 and reset MCG MAC, therefore any transmissions in UL and DL cannot be performed based on the descriptions in TS38.331:

5.7.3b MCG failure information [Omitted 5.7.3b.1] 5.7.3b.2 Initiation A UE configured with split SRB1 or SRB3 initiates the procedure to report MCG failures when neither MCG nor SCG transmission is suspended, the SCG is not deactivated, t316 is configured, and when the following condition is met: 1> upon detecting radio link failure of the MCG, in accordance with 5.3.10.3, while T316 is not running. Upon initiating the procedure, the UE shall: 1> stop timer T310 for the PCell, if running; 1> stop timer T312 for the PCell, if running; 1> suspend MCG transmission for all SRBs, DRBs, multicast MRBs, except SRB0, and, if any, BH RLC channels; 1> reset MCG MAC; [Omitted part]

- It is also not allowed to use split SRB1 to transmit the SCG deactivation command due to the descriptions in TS37.340:

7.13 Activation and Deactivation of SCG To enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured, an activation/deactivation mechanism of SCG is supported. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated. Upon SCG deactivation and while the SCG is deactivated, the network ensures that there is no uplink control PDU transmission to the deactivated SCG (e.g. the network releases statusReportRequired from PDCP entities of SCG bearers if configured, the network does not perform QoS flow remapping from a DRB associated to the deactivated SCG to another DRB). The network ensures the SCG is activated while PDCP duplication is activated for SCG RLC entities associated with a PDCP entity. NOTE: Upon SCG (de)activation, it is up to the network to ensure there is no pending SDUs or PDUs in SCG RLC entity (e.g. instructs the UE to perform PDCP data recovery and RLC re-establishment/release, if needed). [Omitted part]

The timer T316 can only be started if the UE send the MCGFailureInformation message, so “SCG deactivation during fast MCG recovery (i.e., running of T316)” is not a valid scenario based on current spec.

Observation 1: “SCG deactivation during fast MCG recovery (i.e., running of T316)” scenario has been removed from the RAN3 suggest cases, and it is also not a valid scenario from technical point of view in RAN2.

Proposal 1: RAN2 to clarify the “SCG deactivation during fast MCG recovery” is not a valid scenario.

Unaligned scenario 2: PSCell change /addition ongoing scenario

The MCG failure may occur when PSCell change/addition ongoing. For PSCell change/addition the UE may receive the RRCReconfiguration message, and then to apply the corresponding action. Since the addition or change of the PSCell need some processing delay time, it is possible that the UE needs to send the MCGFailureInformation message but the SCG link has not been prepared completely. This scenario can be considered as the same type as the “SCG deactivation before fast MCG recovery” scenario, and it is corresponding to the case of “other cases where SCG is not available” in RAN3 LS [2].

Observation 2: “PSCell change/addition ongoing before fast MCG recovery” scenario is corresponding to “other cases where SCG is not available” cause of the fast MCG recovery failure in RAN3 LS.

If the UE receives the PSCell change after T316 is running, the UE may still not receive the HO command for recovery. But in this case, the T316 will run to expire, so it is a sub-case for “T316 expire” scenario.

Proposal 2: Add the scenario “PSCell change /addition ongoing before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 LS.

Other potential scenarios in RAN2

Furthermore, some potential scenarios not mentioned by RAN3 are also proposed by companies in RAN2:

Successful scenario

The fast MCG link recovery is introduced in R16. The specification mentioned that:

- If radio link failure occurred in MCG, first the MCG failure should be recorded in VarRLF-Report, and then UE should initiate the MCG failure information procedure if condition fulfilled;

- If MCG failure is sent to the network and if any of the response message of RRCRelease/ MobilityFromNRCommand/RRCReconfiguration with reconfigurationwithSync for the PCell is received before T316 expired, the VarRLF-Report will be cleared.

Observation 3: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

The UE action of clearing the VarRLF-Report upon successful fast MCG recovery is agreed in R16. The failure scenarios can have emergency problem which needs to be solved first and foremost. Furthermore, in RAN3 LS [2] it is only mentioned the failure information by UE reporting. Therefore, we consider the enhancement for failure scenarios of fast MCG recovery should be discussed firstly in R18.

Proposal 3: Scenarios involve fast MCG recovery failure are prioritized in R18.

HO command for recovery failure after the MCG failure scenario

In addition, the network may send a HO command after receiving the MCGFailureInformation message for recovery. But if the UE cannot successfully apply the received HO command, the fast MCG recovery failure may also occur. Since the UE can only store one entry of RLF report content, the HOF information will override the previous MCG failure information. To keep the information integrity for network side analysis, the consecutive failures are necessary to be recorded by the UE for the MRO usage.

Proposal 4: Add the consecutive failure scenario of “HO command for recovery failure after MCG RLF”.

Detailed reported content for fast MCG recovery MRO

For current agreed scenarios and the suggested scenarios which need to be align with RAN3 LS [2], we think the UE recorded content can be discussed for 1) only report MCG failure and for 2) report both MCG and SCG failure. Here we only involve the MCG failure cases, and think the near failure case is another issue which may have no relationship with the failure cases. The figure below illustrates the classification of different scenarios:

Figure 1 Classification of different scenarios and the MCG/SCG failure information

1) Report only MCG failure

For all “Report only MCG failure” cases, there is no SCG failure related information recorded in the UE, so the legacy MCG RLF/HOF content could cover most of the failure information. Only MCG failure includes possible cases as below:

- T316 expires;

For this case, the UE could store the VarRLF-Report based on HOF or RLF, and has already sent the MCGFailureInformation message. Based on current specification, if no response message such as RRCRelease/MobilityFromNRCommand/RRCReconfiguration with reconfigurationwithSync for the PCell is received before T316 expired, the VarRLF-Report will be maintained, and the UE will initiate the re-establishment procedure. So to distinguish the legacy RLF/HOF report and the fast MCG recovery failure report, some information should be added into the VarRLF-Report, e.g. running time of T316;

- SCG cannot be used, i.e. SCG deactivated, or PSCell addition/change is ongoing when UE detects MCG failure;

For this case, the SCG is not failed but cannot be used due to some other causes. UE will not start the “Fast MCG recovery procedure”, and directly perform the re-establishment procedure. If T316 value has been configured to the UE, it is the network’s opinion to recover the MCG link via SCG. Therefore it is better to report to the network the MCG failure with the cause that why the SCG link is unusable.

Proposal 5: UE report only MCG failure related content to the network for the scenarios below:

- T316 expires;

- SCG cannot be used without failure, i.e. SCG deactivated, or PSCell addition/change is ongoing.

Proposal 6: For only MCG failure, record the possible content to distinguish with the legacy RLF/HOF in MCG, e.g. to record and report the running time of T316, the SCG unusable cause and the PSCell ID if any.

- In addition, the HO command for recovery failure after the MCG RLF scenario can be considered as a potential scenario.

For this case, the UE has successfully sent the MCGFailureInformation message, and the network also sends the HO command to the UE. If the UE cannot successfully apply the received HO command, the fast MCG recovery failure may also occur. Since the UE can only store one entry of RLF report content, the HOF information will override the previous MCG failure information. To keep the information integrity for network side analysis, the consecutive failures are necessary to be recorded by the UE for the MRO usage. Based on current MCG RLF report information, at least the cell ID in the HO command should be recorded and reported together with the legacy MCG RLF.

Proposal 7: Record the consecutive failure information for “HO command for recovery failure after MCG RLF” scenario.

2) Report both MCG and SCG failures

Report both MCG and SCG failure includes 2 cases as below:

- SCG failure during fast MCG recovery (i.e., running of T316). The MCG link cannot be successfully recovered since SCG is unusable, even if the MCGFailureInformation message has been successfully sent to the network. The already recorded MCG failure related information needs to be report to the network, maybe binding with the failed SCG information for further MRO usage;

- SCG failure before MCG failure. This scenario includes the cases that MCG failure occurs after e.g. SCG RLF, PSCell change failure. In this case, the already recorded SCG failure related information needs to be report to the network, maybe binding with the failed MCG information.

For both cases, to distinguish the scenarios of whether the SCG failure is occurred before/during fast MCG recovery, it is proposed also to include other information besides the MCG/SCG failure information itself to indicate the relationship between the two failures, e.g. which leg failure first indicator, the time elapsed from one failure to the other failure.

Proposal 8: UE report both MCG and SCG failure related content to the network for the scenarios below:

- SCG failure during fast MCG recovery (i.e., running of T316);

- SCG failure before MCG failure.

Proposal 9: For both MCG and SCG failures, record and report the PSCell ID, SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Proposal 10: For both MCG and SCG failures, record the relationship between the two failures in RLF report for both MCG and SCG failure case, e.g. indicate which leg failed first, and the time elapsed between the two failures.

Conclusion

According to the analysis in section 2, we propose:

Scenarios clarification:

Observation 1: “SCG deactivation during fast MCG recovery (i.e., running of T316)” scenario has been removed from the RAN3 suggest cases, and it is also not a valid scenario from technical point of view in RAN2.

Proposal 1: RAN2 to clarify the “SCG deactivation during fast MCG recovery” is not a valid scenario.

Observation 2: “PSCell change/addition ongoing before fast MCG recovery” scenario is corresponding to “other cases where SCG is not available” cause of the fast MCG recovery failure in RAN3 LS.

Proposal 2: Add the scenario “PSCell change /addition ongoing before fast MCG recovery (when UE detects MCG failure)” to align with RAN3 LS.

Observation 3: UE action of clearing the VarRLF-Report upon successful fast MCG recovery is supported in R16.

Proposal 3: Scenarios involve fast MCG recovery failure are prioritized in R18.

Proposal 4: Add the consecutive failure scenario of “HO command for recovery failure after MCG RLF”.

Detailed reported content for fast MCG recovery MRO

Proposal 5: UE report only MCG failure related content to the network for the scenarios below:

- T316 expires;

- SCG cannot be used without failure, i.e. SCG deactivated, or PSCell addition/change is ongoing.

Proposal 6: For only MCG failure, record the possible content to distinguish with the legacy RLF/HOF in MCG, e.g. to record and report the running time of T316, the SCG unusable cause and the PSCell ID if any.

Proposal 7: Record the consecutive failure information for “HO command for recovery failure after MCG RLF” scenario.

Proposal 8: UE report both MCG and SCG failure related content to the network for the scenarios below:

- SCG failure during fast MCG recovery (i.e., running of T316);

- SCG failure before MCG failure.

Proposal 9: For both MCG and SCG failures, record and report the PSCell ID, SCG failure type (at least t310-Expiry, randomAccessProblem, rlc-MaxNumRetx).

Proposal 10: For both MCG and SCG failures, record the relationship between the two failures in RLF report for both MCG and SCG failure case, e.g. indicate which leg failed first, and the time elapsed between the two failures.

Reference

- R3-226004 WF on the MRO scenarios Lenovo

- R3-230992 LS on MRO for CPC and CPA and fast MCG recovery RAN3