3GPP TSG-RAN WG2 Meeting #123	 R2-2307715

Toulouse, France, August 21 – 25, 2023

Source:	KDDI Corporation

Title:	Discussion on NR-DC with selective activation cell of groups

Tdoc Type:	Discussion

Agenda Item:	7.4.3 NR-DC with selective activation cell of groups

Document for:  Discussion and decision

Release:        Rel-18

1	Introduction

The following open issues related to NR-DC with selective activation of cell groups were listed in RP-231311. In this contribution, we give our views on some of these open issues to progress the work.

- Whether to define a term for NR-DC with selective activation of cell group and what the term is. - Whether to support selective activation for MCG. - How many subsequent conditional changes are targeted, and potential impacts. - Security issues, LS sent to SA3 asking the existing handling of sk-counter/ S-KgNB is applicable when UE continues switching between the candidate target PSCells multiple times. - FFS on the following options for subsequent CPC: - Option 1: Source MN generates the execution conditions for all subsequent CPC. - Option 2: Candidate SN may generate execution conditions for subsequent CPC. - FFS if Candidate SN may generate/modify execution conditions for subsequent CPC - Reference SCG configuration (Optionality FFS). Assume as for LTM Reference configuration may be empty. - FFS whether MCG configuration is included. - FFS RRC model for the reference configuration. - For MN initiated procedure, execution conditions based on event A4 are supported. FFS whether A3/A5 are supported.

2	Discussion

2.1 Selective Activation for MCG

Regarding to whether supporting selective activation for MCG, we think this feature will be beneficial on reducing the configuration signalling for candidate cell which can be used for subsequent CHO and the framework of SCPAC can be the baseline for the selective activation for MCG. However, considering that there are only three RAN2 meetings left, we suggest to not supporting it in Rel-18, but left for Rel-19.

Proposal 1: Selective activation for MCG is not supported in Rel-18

2.2 Execution Condition for MN-initiated Subsequent CPAC

For the SN-initiated SCPAC, it has been agreed that the candidate SN can generate execution condition for subsequent CPC. Generally, it is the serving cell generate the execution condition for the candidate cell, in subsequenWe think the same principle can apply to the MN-initiated subsequent CPAC as well.

Regarding the conditional events for MN-initiated SCPAC, as the MN does not manage the subsequent candidate SN, to prevent unnecessary handover or unstable mobility between the prepared candidate cells, it is better that the UE can be configured with A3/A5 for the subsequent candidate cells. If RAN2 agree that the candidate SN can generate execution conditions, it can provide the associated A3/A5 for each candidate cell.

Proposal 2: For MN-initiated subsequent CPAC, RAN2 allow the candidate SN to generate execution conditions for the subsequent CPC

Proposal 3: For MN-initiated subsequent CPAC, the execution condition of A3/A5 can be supported and provided by the candidate SN

2.3 Reference configuration

In the last RP#100, companies discussed whether to down scope the use of reference config and delta config for both LTM objective and the NR-DC selective activation cell group objective. In our understanding the purpose of LTM is to faster the cell switch procedure than L3 handover and thus reduce the connectivity interruption, therefore, the reference config and the delta config does not contribute to the objective, and we are fine to remove it from the LTM objective. The purpose of NR-DC selective activation is to reduce the reconfiguration signalling overhead by maintaining the candidate cells config even after changing CG without reconfiguration. From the signalling overhead perspective, it is beneficial to have the reference config and delta config.

Proposal 4: RAN2 agree to at least support reference config and delta config for NR-DC with selective activation cell groups.

The legacy CPC principle is the CPC configuration contains the configuration of CPC candidates and execution conditions and may contain the MCG configuration for inter-SN CPC, due to coordination between the MCG and SCG, such as some radio bearer changes. Therefore, it is reasonable to include MCG configuration in the reference configuration

Proposal 5: RAN2 agree to include MCG configuration in the reference configuration

2.4 Subsequent CPAC release

RAN2 has agreed to release a CPC candidate explicitly has been agreed. We think no matter in which case, e.g upon SCG release, upon going to RRC idle, or upon reception of RRC release etc, the subsequent CPAC is released explicitly. Besides, if RAN2 agree to use the reference config and delta config as proposed in P2, the associated reference config can be released as well in those cases mentioned above.

Proposal 6: Subsequent CPAC config and the associated reference config are released explicitly in cases, e.g. upon SCG release, going to RRC idle, reception of RRC release, etc.

3	Conclusion

In this paper, we share our views and give the proposals below:

Proposal 1: Selective activation for MCG is not supported in Rel-18

Proposal 2: For MN-initiated subsequent CPAC, RAN2 allow the candidate SN to generate execution conditions for the subsequent CPC

Proposal 3: For MN-initiated subsequent CPAC, the execution condition of A3/A5 can be supported and provided by the candidate SN

Proposal 4: RAN2 agree to at least support reference config and delta config for NR-DC with selective activation cell groups.

Proposal 5: RAN2 agree to include MCG configuration in the reference configuration

Proposal 6: Subsequent CPAC config and the associated reference config are released explicitly in cases, e.g. upon SCG release, going to RRC idle, reception of RRC release, etc.