3GPP TSG RAN2 Meeting #123                                      R2-2307716

# 21nd Aug – 25th Aug 2023, Toulouse, France

Agenda item:	7.9.2

Source:	TCL

Title:  	Discussion on U2U relay

Document for: 	Discussion

Introduction

Support of UE-to-UE relay is essential for the sidelink coverage extension without relying on the use of uplink and downlink. In R18 SL relay WID[1], the objective for U2U relay has been listed as follow,

Specify mechanisms to support single-hop Layer-2 and Layer-3 UE-to-UE relay (i.e., source UE -> relay UE -> destination UE) for unicast [RAN2, RAN3, RAN4].

  - Common part for Layer-2 and Layer-3 relay to be prioritized until RAN#98

    - Relay discovery and (re)selection [RAN2, RAN4]

    - Signalling support for Relay and Source UE authorization if SA2 concludes it is needed [RAN3]

  - Layer-2 relay specific part

    - UE-to-UE relay adaptation layer design [RAN2]

    - Control plane procedures [RAN2]

    - QoS handling if needed, subject to SA2 progress [RAN2]

Note 1A: This work should take into account the forward compatibility for supporting more than one hop in a later release.

Note 1B: A Source UE is connected to only a single relay UE at a given time for a given destination UE.

Meanwhile in the last #122 meeting, RAN2 has also made some agreements as follow:

Agreements:

For Model A discovery, the relay UE should only announce the neighbour UEs for which the SD-RSRP/SL-RSRP between the relay UE and the neighbour UE is above a configured threshold in a discovery announcement message. LS is sent to SA2.

For Model A discovery, upon discovery message reception, remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For Model B discovery, upon discovery response messages reception, the source remote UE considers a relay UE as a candidate relay UE if the SD-RSRP towards the relay UE is above a configured threshold.

For integrated-discovery, when receiving DCR message from one or multiple relay UEs, the target remote UE should consider candidate relay UEs towards which the SL-RSRP is above a configured threshold to respond and that satisfy upper-layer criteria, and select a relay UE from among them.

For non-integrated U2U relay discovery model B, when relay (re)selection is triggered at the remote UE, the discovery transmission may be triggered at the same time to search for candidate relay UEs.

Agreements:

Separate thresholds for SL-RSRP and SD-RSRP are configured for the trigger of U2U relay (re)selection.

Besides the PC5 link quality, RAN2 does not pursue other AS criteria for relay (re)selection.

Agreements:

For the E2E SL-SRB configuration of U2U relay, specified PDCP configuration is used. FFS for the SRAP and PC5 RLC channel configuration for SL-SRB.

AS layer is responsible for QoS split in L2 U2U relay.

Relay UE is responsible for AS layer QoS split in L2 U2U relay.

For OOC U2U relay/remote UE, pre-configuration is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

For RRC_IDLE/INACTIVE U2U relay/remote UE, SIB is used for the E2E SL-DRB and per-hop PC5 RLC channel configuration.

Agreements:

For the possible use of a short ID in U2U relay, RAN2 will downselect between the following options for identifying the source and destination remote UEs at the SRAP layer:

a)	Single ID, identifying the source and destination remote UEs

b)	Source ID and Destination ID

For the possible use of a short ID in U2U relay, the U2U relay UE performs the ID assignment. FFS if this ID should be assigned hop-by-hop or globally.

These agreements do not imply agreement to use a short ID.

Based on above agreements and legacy issues, we continue to discuss some key issues and remaining FFS discussion about U2U discovery, adaptation layer, QoS flow and security as well.

Discussion

Discovery

As one remote UE can only select one relay UE, SA2 has defined that source remote UE is responsible for relay UE selection in both Model A and Model B. In R17 U2N, RAN2 has defined relay selection is based on source UE implementation. Thus, in R18 U2U relay, we can have similar conclusion like R17 U2N: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

Proposal 1: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

RAN2 has discussed whether to support the co-existence between the U2N relay and the U2U relay in the previous meetings. Given the limited capability of a remote UE, a remote UE is only allowed to have one relay in Rel_18, in addition, the coexistence may have many effects on resources, the design of AS layer and the handling of priority, so in the scope of this WI, we don’t need to achieve the coexistence between the U2N relay and the U2U relay.

Proposal 2: In this release, RAN2 does not need to achieve the coexistence between the U2N relay and the U2U relay.

The adaptation layer

The most important issue we think is to reduce signaling overhead in SRAP UE ID allocation. For the U2N relay, there exists a local ID in the SRAP layer to identify the remote UE between the relay UE and the network, which can achieve correct data transmissions. To achieve this function, a similar design should be considered in the U2U relay. Considering signaling overhead and the scalability of multi-hop scenarios, to reduce signaling overhead, 8 bit local UE IDs can be reused just like what we did in R17 U2N relay.

Proposal 3: RAN2 is suggested to take local ID used in the SRAP header.

In addition, as the U2U relay is different with U2N relay, both the source entity and the destination entity are UE, source remote UE and target remote UE, so it should be discussed which UE ID should be included in the SRAP header.

Our understanding is in the legacy sidelink transmission, both source UE ID and target UE ID are needed. So we suggest to extend the UE ID to both source remote UE ID and target remote UE ID.

Proposal 4: For the U2U relay, the UE ID in the SRAP header should be extended to both source remote UE ID and target remote UE ID.

As in Rel_18, only one hop relay is considered for U2U relay. The whole link of a U2U relay is: source remote UE -> Relay UE -> destination relay UE. So it is quite straightforward that the local UE ID should be assigned by the relay UE. There should be a procedure that how the local UE ID is assigned by the relay UE, and the details of how the relay UE decides the local UE ID to the source UE and the target UE.

Proposal 5: For the U2U relay, the local IDs should be assigned by the relay UE, details are FFS.

QoS handling

The QoS for E2E SL relay is split into two parts for SLRBs for U2U Relay. One is for the PC5 interface between source UE and Relay UE (source side), and the other is for the PC5 interface between Relay UE and the target UE (target side). These two parts of links have no management from the gNB, and when and which node to implement QoS split is necessary to be discussed to guarantee the end-to-end QoS needs.

According to legacy mechanisms in Rel-17 5G ProSe, using PC5-RRC/PC5-S procedure to achieve E2E QoS splitting for the UE-to-UE Relay is the basic principle. Given in E2E QoS splitting issue, RRC layer takes charge of the bearer management, we think RRC layer should be able to handle the QoS split, and RRC layer is more flexible. In addition, we need to consider different PC5 links condition to decide QoS split firstly regardless which hop to perform QoS split.

Proposal 6: AS layer is responsible for QoS split in L2 U2U relay.

Since the relay UE can more easily know the status of the two hops, we think it’s more efficient for the relay UE to split the QoS profiles. The relay UE can establish SLRB with two end remote UEs separately, and relay UE can decide separate QoS of these two hops, to fulfill the end to end QoS. So it is up to the implementation of relay UE to decide to hop by hop QoS to fulfill the end to end QoS.

Proposal 7: Relay UE is responsible for QoS split in L2 U2U relay, how the relay UE handles the QoS of two hops of U2U relay is up to the implementation of relay UE.

Conclusions

According to the above discussion, the following proposals are given:

Proposal 1: In Model A and Model B, source remote UE is responsible for selecting a relay UE based on source remote UE implementation.

Proposal 2: In this release, RAN2 does not need to achieve the coexistence between the U2N relay and the U2U relay.

Proposal 3: RAN2 is suggested to take local ID used in the SRAP header.

Proposal 4: For the U2U relay, the UE ID in the SRAP header should be extended to both source remote UE ID and target remote UE ID.

Proposal 5: For the U2U relay, the local IDs should be assigned by the relay UE, details are FFS.

Proposal 6: AS layer is responsible for QoS split in L2 U2U relay.

Proposal 7: Relay UE is responsible for QoS split in L2 U2U relay, how the relay UE handles the QoS of two hops of U2U relay is up to the implementation of relay UE.

References

- 3GPP RP-221262, WID for NR sidelink relay enhancements.

- 3GPP TR 38.836, Study on NR sidelink relay.