3GPP TSG-RAN WG2 Meeting #123                           R2- 2307739

Toulouse, France, 21st Aug – 25th Aug 2023

Agenda item:	7.7.4.1.1

Source:	TCL

Title:	Discussion on NTN to TN cell reselection enhancements

Document for:	Discussion and Decision

- Introduction

In Rel-18 WID[1], Following objectives are specified for mobility enhancements.

- For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

- Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signaling overhead. [RAN2, RAN3]

- Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

- Study and, if needed, specify enhancement to Xn[/NG] signaling to support feeder link switch-over, CHO, e.g., exchange of necessary information between gNBs. [RAN3]

Regarding the issue of NTN- TN cell reselection, during last two meeting, the following agreements have been reached.

RAN2#122:

1. TN coverage info is NOT included in SIB19. FFS if we use an existing SIB or a new one.

2. We don’t introduce RRC dedicated signalling to provide more accurate TN coverage information.

3. We no longer consider option 3 alone for signaling the frequency information for TN coverage area (in case option 3 should be combined with option 1). Come back in the next meeting to decide between option 2 (plus possible fixes if needed) and option 1+3.

In this contribution, we will further provide the discussion on NTN – TN cell reselection enhancements.

- Discussion

Regarding the issue that how to signalling the frequency of TN coverage information, during R2#122 meeting, the following three options have been discussed, and in the last the option 2 (plus possible fixes if needed) and option 1+3 are put on the table.

Option 1: Frequency information (i.e. a list of TN frequencies) for each TN coverage area is indicated directly under each TN coverage area configuration.

Option 2: A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

Option 3: Frequency index bitmap is indicated under each TN coverage area, where the frequency index refers to the frequency’s position in the frequency list of the current SIB4.

From our perspective, the basic logic of the two alternatives (Alternative Ⅰ: 1+3 VS Alternative Ⅱ : 2) is as following.

- Alternative Ⅰ: creating index using the form of bitmap for the frequencies carried in SIB4 and SIB5, and associating the indices with the TN coverage areas.

- Alternative Ⅱ: creating index for TN coverage Area, associating the indices with the frequencies listed in SIB4 and SIB5.

When both ways can work, the comparison of these two alternatives can be made from the signalling efficiency perspective.

In terms of signalling efficiency, we can analyse with an extreme case. NTN cells can broadcast up to eight NR neighbouring frequencies and eight E-UTRAN neighbouring frequencies, considering each frequency may be associated with one TN coverage, we think we can start with sixteen TN coverage areas. Considering the extreme case with eight NR neighbouring frequencies and eight E-UTRAN frequencies of TN, and each frequency is associated to a TN coverage area, there are 16 TN frequencies and 16 TN coverage areas.

For alternative Ⅰ, for each frequency carried in SIB4 and SIB5, the bits number of index using the form of bitmap is 16. Herein the total number of bits required for 16 TN coverage areas is 16*16=256. For alternative Ⅱ, for 16 TN coverage areas, 4bits is needed, which means for each frequency listed in SIB4 and SIB5, 4bits IE should be added to associate with the frequency. Herein the total number of bits required for 16 TN coverage areas is 4*16=64. Therefore it is obvious that the alternative Ⅱconsumes less signalling. Especially considering one frequency might be associated with multiple TN coverage areas, alternative Ⅱconsumes even less signalling. It is obvious that alternativeⅠperforms better in terms of signalling consuming.

Proposal 1: A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

Regarding the issue that which SIB should be used for signalling the TN coverage area information, SIB19 has been excluded. The main reason is that SIB19 is to provide the system assistance information for NTN access. And the TN coverage area information is provided to improve the enhancements of cell reselection from NTN to TN for UEs are capable of accessing both NTN and TN, and it is to describe the TN cells information, therefore it is inappropriate to be carried by SIB 19 which is basically only includes NTN cells information.

Regarding between existing SIB and new SIB, which one is more suitable to be used for providing the TN coverage area information, we prefer to use SIB4 and SIB5. TN coverage information is some more details information regarding cell which is a TN cell, therefore it is straightforward that this information is added in the existing SIBs. It is unnecessary to introduce a new SIB.

Moreover, the original purpose to introduce the TN coverage area information is to enhance the performance of cell reselection from NTN cell to TN cell, therefore it implies that it is targeting the UEs who are capable of accessing both NTN and TN cells. Herein in case the TN coverage area information is added is existing SIBs, under the scenario that UEs access TN currently, it might be also beneficial for the TN to TN cell reselection procedure due to more detailed neighbour cell information is provided.

Proposal 2: SIB4 and SIB5 are used for providing TN coverage area information, and no new SIB is introduced for this purpose.

During in RAN2# 122 meeting, the issue of provision of NTN neighbouring cell info in TN cell was discussed, to reduce the time of movement from TN to NTN network. But no conclusion was achieved because of no beneficial use cases were identified, and moreover the main objective of R18 is for enhancements of NTN to TN cell reselection instead of TN to NTN cell reselection.

From our perspective, Rel-18 has specified enhancements for the direction NTN-to-TN, to ensure the ubiquitous connection to ensure the seamless mobility in the opposite direction (TN-to-NTN) is also necessary, even though the standardization work might not be done in R18. In this case, it is nature for a TN cell to provide satellite ephemeris of neighbouring NTN cells to its UEs, to prepare them for mobility towards a neighbouring NTN cell, and the spec impact is not huge.

Proposal 3: TN cell provides satellite ephemeris of neighbouring NTN cells to UEs.

- Conclusion

Based on the above discussion, we recommend RAN2 to discuss and adopt the following proposal:

Proposal 1: A TN coverage area configuration is associated with a TN coverage Area ID. The frequency information for TN coverage area is indicated by adding TN coverage area IDs in SIB4 and SIB5.

Proposal 2: SIB4 and SIB5 are used for providing TN coverage area information, and no new SIB is introduced for this purpose.

Proposal 3: TN cell provides satellite ephemeris of neighbouring NTN cells to UEs.

Reference

- RP-221819, WID: NR NTN (Non-Terrestrial Networks) enhancements